3GPP TSG-RAN WG2 Meeting #129bis	R2-2502032

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.6.3

Source: 	Fujitsu

Title: 	Discussions on measurement related enhancements for LTM

Document for:	Discussion and decision

1. Introduction

The following objective of the work item aims at specifying measurement related enhancements for New WID [1] on NR mobility enhancements:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution will provide our proposal to RAN1 LS [2] and more considerations on the event triggered L1 measurement reporting.

2. Discussion

2.1 RAN2 impacts from RAN1 LS

According to the RAN1 LS [2], MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource, and this is specified in RAN2.

In legacy, the network may activate and deactivate the configured Semi-persistent CSI-RS/CSI-IM resource sets of a Serving Cell by sending the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE, as shown in the figure below. The configured Semi-persistent CSI-RS/CSI-IM resource sets are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. If an SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE on a Serving Cell is received, the MAC entity will indicate to lower layers the information.

Figure 6.1.3.12-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE [3]

To implement the MAC CE used to activate/deactivate the semi-persistent CSI-RS resource of LTM candidate cells, there are two options. One possibility is to reuse the SP CSI-RS/CSI-IM Resource Set Activation/ Deactivation MAC CE which is used to activate/deactivate the semi-persistent CSI-RS resource of a serving cell. Alternatively, a new MAC CE is introduced. Considering LTM candidate configuration ID rather than serving cell ID and no BWP ID in the MAC CE for LTM candidate cells, it is simple to introduce a new MAC CE.

Proposal 1: A new MAC CE is introduced to activate/deactivate the semi-persistent CSI-RS resource of LTM candidate cells.

Proposal 2: The format of the new MAC CE uses the SP CSI-RS/CSI-IM Resource Set Activation/ Deactivation MAC CE as baseline. Candidate configuration ID is included in the new MAC CE to indicate the LTM candidate cell. No BWP ID field is included.

2.2 Event triggered L1 measurement reporting

MAC CE for measurement report (MR MAC CE)

Regarding the L1-RSRP in the MR MAC CE, RAN2 has agreed to use Rel-18 L1-RSRP as baseline [4]. In legacy, if there is one L1-RSRP to be reported, absolute L1-RSRP value with 7 bits will be used. This can apply to the L1-RSRP reporting in the MR MAC CE, for event LTM2/3/4/5. In this case, only the beam triggering the L1 measurement reporting will be included. With this, common format can be used for both non-truncated MR MAC CE and truncated MR MAC CE.

Proposal 3: If there is one L1-RSRP to be reported, absolute L1-RSRP value using 7 bits for the beam triggering the L1 measurement reporting will be included in the (truncated) MR MAC CE.

In case of multiple L1-RSRP values for reporting, differentiated L1-RSRP based mechanism is used. In Rel-18 mechanism, the largest L1-RSRP for the strongest beam is reported using 7-bit absolute value while 4-bit differentiated L1-RSRP value is used for other beams with reference to the largest L1-RSRP value. However, for the LTM event triggered MR MAC CE, more discussions are needed considering the beam triggering the LTM event may be not the strongest beam not only for the event LTM2 but also for the event LTM3/4/5. It has impacts to the design of MR MAC CE format.

For event LTM2, the beam triggering the LTM event is the current beam of serving cell and its L1-RSRP is below a threshold. If the event LTM2 triggered MR MAC CE is also subject to the maximum number of reporting beam configured by the network, i.e. N and there is available L1 measurement result of candidate beams, it is beneficial to include the available L1 measurement result in addition to the one for the current beam of serving cell. If at most N beams will be reported in the event LTM2 triggered MR MAC CE, the strongest beam could be one of candidate beams rather than the current beam of serving cell, i.e. the triggering beam.

For event LTM3/4/5, the triggering beam is the first beam satisfying the entering condition of event LTM3/4/5 during timeToTrigger. So, the triggering beam could be or not be the strongest beam.

To solve the issue, the following options for the format of MR MAC CE can be considered in case of multiple beams for reporting:

- Option 1: legacy mechanism

In this option, the largest L1-RSRP using 7-bits absolute L1-RSRP value for the strongest beam is included as the first L1 measurement result in the MR MAC CE no matter if the strongest beam is the triggering beam or not. And then, according to the measurement quality, 4-bits differentiated L1-RSRP values for other beams will be included.

- Option 2: legacy mechanism + 7-bits absolute L1-RSRP value for the triggering beam

In this option, 7-bits absolute L1-RSRP value for the strongest one the triggering beams will be included as the first L1 measurement result in the MR MAC CE no matter if it is the strongest beam or not. If multiple triggering beams are included in the MR MAC CE, 4-bits differentiated L1-RSRP values will be used for other triggering beams with reference to the L1-RSRP value of the strongest triggering beam. For the beams not satisfying the condition, legacy mechanism applies.

In Option 1, L1-RSRP of the triggering beam could be reporting using 4-bits differentiated L1-RSRP value and thus the reported measurement result of the triggering beam is inaccurate. Different beams could be included at the beginning in case of multiple reporting beams compared to the MR MAC CE containing only one beam information. It incurs some complexity of the UE implementation. Besides, per beam indication on whether the LTM event is satisfied or not is needed. If Option 2 applies, 7 bits more will be used to include additional 7-bits absolute L1-RSRP value for the triggering beam, in case that the triggering beam is the strongest beam, compared to Option 1.

Proposal 4: If there is more than one L1-RSRP to be reported, RAN2 to discuss which option will be used:

    - Option 1: legacy mechanism

    - Option 2: legacy mechanism + 7-bits absolute L1-RSRP value for the strongest triggering beam

Other

For L3 measurement reporting, to address transmission failure of the L3 measurement report, a timer T312 can be configured. The timer T312 can be started if the L3 measurement report is triggered during T310 is running. Failure reporting or connection re-establishment procedure is initiated when the timer expires.

For L1 measurement reporting, RAN2 has agreed that the event triggered L1 measurement reporting should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization

- Select the target beam/cell and trigger LTM cell switch procedure

A similar mechanism can be used for the L1 measurement reporting triggered by the events defined for LTM decision. With this, the UE can take some actions when the transmission of the event triggered L1 measurement report fails.

Proposal 5: RAN2 considers T312-like mechanism, e.g. for the event configured for LTM decision.

According to RAN2 agreement [5], RS associated with indicated beam for the serving cell is used for event evaluation. However, if the indicated beam is associated with PCI of non-serving cell, LTM cell switch based on event-triggered L1 measurement report cannot be supported. In case of inter-cell beam management, the UE is using the beam from the neighbouring cell associated to the serving cell and thus the RS from the neighbouring cell is used for event evaluation. Event e.g. LTM2/3/5 may not be satisfied since the quality of beam of serving cell used for event evaluation, i.e. the RS from the neighbouring cell, is good.

To support LTM cell switch after ICBM, a set of RS(s) can be configured for the serving cell for L1 measurement. Alternatively, RS associated with activated TCI states of the serving cell can be used for event evaluation.

Proposal 6: RAN2 is asked to support LTM cell switch after ICBM.

Proposal 6bis: For L1 event evaluations, if LTM cell switch after ICBM is supported, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

3. Conclusion

In this contribution, we have some discussions on potential measurement enhancements for LTM and made the following proposals:

RAN2 impacts from RAN1 LS

Proposal 1: A new MAC CE is introduced to activate/deactivate the semi-persistent CSI-RS resource of LTM candidate cells.

Proposal 2: The format of the new MAC CE uses the SP CSI-RS/CSI-IM Resource Set Activation/ Deactivation MAC CE as baseline. Candidate configuration ID is included in the new MAC CE to indicate the LTM candidate cell. No BWP ID field is included.

MAC CE for measurement report

Proposal 3: If there is one L1-RSRP to be reported, absolute L1-RSRP value using 7 bits for the beam triggering the L1 measurement reporting will be included in the (truncated) MR MAC CE.

Proposal 4: If there is more than one L1-RSRP to be reported, RAN2 to discuss which option will be used:

    - Option 1: legacy mechanism

    - Option 2: legacy mechanism + 7-bits absolute L1-RSRP value for the strongest triggering beam

Other

Proposal 5: RAN2 considers T312-like mechanism, e.g. for the event configured for LTM decision.

Proposal 6: RAN2 is asked to support LTM cell switch after ICBM.

Proposal 6bis: For L1 event evaluations, if LTM cell switch after ICBM is supported, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

Reference

- RP-242356, “Revised Work Item: NR mobility enhancements Phase 4”, RAN#105, Sep. 9-12, 2024

- R2-2501709, “LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells”, (R1-2501500; contact: Fujitsu), RAN2#129

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

- R2-2501331, “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung), RAN2#129

- R2-2410911, “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#128