3GPP TSG-RAN WG2 Meeting #129bis	R2-2502035

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.7.4.2

Source: 	Fujitsu

Title: 	Discussions on DSR enhancements

Document for:	Discussion and decision

Introduction

On DSR enhancements, there are following agreements [1] in RAN2#128 meeting:

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In RAN2#129 meeting, the following are agreed on DSR enhancements [2]:

Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

In this contribution, we would like to discuss remaining open issues on DSR enhancements.

Discussion

Legacy DSR and enhanced DSR coexistence

In the last RAN2 meeting, there is an FFS on whether the new DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds.

In our understanding, we can always use a new DSR in case there is at least one LCG configured with multiple thresholds. For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG. This is much simpler compared with determining whether legacy DSR or enhanced DSR to be reported based on buffer size or number of reporting threshold(s) of each LCG.

Observation 1: For the LCG configured with multiple reporting thresholds but with only one pair of remaining time and buffer size to be reported, or for the LCG configured with only one reporting threshold, the enhanced DSR format can include just one pair of remaining time information and buffer size information for this LCG.

Proposal 1: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

Configuration of triggering threshold and reporting threshold(s)

As specified in Rel-18, the DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. That is to say, if DSR is triggered for an LCH in an LCG, the DSR MAC CE shall include the delay information of this LCG.

In Rel-19, at least one reporting threshold in addition to the triggering threshold can be configured to an LCG, it may happen that the configured triggering threshold is higher than any of the configured reporting thresholds for the LCG and the remaining time of the data in the LCG is above all the reporting thresholds (there is no control PDU or retransmitted data as well), as shown in Figure 1.

Figure 1 example of triggering threshold and reporting thresholds

In this case, a DSR is triggered when the remaining time of the data in the LCG becomes below the configured triggering threshold, so the delay information of this LCG shall be reported in the new DSR MAC CE if Rel-18 principle is followed. From our point of view, for this LCG, the Buffer Size field in the enhanced DSR MAC CE should be set to 0 and the corresponding remaining time field should be reserved.

Proposal 2: if a DSR is triggered for an LCH in an LCG and the remaining time of the data in the LCG is higher than all the reporting threshold(s), the Buffer Size field in the enhanced DSR MAC CE should be set to 0 and the corresponding remaining time field should be reserved for this LCG.

Control PDU and/or retransmission data

In RAN2#129 meeting, there are offline and online discussions on how to calculate the control PDU and retransmitted data in DSR reporting. 3 options are summarized as the result of offline discussions as following:

- Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting.

- Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

- Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

In our understanding, there are two cases where an LCH having control PDUs and retransmitted data has no data with remaining time lower than a reporting threshold, as shown in Figure 2 and Figure 3. In Figure 2, LCH1 has only control PDU and retransmitted data and has no other data. In Figure 3, LCH1 has control PDU/retransmitted data and also has data with remaining time higher than all the reporting threshold(s), e.g., Th1a and Th 1b which are the configured reporting threshold for the LCG including LCH1 and LCH2. In both cases, control PDUs and retransmitted data can be considered into the shortest reporting threshold included in the DSR or the shortest configured reporting thresholds, since anyway the LCG has data to be reported in the DSR MAC CE.

Figure 2 LCH having only control PDU/retransmitted data

Figure 3 LCH having control PDU/retransmitted data and data with remaining time higher than the reporting threshold(s)

If LCH1 and LCH2 are in different LCG, when a DSR is triggered for LCH1 (if the remaining time of the data in LCH1 becomes lower than the triggering threshold), control PDUs and retransmitted data should be considered into the shortest configured reporting thresholds for the LCG including LCH1.

In our view, to have a unified solution for the cases that LCH1 and LCH2 are in the same LCG or in different LCG, control PDUs and retransmitted data can always be considered into the shortest configured reporting thresholds and option 2 is preferred from our perspective.

Proposal 3: Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

If Proposal 3 can be agreed, the remaining time field in the DSR MAC CE for this LCG should be set to the value of the lowest configured reporting threshold of this LCG, if there are only control PDUs and/or retransmitted data to be reported for this LCG.

Proposal 4: The value of the remaining time field in the enhanced DSR MAC CE is set to the value of the lowest configured reporting threshold of this LCG, if there are only control PDUs and/or retransmitted data to be reported for this LCG.

Definition of Non-delay-reporting data

RAN2 has agreed that the UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. In current PDCP running CR and RLC running CR, the term Non-delay-reporting PDCP/RLC SDU is used representing the non-delay critical data ahead of delay critical data. Use PDCP running CR as example, currently the definiton of non-delay-reporting PDCP SDU is as following:

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that is associated with a COUNT value smaller than the largest COUNT value of the delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

In the above definition and similarly in RLC running CR, the COUNT/SN value of the SDU is used to determine there position in the buffer. Based on companies’ contributions and previous discussions, there may be several scenarios for non-delay-critical data ahead of delay-critical data:

1. Low importance data when discardTimerForLowImportance is configured and PSI based discard is activated;

2. non-delay-critical data ahead of queue for the same LCH;

3. non-delay-critical data in another LCH which has higher priority than the LCH which has delay-critical data.

We think that the use of COUNT/SN value cannot cover all these cases. To cover all the cases, one solution is to use a general definition like the following:

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that is not a delay-reporting PDCP SDU associated with the k:th dsr-ReportingThreshold where k<i (if i>1), and is ahead of the a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold but behind all delay-reporting PDCP SDU(s) associated with the i-1:th dsr-ReportingThreshold (if i >1) in the buffer.

If more precise definitions are preferred to reflect the above scenarios, a NOTE may be added like the following:

NOTE: the non-delay-reprting PDCP SDU includes SDU(s) with low PSI when discardTimerForLowImportance is configured and PSI based discard is activated, and SDU(s) from another logical channel which has higher priority than the logical channel corresponding to this PDCP entity.

RAN2 may start from the above proposed definition in PDCP layer and discuss whether any NOTE is added for clarification. The definition in RLC layer may use the same approach.

Proposal 5: To cover all scenarios for non-delay-critical data ahead of delay-critical data, change the non-delay-reporting PDCP SDU to the following:

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that is not a delay-reporting PDCP SDU associated with the k:th dsr-ReportingThreshold where k<i (if i>1), and is ahead of the a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold but behind all delay-reporting PDCP SDU(s) associated with the i-1:th dsr-ReportingThreshold (if i >1) in the buffer.

Conclusion

In this contribution, we have discussed the further details of the DSR enhancements. We have the following proposals:

Proposal 1: Enhanced DSR is always used in case there is at least one LCG configured with multiple reporting thresholds.

Proposal 2: if a DSR is triggered for an LCH in an LCG and the remaining time of the data in the LCG is higher than all the reporting threshold(s), the Buffer Size field in the enhanced DSR MAC CE should be set to 0 and the corresponding remaining time field should be reserved for this LCG.

Proposal 3: Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

Proposal 4: The value of the remaining time field in the enhanced DSR MAC CE is set to the value of the lowest configured reporting threshold of this LCG, if there are only control PDUs and/or retransmitted data to be reported for this LCG.

Proposal 5: To cover all scenarios for non-delay-critical data ahead of delay-critical data, change the non-delay-reporting PDCP SDU to the following:

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that is not a delay-reporting PDCP SDU associated with the k:th dsr-ReportingThreshold where k<i (if i>1), and is ahead of the a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold but behind all delay-reporting PDCP SDU(s) associated with the i-1:th dsr-ReportingThreshold (if i >1) in the buffer.

References

- R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#128 meeting

- Report from session on R18 MBS, R18 QoE and R19 XR,	Session chair (Huawei), RAN2#129 meeting.

- R2-2501950, PDCP running CR for R19 XR, RAN2#129bis meeting

- R2-25xxxxx, Running RLC CR for XR, RAN2#129bis meeting