3GPP TSG-RAN WG2 Meeting #129bis	R2-2502044

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.8.4 Support of broadcast service

Title:		Discussion on the support of broadcast service in NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 102th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

- Specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]. - Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary between CN and NG-RAN. [RAN3]

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for the support of broadcast service in NTN.

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behaviour is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

During the 128th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

During the 129th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list. 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH). 3. Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented).

In this paper, we would like to provide our views on the highlighted aspects as well as remaining issues on support of broadcast service in NTN.

# 2. Discussion

2.1	MCCH skipping

Since the 127bis meeting, RAN WG2 has been studying a potential enhancement allowing the UE to skip MCCH re-acquisition when outside the intended broadcast service area, based on the assumption that geographic service area information is semi-static and infrequently updated. It was agreed in the previous meeting to continue discussing this topic in the next meeting.

For NTN enhancements of MBS broadcast services in Rel-19, efficient UE power saving is critical due to operational constraints specific to NTN. Under current behaviour, the UE always reacquires MCCH upon receiving a notification of MCCH changes, even if it is outside the intended service area. Such unnecessary MCCH re-acquisition leads to inefficiencies and increased power consumption, which is particularly problematic in power-sensitive NTN environments.

Observation 1: Unnecessary MCCH (re-)acquisition may lead to inefficiencies and increased power consumption, which is particularly problematic in power-sensitive NTN environments.

Proposal 1: RAN WG2 to introduce a mechanism for UE to skip MCCH (re-)acquisition when it is outside of the ISAs of an interested service.

Under current agreements, the UE initially receives a new SIB broadcasted by the network, which includes a list of Intended Service Areas (ISAs) along with their corresponding unique ISA IDs, indicating geographical areas where MBS services may be provided. The UE can use this ISA information to determine whether its current location is within any of the listed ISAs. Afterward, upon receiving SIB20, the UE obtains MCCH configuration information. Based on the MCCH configuration information, the UE acquires MCCH. The detailed mapping between specific MBS services and multiple ISA IDs is available only after acquiring the MCCH. Thus, before initial MCCH acquisition, the UE cannot identify precisely which ISAs are associated with its interested MBS services—it only knows the general list of ISAs supporting any MBS service. Considering this limitation, for power-saving purposes, the UE can use the ISA list provided by the new SIB to decide whether to skip initial MCCH acquisition. Specifically, the UE should skip initial MCCH acquisition if it determines, based on the received SIB, that it is located outside all listed ISAs.

Proposal 2: RAN WG2 to allow UE to skip initial MCCH acquisition when it is not in or close to any ISA(s) provided via the new SIB.

After acquiring MCCH, the UE periodically monitors the MCCH change notification field provided in SIB20 or via dedicated signalling. When the UE detects a notification indicating MCCH changes (e.g., the change notification bit set to ‘1’), it recognizes the need to update its stored MCCH information. Upon re-acquiring MCCH, the UE updates its stored MCCH configurations and ISA mapping. At this stage, the UE knows the general list of Intended Service Areas (ISAs) from the new SIB and also knows the specific ISA associations for its interested MBS services from previously acquired MCCH information. Considering these, for power-saving purposes, the UE can consider the following two options to determine whether to skip MCCH re-acquisition:

- Option 1: Based on the new SIB providing a list of ISA(s)

  - The UE skips MCCH re-acquisition if it determines, based solely on the new SIB, that it is outside all listed ISAs.

- Option 2: Based on the previous MCCH information.

  - The UE skips MCCH re-acquisition if it determines, based on previously stored MCCH information, that it is outside the specific ISAs associated with its interested MBS services.

In Option 1, the same mechanism is applied for both initial MCCH acquisition and MCCH re-acquisition. However, the UE cannot skip MCCH re-acquisition when it is outside the ISA(s) of the interested MBS service if it remains within the general list of ISA(s) provided via the new SIB. On the other hand, in Option 2, the UE can skip MCCH re-acquisition if it determines it is outside the ISA(s) associated with the interested MBS service. However, Option 2 requires different mechanisms for initial MCCH acquisition and MCCH re-acquisition.

Proposal 3: For skipping MCCH re-acquisition, RAN WG2 to decide whether to

- Allow the UE to skip MCCH re-acquisition when it is not within or near any ISA(s) provided by the new SIB (providing a list of ISA(s)).

- Allow the UE to skip MCCH re-acquisition when it is not within or near the specific ISA(s) associated with its interested MBS services (based on the previous/stored MCCH info.).

Moreover, the MCCH change notification via SIB20 or dedicated signalling might include detailed mapping changes between specific MBS services and multiple ISA IDs. In this scenario, the UE should not skip MCCH re-acquisition, as it needs to obtain updated ISA information related to the interested MBS service. Consequently, under Option 2, the MCCH change notification via SIB20 or dedicated signalling in NTN should distinguish between general MCCH updates and detailed mapping changes involving specific MBS services and multiple ISA IDs. To support this, a new MCCH change notification field can be introduced, clearly indicating the type of MCCH update. Thus, when the UE detects this specific notification (e.g., a dedicated notification bit set to ‘1’), it recognizes the necessity to perform MCCH re-acquisition regardless of its current location, ensuring updated mappings between specific MBS services and ISAs are accurately stored.

Observation 2: Skipping MCCH re-acquisition based on specific ISA(s) associated with interested MBS services (from the previous/stored MCCH info.) may not be feasible when the MCCH change notification includes detailed mapping changes between MBS services and multiple ISA IDs.

Proposal 4: RAN WG2 to introduce a new MCCH change notification field within SIB20 or dedicated signalling to skip MCCH re-acquisition based on the previous/stored MCCH info.

- “To explicitly indicate updates to detailed mappings between specific MBS services and multiple ISA IDs.”

2.2	Service Continuity

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

- 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

- 2) Either:

  - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

  - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

  - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

Considering that SIB21 contains the mapping relationship between the current and/or neighbouring carrier frequencies and MBS FSAI, this legacy mechanism for MBS service continuity assumes that the MBS service area is mapped onto one or more cells. However, in NTN systems, this may not work effectively because the UE may not consider that frequency as the highest priority if the MBS intended service area(s) of that frequency do not include the intended service area of the MBS session that the UE is receiving or interested in, even if the above conditions 1 and 2 are met.

In the 127th meeting, the following agreements were reached regarding UE behaviour for the support of broadcast service in NTN.

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2).

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3).

Given this agreed UE behaviour, ETRI believes that enhancements to the legacy mechanism for MBS service continuity are necessary. Specially, the UE may consider that frequency of neighbouring cell providing the interested MBS as the highest priority only if it will be within the intended service areas after cell reselection. Conversely, the UE should not assign the highest frequency to that frequency if it will be outside the intended service areas.

To address these issues, the following two options can be considered.

- Option 1: To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs. This approach would involve modifying SIB21 to include:

  - Carrier frequency list (already existing in SIB21)

  - MBS intended service area ID(s) per each carrier frequency

- Option 2: To utilize intended service area information of neighbouring cells from a new SIB and MCCH. This approach would include:

  - A list of ISAs of neighbouring cells in a new SIB

  - The ISA IDs of neighbouring cells in MCCH

In option 1, when the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session if the UE will be within the intended service areas corresponding to one or more intended service area ID(s) of that frequency indicated in SIB21. Conversely, the UE should not prioritize that frequency as the highest priority if it will be outside the intended service areas.

In option 2, the UE may consider that frequency as the highest priority during the MBS broadcast session if the UE will be within the intended service areas corresponding to one or more intended service ID(s) of neighbouring cells indicated in MCCH.

Proposal 5: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency as the highest priority during the MBS broadcast session only if it will be only within the intended service areas.

Proposal 6: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE should not consider that frequency as the highest priority during the MBS broadcast session if the UE will be outside the intended service areas.

Proposal 7: For service continuity, RAN WG2 to decide whether to

- Introduce intended service area information (e.g. intended service area ID(s)) associated to the current and/or neighbouring carrier frequencies in SIB21

- Utilize intended service area information of neighbouring cells from a new SIB and MCCH.

# 3. Conclusions

Observation 1: Unnecessary MCCH (re-)acquisition may lead to inefficiencies and increased power consumption, which is particularly problematic in power-sensitive NTN environments.

Proposal 1: RAN WG2 to introduce a mechanism for UE to skip MCCH (re-)acquisition when it is outside of the ISAs of an interested service.

Proposal 2: RAN WG2 to allow UE to skip initial MCCH acquisition when it is not in or close to any ISA(s) provided via the new SIB.

Proposal 3: For skipping MCCH re-acquisition, RAN WG2 to decide whether to

- Allow the UE to skip MCCH re-acquisition when it is not within or near any ISA(s) provided by the new SIB.

- Allow the UE to skip MCCH re-acquisition when it is not within or near the specific ISA(s) associated with its interested MBS services.

Observation 2: Skipping MCCH re-acquisition based on specific ISA(s) associated with interested MBS services (from the previous/stored MCCH info.) may not be feasible when the MCCH change notification includes detailed mapping changes between MBS services and multiple ISA IDs.

Proposal 4: RAN WG2 to introduce a new MCCH change notification field within SIB20 or dedicated signalling to skip MCCH re-acquisition based on the previous/stored MCCH info.

- “To explicitly indicate updates to detailed mappings between specific MBS services and multiple ISA IDs.”

Proposal 5: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency as the highest priority during the MBS broadcast session only if it will be only within the intended service areas.

Proposal 6: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE should not consider that frequency as the highest priority during the MBS broadcast session if the UE will be outside the intended service areas.

Proposal 7: For service continuity, RAN WG2 to decide whether to

- Introduce intended service area information (e.g. intended service area ID(s)) associated to the current and/or neighbouring carrier frequencies in SIB21

- Utilize intended service area information associated to neighbouring cells, as indicated in MCCH.