3GPP TSG-RAN WG2 Meeting #129bis	R2-2502047

Wuhan, China, April 7 – 11 2025

Agenda item:	7.0.2.20

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN2 Impacts for NR-NTN NB-IOT Inband operation

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this discussion paper we analyse the RAN2 impacts to support NB-IoT NTN operating within the operating bandwidth of NR.

# 2	Discussion

In the LS from RAN4 [R4-2505035] RAN4 indicated the following.

RAN4 introduced the inband operation for NTN IoT in NR NTN in TEI18. RAN4 has assumed there is no RAN2 impact. ​

RAN4 has made below agreements ​

• In Rel-18, UE is only required to support the same operation mode for anchor and non-anchor carriers​

• For in-band operation, RRC signalling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests.​

• in Rel-18 for in-band operation, UE is not expected to access a cell indicating inband-SamePCI or inband-DifferentPCI in operationMode IE in MasterInformationBlock-NB.​

RAN4 would like to respectfully ask RAN2 to confirm above assumptions aligning with RAN2 specification.​

The basic operating mode supported for NB-IoT-NTN Rel-17 was only standalone mode. Support of other modes is new functionality for Rel-18 UE. Hence new UE capability without signalling is needed to differentiate the UE behaviour in idle mode.

In addition to the above, the NW configuration restrictions for the operating mode indicated in the RAN4 LS may need to be captured in stage-2 description and the field description of operating mode in stage-3 specification.

Proposal 1: RAN2 to discuss need for new UE capability in Rel-18 (without signalling) for the proposed NB-IoT Guardband mode operation for NB-IoT NR Inband deployment.

Proposal 2: Network configuration for NB-IoT operating mode for NR Inband should be  captured in RAN2 specification.

In Rel-17 the NB-IoT-NTN cell can be configured only with standalone mode. With the proposed extension for NR-Inband support the mode can also be configured as guard band.  Rel-18 NB-IoT-NTN cell cannot be configured with inband-same-PCI or inband-DifferentPCI at all in this case. Hence there is no valid scenario exist where Rel-18 NB-IoT NTN cell broadcast Inband configuration parameter. `

Proposal 3: No UE behaviour changes needed related to Inband configuration parameters in Rel-18 as this parameter is not applicable for IoT-NTN in NR Inband.

# 3	Conclusion

In this paper we analysed the RAN2 impacts for NB-IoT NTN operation as Inband within NR-NTN based on the observations from RAN4.

Proposal 1: RAN2 to discuss need for new UE capability in Rel-18 (without signalling) for the proposed NB-IoT Guard band mode operation for NB-IoT NR Inband deployment.

Proposal 2: Network configuration for NB-IoT operating mode for NR Inband should be  captured in RAN2 specification.

Proposal 3: No UE behaviour changes needed related to Inband configuration parameters in Rel-18 as this parameter is not applicable for IoT-NTN in NR Inband.