3GPP TSG-RAN WG2 Meeting #129bis	 R2-2502056

Wuhan, China, April 7 – 11, 2025

Agenda Item:	8.8.6

Source: 	CATT

Title: 	Discussion on the remaining issue for LTE_TN_NR_NTN_mob WI (dedicated cell reselection priority aspect)

Document for: 	Discussion and Decision

Introduction

During the email discussion [Post129][303][LTE_TN_NR_NTN], companies raised an issue on whether the dedicated priority in RRCConnectionRelease message (cited below from [1]) can be configured for cell reselection from E-UTRA TN to NR NTN.

IdleModeMobilityControlInfo ::= SEQUENCE { freqPriorityListEUTRA FreqPriorityListEUTRA OPTIONAL, -- Need ON freqPriorityListGERAN FreqsPriorityListGERAN OPTIONAL, -- Need ON freqPriorityListUTRA-FDD FreqPriorityListUTRA-FDD OPTIONAL, -- Need ON freqPriorityListUTRA-TDD FreqPriorityListUTRA-TDD OPTIONAL, -- Need ON bandClassPriorityListHRPD BandClassPriorityListHRPD OPTIONAL, -- Need ON bandClassPriorityList1XRTT BandClassPriorityList1XRTT OPTIONAL, -- Need ON t320 ENUMERATED { min5, min10, min20, min30, min60, min120, min180, spare1} OPTIONAL, -- Need OR ..., [[ freqPriorityListExtEUTRA-r12 FreqPriorityListExtEUTRA-r12 OPTIONAL -- Need ON ]], [[ freqPriorityListEUTRA-v1310 FreqPriorityListEUTRA-v1310 OPTIONAL, -- Need ON freqPriorityListExtEUTRA-v1310 FreqPriorityListExtEUTRA-v1310 OPTIONAL -- Need ON ]], [[ freqPriorityListNR-r15 FreqPriorityListNR-r15 OPTIONAL -- Need ON ]] } […] freqPriorityListX Provides a cell reselection priority for each frequency, by means of separate lists for each RAT (including E-UTRA). The UE shall be able to store at least 3 occurrences of FreqsPriorityGERAN. If E-UTRAN includes freqPriorityListEUTRA-v9e0 and/or freqPriorityListEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListEUTRA (i.e. without suffix). Field freqPriorityListExt includes additional neighbouring inter-frequencies, i.e. extending the size of the inter-frequency carrier list using the general principles specified in 5.1.2. EUTRAN only includes freqPriorityListExtEUTRA if freqPriorityListEUTRA (i.e without suffix) includes maxFreq entries. If E-UTRAN includes freqPriorityListExtEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListExtEUTRA-r12..

This issue has more or less similarity with the E-UTRA TN to NR NTN redirection concluded at the last meeting, and it is a functional open issue concerns whether such dedicated cell reselection priority functionality is supported in the context of the E-UTRA TN to NR NTN IDLE mode mobility.

This contribution, therefore, aims to conclude this remaining functional open issue for LTE_TN_NR_NTN_mob WI.

Discussion

It was agreed in RAN2 #127 to introduce an optional UE capability w/o signaling for this E-UTRA TN to NR NTN cell reselection feature, and it is now captured in the TS 36.306 running CR as follows: [2]

Since the E-UTRA TN to NR NTN cell reselection feature itself is an optional UE feature introduced in Rel-19, the key point here is whether the E-UTRAN which broadcasts SIB33 including the NR Satellite information first needs to know whether a Rel-19 UE supports this IRAT cell reselection feature at all, before it can really decide to configure the dedicated priority for an NR NTN frequency in RRCConnectionRelease msg to the UE. Logically speaking, the support of this IRAT cell reselection feature by the UE should be a prerequisite for the NW to configure this dedicated priority to an NR NTN frequency.

Observation: It was agreed that the IRAT E-UTRA TN to NR NTN cell reselection is an optional UE feature for Rel-19 LTE UEs. Whether the dedicated priority for an NR NTN frequency can be configured by E-UTRAN to a Rel-19 UE depends on whether the UE supports this IRAT cell reselection feature at all.

Then there can be the following possible options to address this issue, with difference on whether to introduce a capability signaling for this IRAT cell reselection feature and how NW behaves correspondingly.

- Option #1: Introduce a new capability signaling specific for E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability reported from the UE (based on NW implementation).

This is actually extending the optional capability w/o signaling we already captured in [2] to an optional capability with signaling, and making it applied to also the CONNECTED UEs (for the dedicated priority configuration purpose). A new capability signaling thus needs to be introduced in this option.

- Option #2: Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

RAN2 agreed to introduce a UE capability for E-UTRA TN to NR NTN redirection, as captured in RRC running CR (i.e. "ntn-RedirectionNR-r19" in [3]). The rationale for this option is that the UE operation that is actually required by this "ntn-RedirectionNR-r19" capability should be the application of the NR satellite assistance info newly introduced in SIB33 to perform the autonomous adjustment of the SMTC within SIB24, and this seems to be an operation that is common to E-UTRA TN to NR NTN redirection and cell reselection.

- Option #3: No UE capability signaling is needed to indicate support of E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives supported NR NTN band(s) indicated in UE capability signaling. FFS whether any clarification is needed in the Spec.

This option assumes that as long as a supported NR NTN band is reported by a Rel-19 UE, E-UTRAN anyway assumes that the UE is able to perform the cell reselection to the NR NTN frequencies on the reported band. So not any form of new capability signaling is needed. On the other hand, whether this assumption, i.e. reporting supported NR NTN bands directly means the support of cell reselection to NR NTN, holds or not may need to be clarified. If it really holds, it is also questionable whether such clarification needs to be reflected in the Spec to avoid possible ambiguity. This is the "FFS" part in the above Option #3 description.

RAN2 is thus suggested concluding how/whether the feature of dedicated priority for E-UTRA TN to NR NTN cell reselection is supported in Rel-19, and down-selects from the above options.

Conclusion

This contribution discusses the remaining functional open issue for LTE_TN_NR_NTN_mob WI, regarding whether to support dedicated priority configuration for E_UTRA TN to NR NTN cell reselection. The proposal of this contribution is as follows:

Proposal: RAN2 discusses whether/how to support the dedicated priority configuration for E-UTRA TN to NR NTN cell reselection, and down-selects from below options:

- Option #1: Introduce a new capability signaling specific for E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability reported from the UE (based on NW implementation).

- Option #2: Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

- Option #3: No UE capability signaling is needed to indicate support of E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives supported NR NTN band(s) indicated in UE capability signaling. FFS whether any clarification is needed in the Spec.

Reference

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"TS 36.331

- R2-2501417	Introduction of LTE TN to NR NTN Mobility UE Capability            vivo      CR            Rel-19  36.306  18.4.0   1900     2          B            LTE_TN_NR_NTN_mob-Core

- R2-2501418   Introduction of LTE TN to NR NTN IDLE mode mobility    CATT    CR       Rel-19            36.331  18.4.0   5065     3          B          LTE_TN_NR_NTN_mob-Core