3GPP TSG-RAN WG2 Meeting #129bis	R2-2502057

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on downlink coverage enhancements

Document for: 	Discussion and Decision

Introduction

This contribution further discusses possible RAN2 impacts of DL coverage enhancements.

Discussion

2.1	Impact analysis due to extended SSB periodicity

2.1.1		SMTC enhancements

In RAN2#129, RAN2 discussed the beam hopping pattern for the DL coverage enhancement scenario and agreed to support clustered cells illuminated case and scattered cells illuminated case. Some companies argued that if the cells activated simultaneously are scattered, the existing 4 SMTC windows per frequency may not be enough.

From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1. RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

Given that the UE complexity increases along with measurements on additional SMTC windows, RAN2 has agreed not to increase the maximum number of SMTC/gap an UE can measure simultaneously. Consequently, if the number of configured SMTC windows per frequency is increased, the issue becomes how the UE selects which SMTC windows to measure among these configured SMTCs.

The number of SMTC/gaps a UE needs to consider at any time will not be increased further

Apart from increasing the number of SMTCs windows, companies also proposed enhancements on the SMTC mechanism by taking location factor into consideration, such as configuring SMTC list per SSB of the serving cell or configuring SMTC list per GNSS area. From our perspective, we see such per SSB/location configured SMTC as assistance information for UE in selecting SMTCs to be really measured. Regardless of whether such assistance information is configured, UE selection on the SMTC actually measured anyway can be left to implementation, considering relevant factors (e.g. location, serving SSB, etc.).

Proposal 1: Reuse the existing mechanism on SMTC selection, i.e., if the UE supports less SMTCs than what the network configured, it is up to the UE to select which SMTCs to consider.

Then, RAN2 can further discuss what assistance information related to SMTCs can be configured to the UE for IDLE/INACTIVE measurement to enable above UE implementation based SMTC selection (e.g. per SSB SMTC configuration):

Proposal 2: RAN2 discusses whether to support further assistance information for SMTC configurations in SIB2 and SIB4 (e.g., per SSB SMTC configuration).

2.1.2		Impact on RRC procedure

The following is to discuss whether any other enhancements need to be introduced due to extended SSB periodicity of 160 ms. Note that since the extended SSB periodicity was introduced for the default SSB value, which impacts only UEs' access from RRC_IDLE/INACTIVE, the following will analyse the SI/paging/RACH related aspects.

1) System information

According to current spec [1], when the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as SSB, i.e., if the SSB periodicity is 160 ms, the MIB is transmitted with periodicity of 160 ms too. No spec impact is needed.

- the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB; NOTE 1: If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB.

Observation 1: Current TS 38.331 supports the MIB broadcasted with the same periodicity as SSB when the periodicity of SSB is larger than 80 ms.

The scheduling of SIB1 relies on the SSB and CORESET multiplexing pattern, it is up to RAN1’s design to ensure that the SIB1 periodicity under different multiplexing patterns could match the SSB periodicity of 160 ms. RAN2 could wait for RAN1’s progress.

Observation 2: It is up to RAN1 whether any change is needed for the resource configuration used for SIB1 scheduling.

- the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13).

Other SI is scheduled with the SI-window mechanism. The SI-window is determined with parameters of si-WindowLength and si-Periodicity. As a reference to the current parameter value range, the SI-window can be configured to adapt to the SSB periodicity by network implementation. For example: the si-Periodicity is configured as rf16 or multiples of rf16 and the si-WindowLength is configured with a suitable value to ensure the transmission of system information to be completed during the beam coverage time of the serving cell. As another possible configuration, the SI window length can be configured with 160 ms or multiples of 160 ms, which can be supported based on current configuration as well.

Hence based on current RRC spec [1], suitable configuration for SI scheduling can adapt to the 160 ms SSB periodicity, and no extra Spec impact is foreseen from RAN2 perspective, unless RAN1 requires any enhancements.

SI-SchedulingInfo ::= SEQUENCE { schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo, si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, si-RequestConfig SI-RequestConfig OPTIONAL, -- Cond MSG-1 si-RequestConfigSUL SI-RequestConfig OPTIONAL, -- Cond SUL-MSG-1 systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, -- Need R ... } SchedulingInfo ::= SEQUENCE { si-BroadcastStatus ENUMERATED {broadcasting, notBroadcasting}, si-Periodicity ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo SIB-Mapping }

Observation 3: By network implementation, the SI scheduling in the current Spec can adapt to the SSB periodicity of 160 ms.

2) PCCH-Config

As specified in [2], the DRX cycle is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any) and a default DRX value broadcast in system information. The value range of all these DRX values is {rf32, rf64, rf128, rf256}. Furthermore, the paging frame and offset are configured with the parameter nAndPagingFrameOffset. When this parameter is set to oneSixteenthT, the PF period equals to 16rf. The paging configuration on paging cycle and PF can fulfil the needs for extended SSB periodicity of 160 ms, so that enhancement is not needed from RAN2 perspective.

nAndPagingFrameOffset CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15) },

Observation 4: The PCCH-Config in the current Spec can adapt to the extended SSB periodicity of 160 ms.

3) RACH configuration

The RACH occasion is configured per SSB with the parameter of ssb-perRACH-OccasionAndCB-PreamblesPerSSB and the mapping relationship of SS/PBCH block indexes to PRACH occasions is defined in [3]. It is up to RAN1 to check whether any enhancement is needed for PRACH related parameters due to extended SSB periodicity.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { oneEighth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneFourth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneHalf ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, two ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32}, four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4) } OPTIONAL, -- Need M

Observation 5: It is up to RAN1 to check whether any enhancement is needed for PRACH configuration.

4) Common SearchSpace configuration

The NW configures some common search spaces for UE (including searchSpaceOtherSystemInformation, pagingSearchSpace and ra-SearchSpace), so that the UE can determine the time domain resource of PDCCH monitoring occasions. When SearchSpaceID = 0 is configured for UE, the UE follows the predefined tables in [3], and it is up to RAN1 to check whether any enhancement is needed to for SSB periodicity 160 ms. When SearchSpaceID other than 0 is configured for these parameters, the UE applies the configurations provided via RRC signalling. After checking the current spec, we think the current value range can fulfil the possible requirement caused by the extended SSB periodicity of 160 ms (since the monitoring periodicity of search space can be up to 2560 ms for SCS 15 kHz and 320 ms for SCS 120 kHz). It seems there is no enhancement required on the configuration of search space from RAN2 perspective.

monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639), sl1280 INTEGER (0..1279), sl2560 INTEGER (0..2559) } OPTIONAL, -- Cond Setup4

Observation 6: The current common search space configuration provided via RRC signalling can adapt to the extended SSB periodicity of 160 ms.

Based on the above observations, we found no enhancement needed from RAN2 perspective due to SSB extension to 160 ms, w.r.t. SI, paging, RACH related aspects (unless any enhancement is required by RAN1). We propose this as follows:

Proposal 3: From RAN2’s perspective, the current RAN2 spec can support the extended SSB periodicity of 160 ms. No enhancement on SI-scheduling, PCCH-Config, RACH configuration and Common SearchSpace is foreseen (unless required by RAN1).

2.1.3	 Need of barring mechanism for extended SIB periodicity

Per analysis in section 2.1.1 and 2.1.2, extended SIB periodicity exclusively affects the initial cell search for the NTN cell in DL CE. Upon successful camping on such an NTN cell, the UE could work normally as the existing configurations for SI/paging/RACH does not need to be extended so far. The SMTC enhancement should be an optional feature to support DL CE, and the UEs that do not support DL CE could also perform measurement based on legacy smtc4list after having camped on a DL CE cell. Hence, there is no need to bar the UEs not supporting extended SIB periodicity from accessing the NTN cell with SSB periodicity of 160ms, unless essential issue is identified by RAN1

Proposal 4: No need to introduce a barring mechanism for a DL CE cell with extended SSB periodicity of 160ms (unless essential issue is identified by RAN1).

2.2	Discussion on link level enhancement

RAN1 is discussing support of Type0 PDCCH CSS repetition, PDSCH repetition for SIB1 and Msg4 PDSCH repetition [4][5]. This section discusses possible RAN2 impacts based on RAN1 progress.

- Type0 PDCCH CSS repetition

RAN1 has discussed candidate solutions for supporting Type0 PDCCH CSS repetition. Based on RAN1 discussion, it seems that RAN1 intended to design solutions compatible with legacy UEs and the backward compatibility of the final concluded solution has also impact on further RAN2 discussion. RAN2 could wait for further progress of RAN1 in this aspect.

Agreement At least for enabling PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, RAN1 to consider the following options - Option 1: Using the spare 1 bit in MIB - Option 2: Using reserved bit(s) in PBCH payload - Option 3: Using codepoint(s) in PBCH payload - Option 4: UE blind decoding without signaling from the network during initial access Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, consider the following: - Option 1: Support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - FFS: Details including how the two PDCCH candidates are counted toward the BD limits - Note: with option 1, if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. - FFS: whether/how option 1 can be applicable for M=1 and M= ½ - Option 2: Support repeated PDCCH candidates in the two slots n0 and n0+X [or n0+1 and n0+1+X ] associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) - Value of X>1, predefined or configured - FFS: Value of X - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - FFS: Backward compatibility to legacy UE - Option 3: - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index - For M=1/2 and M=1, the repeated PDCCH candidates in two consecutive slots associated with different SSB indexes; - For M=2, the PDCCH candidates in slots n0+1 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index - Option 4: Option 2 with cross SSB beam repetition support - The PDCCH candidates in slots n0 associated respectively with different SSB indexes are repetitions of each other and share the same aggregation level, coded bits and same candidate index

- PDSCH repetition for SIB1

RAN1 has discussed the candidate solutions for supporting PDSCH repetition for SIB1, and considered whether to joint repetition of SIB1 PDSCH and Type0-PDCCH CSS [5]. RAN1 also intended to design solutions compatible with legacy UEs. Thus, similar to above Type0 PDCCH CSS repetition, RAN2 could wait for further progress of RAN1 on this topic.

Agreement For SIB1 link level enhancement, RAN1 to consider the following options: - Option 1: PDSCH repetition of SIB1 is transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV - Option 2: Option 1 and an additional PDSCH with SIB1 repetition can occur after the slot of type0-PDCCH CSS repetition. - FFS: How to schedule the SIB1 repetition - Option 3: The repetition of PDSCH with SIB1 is indicated by the scheduling PDCCH - PDSCH is repeated in two slots Note: Backward compatibility should be maintained

- Msg4 PDSCH repetition

During the RAN1 discussion, companies’ views on whether UE needs to reports its capability for Msg4 PDSCH repetition are divergence. This is related to whether the Msg4 PDSCH repetition is scheduled in a UE specific manner or not, which is further related to the down-selection on the following candidate options. Therefore, corresponding RAN2 discussions (e.g. whether any forms of early identification is needed as raised by some companies in last meeting) need be postponed till RAN1 make the down-selection first.

Agreement For Msg4 PDSCH repetition support, RAN1 to consider: - Option 1: UE specific repetition indication via DCI - Option 2: Msg4 repetition is configured by SIB1 - Option 3: Msg4 PDSCH repetition is implicitly determined by SIB1 PDSCH repetition

In summary, all the discussions on link level enhancement need to depend on further RAN1 progress. Therefore, it is suggested that RAN2 postpone the discussion on link level enhancements until RAN1 has decision on these candidate solutions.

Proposal 5: RAN2 postpones the discussion on link level DL coverage enhancements until RAN1 has further progress

Conclusion

In this contribution, DL coverage enhancement is further discussed with the corresponding observations and proposals listed below:

Observation 1: Current TS 38.331 supports the MIB broadcasted with the same periodicity as SSB when the periodicity of SSB is larger than 80 ms.

Observation 2: It is up to RAN1 whether any change is needed for the resource configuration used for SIB1 scheduling.

Observation 3: By network implementation, the SI scheduling in the current Spec can adapt to the SSB periodicity of 160 ms.

Observation 4: The PCCH-Config in the current Spec can adapt to the extended SSB periodicity of 160 ms.

Observation 5: It is up to RAN1 to check whether any enhancement is needed for PRACH configuration.

Observation 6: The current common search space configuration provided via RRC signalling can adapt to the extended SSB periodicity of 160 ms.

Proposal 1: Reuse the existing mechanism on SMTC selection, i.e., if the UE supports less SMTCs than what the network configured, it is up to the UE to select which SMTCs to consider.

Proposal 2: RAN2 discusses whether to support further assistance information for SMTC configurations in SIB2 and SIB4 (e.g., per SSB SMTC configuration).

Proposal 3: From RAN2’s perspective, the current RAN2 spec can support the extended SSB periodicity of 160 ms. No enhancement on SI-scheduling, PCCH-Config, RACH configuration and Common SearchSpace is foreseen (unless required by RAN1).

Proposal 4: No need to introduce a barring mechanism for a DL CE cell with extended SSB periodicity of 160ms (unless essential issue is identified by RAN1).

Proposal 5: RAN2 postpones the discussion on link level DL coverage enhancements until RAN1 has further progress.

Reference

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

- 3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state"

- 3GPP TS 38.213: "Physical layer procedures for control"

- RAN1#120 Chair notes

- R1-2500042	FL Summary #4: NR-NTN downlink coverage enhancements