3GPP TSG-RAN WG2 Meeting #129bis	 R2-2502059

Wuhan, China, April 7th – 11st, 2025

Agenda Item:	8.17

Source: 	CATT

Title: 	Discussion on support of NB-IoT NTN TDD

Document for: 	Discussion and Decision

Introduction

In WID [1], RAN2 objectives are shown as follows:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

This contribution analyzes potential RAN2 impacts to support NB-IoT TDD NTN on “other necessary impacts on higher layer” caused by the new frame structure of 90 ms period, from CP and UP perspectives, based on the agreements of the RAN1 and RAN2 in the previous meetings.

Discussion

Potential impacts on CP

The periodicity of the TDD frame is 90 ms; however the periodicities of MIB, SIB1, other system information and paging are now times of 2 radio frames in general (e.g. 16 RF, 32 RF, etc.), so the mismatch between the IoT NTN TDD configuration and the periodicity of MIB/SI/SIB1/Paging may bring some issues.

### 2.1.1	MIB-NB

Currently, the periodicity of MIB-TDD-NB is 640 ms, and repetition is made within 640 ms in the fixed subframe, as follows [2]:

The MasterInformationBlock-TDD-NB (MIB-TDD-NB) uses a fixed schedule with a periodicity of 640 ms and repetitions made within 640 ms. The first transmission of the MIB-TDD-NB is scheduled in subframe #9 of radio frames for which the SFN mod 64 = 0 and repetitions are scheduled in subframe #9 of all other radio frames. The transmissions are arranged in 8 independently decodable blocks of 80 ms duration.

Obviously, within one period, the MIB should be broadcast in subframe #9 every radio frame within 8 radio frames (i.e. 80 ms). However, in the 90 ms IoT NTN TDD frame structure, the DL channel is only kept for totally 8 ms as per RAN1 agreement. As a result, the subframes, which are determined by the current Spec to broadcast MIB with above periodicity and repetition pattern, are not ensured to be the DL channel with this 90 ms TDD configuration.

The following agreement was further achieved in RAN1#120 meeting [3]:

NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - FFS: handling for SIB1-NB.

That is, if the subframes determined by the current Spec to broadcast MIB are non-D NB-IoT subframes, the MIB-TDD-NB transmission will be dropped. RAN2 can just follow the agreement of RAN1, and discuss whether to capture in the RRC Spec that UE skips the MIB reception if the NPBCH occasion is a non-D NB-IoT subframe

Proposal 1: For MIB-NB, RAN2 follows the agreement of RAN1 and discusses whether to capture in the system information procedure in TS 36.331 that the UE skips the MIB-NB reception if the NPBCH occasion determined based on the current Spec is a non-D NB-IoT subframe.

### 2.1.2	SIB1-NB

For TDD, SIB1-NB transmission on the anchor carrier occurs in either subframe #0 or subframe #4 of every other frame in 16 continuous frames and SIB1-NB transmission on a non-anchor carrier occurs in subframe #0 and next in subframe #5 of every other frame in 16 continuous frames. The starting frame for the first transmission of the SIB1-NB is derived from the cell PCID and the number of repetitions within the 2560 ms period and repetitions are made, equally spaced, within the 2560 ms period (see TS 36.213 [23]). TBS for SystemInformationBlockType1-NB, the repetitions made within the 2560 ms, and the subframe index (#0 or #4) are indicated by schedulingInfoSIB1 field in the MIB-TDD-NB

The SIB1 is broadcast with fixed period (2560 ms) and repetition with fixed subframe (e.g. either subframe #0 or subframe #4 of every other frame in 16 continuous frames). Similar as MIB, it cannot be ensured that the subframes needed for SIB1-NB transmission as per the current Spec are really the DL channel with this 90 ms TDD configuration. Besides, RAN1 has left FFS on SIB1-NB in RAN1#120 meeting as shown by the RAN1 agreements above. So RAN2 needs to further wait for the RAN1 progress on SIB1-NB handling before RAN2 further discusses this issue.

Observation 1: For the SIB1-NB transmission in the newly defined TDD frame structure, RAN2 waits for further RAN1 progress.

### 2.1.3	Other SI

SI is broadcast and repeated within SI window with the configured periodicity and repetition pattern. In the current Spec, the length of SI window can be configured with the following values [2]:

si-WindowLength-r13					ENUMERATED {ms160, ms320, ms480, ms640,

ms960, ms1280, ms1600, spare1},

And the periodicity and the repetition pattern can be configured with following values:

si-Periodicity-r13				ENUMERATED {rf64, rf128, rf256, rf512,

rf1024, rf2048, rf4096, spare},

si-RepetitionPattern-r13		ENUMERATED {every2ndRF, every4thRF, every8thRF, every16thRF},

Within the SI-window, the corresponding SI message can be transmitted several times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS. For example, the SI is broadcast within 8 consecutive NB-IoT downlink subframes every 2 frames. Based on the 90 ms TDD configuration, it is also difficult to ensure that the subframe and frame used to transmit SI as determined by the current Spec are really DL channel.

RAN1 has achieved some agreements for the NPDSCH transmission at last meeting, with however the SI case kept open:

Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

RAN1 also reached the following conclusion in RAN1#120 meeting:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

From RAN2 perspective, we think that the simple solution to deal with other SI can also be that if there are subframes in an SI window (as determined by the current Spec) that are non-D NB-IoT NTN subframes, the UE may skip the corresponding SI reception on these subframes. RAN2 may discuss whether to capture this in RRC (e.g. system information procedure) or rely on RAN1 Spec (e.g. whether RAN1 intends to specify a common UL/DL subframe skipping mechanism in PHY on the non-DL/non-UL slots).

Proposal 2: RAN2 discusses whether to capture in RRC Spec (system information procedure) that if a subframe is within the SI window as determined by the current Spec but is a non-DL subframe, UE skips the reception of the corresponding SI thereon.

### 2.1.4	Paging

The UE monitors paging message based on the configuration of paging cycle, nB-r13 and repetition [2]:

PCCH-Config-NB-r13 ::=					SEQUENCE {

defaultPagingCycle-r13					ENUMERATED {rf128, rf256, rf512, rf1024},

nB-r13									ENUMERATED {

fourT, twoT, oneT, halfT, quarterT, one8thT,

one16thT, one32ndT, one64thT,

one128thT, one256thT, one512thT, one1024thT,

spare3, spare2, spare1},

npdcch-NumRepetitionPaging-r13			ENUMERATED {

r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

}

Based on the current spec, the density of the PO ranges from every frame to every 1024 radio frames. Similar as above discussion, the 90 ms TDD configuration may not be able to support the POs determined by the current Spec.

Observation 2: RAN2 waits for further RAN1 progress before carrying out the discussion on how to adapt the paging procedure to the 90 ms IoT NTN TDD frame structure.

### 2.1.5	Access control

According to the objective of the WID [1], in the newly defined NB-IoT TDD mode UE will work in the targeted unpaired MSS allocated band (1616-1626.5 MHz MSS allocated band), and TN deployment is not expected in this band. So companies may think the legacy cellBarred bit which was used for the TN UE access control can be used to control the NB-IoT TDD mode UE to access the cell defined in the targeted 1616-1626.5 MHz MSS allocated band. RAN2 has achieved the following agreement on this issue:

 Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

On the other hand, we think that, for the NTN network, a unified access control mechanism should be used at least for the NTN UEs. That is, cellBarred-NTN should also be used in this case. One more bit broadcast does not matter much from overhead perspective.

Proposal 3: cellBarred-NTN can be used for access control in NB-IoT NTN TDD mode for legacy IoT NTN UEs.

Potential impacts on UP

### 2.2.1	Potential issues on related timer/window handling in MAC

A potential issue we found is that the MAC windows/timers related to the reception of NW response may not adapt to the 90 ms IoT NTN TDD frame structure as agreed by RAN1. In particular, considering the agreements by RAN1, the related timer/window length may cover too limited DL occasions for the UE to receive NW response, so that the UE may end up with inefficient/unexpected handling upon the timer/window expiry. As some examples: for RAR window and contention resolution timer, if the NW has not enough DL transmission occasions to send the responses (with only one DL slot having to send all DL signals in the 90ms periodicity), it is likely that some UEs end up with RACH failure per the current Spec and enter exceptional handling; for DRX, the UE may inefficiently start the on duration timer and wake up again and again, but end up with receiving nothing in most of the DRX cycles; for retxBSR-Timer, if the UE cannot receive DL scheduling timely, it may end up with the timer expiry and unexpected BSR triggered again, even if the NW has already planned to schedule the UE in future 90 ms frame based on the previous BSR already received from the UE.

Although in the current Spec some timers are already with a configurable values that can cover several 90 ms periodicities of the IoT NTN TDD radio frame, it is still uncertain whether these existing values are already sufficient to overcome such unexpected/efficient handling due to the limited availability of DL occasions (esp. considering that those existing values were absolutely not introduced due to the reasons of this 90 ms IoT NTN TDD frame periodicity). Therefore, RAN2 may need to discuss whether/how to enhance the related MAC timers/windows to adapt to this 90 ms IoT NTN TDD radio frame periodicity, e.g. introducing longer timer/window length, consider the length in the number of DL slot numbers, etc.

Proposal 4: RAN2 discusses whether/how to enhance related MAC timers/windows to adapt to the 90 ms IoT NTN TDD radio frame structure (e.g. RAR window, CR timer, retx BSR timer, DRX timer, etc.).

### 2.2.2	Potential issues on periodical operations in MAC

For NB-IoT, UL Semi-Persistent Scheduling configuration is supported for BSR transmission per Serving Cell. Per the current specification, the values configured for SPS periodicity for NB-IoT are not multiples of 90 ms. Further considering RAN1 agreements that only one UL slot is usable within a 90 ms period, it may happen that there is no uplink subframe on the UL SPS occasion.

SR-SPS-BSR-Config-NB-r15 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

semiPersistSchedC-RNTI-r15			C-RNTI,

semiPersistSchedIntervalUL-r15		ENUMERATED {sf128, sf256, sf512, sf1024,

sf1280, sf2048, sf2560, sf5120}

}

}

Similar situation is faced by PUR. We can see that in the current Spec, preconfigured uplink resource can be configured by the network with specific periodicity and offset, but the configurable periodicity values for PUR resources are not multiple of 90 ms either.

PUR-PeriodicityAndOffset-NB-r16 ::=	CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..257),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

}

RAN1 has made the following agreements about NPUSCH mapping:

Agreement The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NPUSCH is mapped as per current specifications. - NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure.

Therefore, SPS and PUR can follow the same rules for NPUSCH given by RAN1 above.

Proposal 5: For SPS/PUR, RAN2 discusses whether to capture in the MAC Spec that UE skips an SPS/PUR occasion if it is a non-UL slot.

### 2.2.3	Potential issues on SFN wrap around

(H-)SFN plays a key role in MAC. For example, SFN_id and H-SFN are required for RA-RNTI calculation for NB-IoT in TDD mode.

For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

Apart from RA-RNTI calculation, SFN is also needed to determine when to start onDurationTimer.

- if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset: - if NB-IoT: - if there is at least one HARQ process for which neither HARQ RTT Timer nor UL HARQ RTT Timer is running, start onDurationTimer. - else: - start onDurationTimer.

Considering that the SFN is of 10-bit length with 1024 as maximum value (0-1023), the TDD pattern needs to repeat every 10240 msec. If the H-SFN is enabled, the TDD pattern may need to repeat every 10240*1024 msec. Considering 90 ms periodicity frame structure is adopted for TDD in IoT-NTN, the SFN wrap around periodicity is not a multiple of 90 ms (e.g. with only a 70 ms left at the very end as shown in Figure 1), which means that subset of UL/DL subframes may be different in different hyper frames.

Observation 3: The SFN wrap around periodicity is not a multiple of the periodicity of IoT-NTN TDD frame, which may bring impacts on SFN wrap around.

Figure 1: SFN and periodicity issue in TDD for IoT-NTN

In RAN1#119 meeting, RAN1 made the following agreements:

Agreement The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Hence, RAN2 can wait for further inputs from RAN1 to address SFN wrap around issue.

Proposal 6: RAN2 can wait for further inputs from RAN1 to address SFN wrap around issue.

Conclusion

In this contribution, the issue of RAN2 impact of TDD is discussed, with the following observations and proposals:

Observation 1: For the SIB1-NB transmission in the newly defined TDD frame structure, RAN2 waits for further RAN1 progress.

Observation 2: RAN2 waits for further RAN1 progress before carrying out the discussion on how to adapt the paging procedure to the 90 ms IoT NTN TDD frame structure.

Observation 3: The SFN wrap around periodicity is not a multiple of the periodicity of IoT-NTN TDD frame, which may bring impacts on SFN wrap around.

Proposal 1: For MIB-NB, RAN2 follows the agreement of RAN1 and discusses whether to capture in the system information procedure in TS 36.331 that the UE skips the MIB-NB reception if the NPBCH occasion determined based on the current Spec is a non-D NB-IoT subframe.

Proposal 2: RAN2 discusses whether to capture in RRC Spec (system information procedure) that if a subframe is within the SI window as determined by the current Spec but is a non-DL subframe, UE skips the reception of the corresponding SI thereon.

Proposal 3: cellBarred-NTN can be used for access control in NB-IoT NTN TDD mode for legacy IoT NTN UEs.

Proposal 4: RAN2 discusses whether/how to enhance related MAC timers/windows to adapt to the 90 ms IoT NTN TDD radio frame structure (e.g. RAR window, CR timer, retx BSR timer, DRX timer, etc.).

Proposal 5: For SPS/PUR, RAN2 discusses whether to capture in the MAC Spec that UE skips an SPS/PUR occasion if it is a non-UL slot.

Proposal 6: RAN2 can wait for further inputs from RAN1 to address SFN wrap around issue.

Reference

- RP-243264 Revised WID on introduction of IoT-NTN TDD_cl

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"

- Draft Report of 3GPP TSG RAN WG1 #120 v0.3.0