3GPP TSG-RAN WG2 Meeting #129bis	R2-2502069

Wuhan, China, Apr. 7th – 11, 2025

Agenda Item:	8.9.3

Source: 	OPPO

Title:  	Discussion on CB-msg3 EDT and msg4 enhancement

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the following agreements were made for uplink capacity enhancement.

Agreements: 1. It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used 2. RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC 3. RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS 4. For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. 5. The UE shall at most have one ongoing CB EDT procedure at any time. 6. The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. 7. The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. 8. CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. 7. As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. 8. RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: 1. One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) 2. For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19

In this contribution, we would like to provide our considerations on open issues for CB-msg3 EDT and msg4 enhancement.

Discussion

Configuration

In RAN2#128 meeting, how to provide CB-msg3 EDT related configurations was discussed and the following agreement was made.

- Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

Further discussion is needed on whether dedicated signalling is required to provide TA validation parameters for the case of 15kHz SCS NB-IoT and eMTC CE modeB. In our understanding, It should rely on RAN1/RAN4 discussion, i.e., in their previous reply LS, it is noted that the pre-compensated TA is sufficient for CB-msg3 transmission in case of NB-IoT with 3.75kHz SCS and eMTC CE mode A, further discussion is needed for NB-IoT with 15kHz SCS and eMTC CE mode B.

1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B

Since RAN2 may not be able to evaluate the case, it is suggested to send an LS to RAN1/RAN4 to facilitate progress.

Send an LS to RAN1/RAN4 to ask whether TA validation parameters are needed for the case of 15kHz NB-IoT and eMTC CE mode B.

In last RAN2 meeting, CB-msg3 EDT using OCC was discussed based on the LS from RAN1 and the agreements were made as follows:

1. It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used 2. RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

For the FFS, if the same CB-msg3 EDT resources are applied, the OCC UEs and non-OCC UEs will interfere with each other due to the use of different resource mapping rules and the orthogonality required by OCC UEs will be broken by non-OCC UEs. Therefore, separate CB-msg3 EDT resources should be configured for OCC and non-OCC.

Separate CB-msg3 EDT resources are configured for OCC and non-OCC.

For EDT, the UE needs to check the TBS-threshold associated with the selected level when determining whether to trigger the procedure. For CB-msg3 EDT, it was also agreed that the TBS needs to be considered as one of the trigger conditions.

- The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

Regarding whether the maximum TBS is the same or different for different CE level, it is preferred to follow the same way as EDT for flexibility.

TBS-threshold is configured per CE level, whether to configure the same value is up to the NW.

For NB-IoT, the EDT resources can be configured on anchor carrier and non-anchor carriers for each CE level as specified in TS36.321. Also for load balance purpose, the same way of configuration can be followed for CB-msg3 EDT.

- for EDT, each enhanced coverage level has zero or one anchor carrier PRACH resource present in nprach-ParametersList-EDT and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

For NB-IoT, the CB-msg3 EDT resources can be configured on anchor and/or non-anchor carriers for a CE level.

For NB-IoT UEs, the SCS of the UL grant in RAR can be configured as 3.75kHz or 15KHz and the NW selects the numerology to be scheduled based on the radio link quality. To accommodate UEs operating under different radio link conditions, it is necessary to configure CB-msg3 EDT resource with multiple numerologies.

For NB-IoT, the CB-msg3 EDT resources associated with a CE level can be configured with different numerologies (e.g. 3.75kHz SCS and 15kHz SCS).

DL response reception

For SA case, it was agreed to rely on a window for DL response reception after CB-msg3 PUSCH transmission and further discussion is needed on whether to consider the processing time when starting the reception window.

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

For PUR, 4 subframes processing delay was considered when starting the DL response reception window, i.e., pur-ResponseWindowTimer.

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes.

For saving UE power consumption, processing delay should also be considered when starting the PDCCH monitoring window in CB-msg3 EDT. Different from PUR, the eNB may face greater challenges in processing the CB-msg3 because of contention, thus it is questionable whether 4 subframes is enough and we think the proper value should be decided by RAN1.

The processing time is considered for the start of PDCCH monitoring window. Up to RAN1 to determine the value.

As agreed, the Contention Resolution Identity should be included in CB-msg4 to declare the contention resolution.

9.  The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

If msg4 enhancement is not considered, CB-msg3 EDT should also be terminated by receiving DL RRC message as in EDT. Therefore, if there is no RRC message together with the CB-msg4, the UE needs to keep monitoring the RRC message after the contention resolution. In this case, a UE-dedicated RNTI, i.e., C-RNTI, used for follow-up RRC message monitoring needs to be provided in CB-msg4.

Besides, the C-RNTI is also required if the RRC message together with the CB-msg4 directs the UE to perform state transmission, e.g., fallback to legacy RRC Connection establishment/resume procedure.

C-RNTI needs to be included in CB-msg4 if RRC message is not delivered together CB-msg4 or the RRC message together with CB-msg4 triggers state transition.

For DSA, it was agreed that a configurable window will be applied for the selection of CB-msg3 PUSCH occasions. In last RAN2 meeting, whether to use a sliding transmission window or a fixed transmission window was further discussed as in [AT129][306] and the following working assumption was made.

2. For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19

As an FFS left in current MAC CR, it is unclear on how to design the reception window and the RNTI for DSA. The issue  was also discussed in [AT129][306] but no consensus was made. As summarized in R2-2501420, the following options are on the table for further dicussion .

For CB-Msg3 DSA monitor window and RNTI design, RAN2 to discuss below two options: - Option 1 - RNTI is calculated for each Replica respectively according to the selected resource (same as SA). - After the end of all repetition of CB-Msg3 PUSCH transmission of each replica, UE starts the corresponding monitor window, taking UE-eNB RTT into account. - The UE has to monitor multiple RNTIs in multiple monitor windows. - Option 2 - A single RNTI is used for all replicas and it is calculated based on the location of the selected DSA transmission window. - A single monitor window is used for all replicas.

If a single RNTI is calculated based on the location of the selected DSA transmission window, it will introduce additional effort for the UE to decode every CB-msg4 payload to check whether there is a matching contention resolution ID. Conversely, if the RNTI used for DL reception is calculated based on the PUSCH resources associated with each replica, the UE only needs to decode the CB-msg4 payload scheduled by the matching RNTI. From this perspective, we prefer to go with the solution that the RNTI is calculated for each replica according to the selected resource (same as SA).

For the monitoring window, if only one window is used, the UE would need to keep monitoring the DL responses addressed to all these RNTI during the time window, leading to increased power consumption. Therefore, it is preferred to have multiple PDCCH monitoring windows. When the contention is considered as resolved, the UE stops all the windows.

For DSA, RNTI is calculated for each replica based on the selected CB-msg3 PUSCH occasion.

For DSA, separate PDCCH monitoring window is started for each replica, during which the UE monitors the corresponding RNTI.

When the contention is resolved, the UE stops all the PDCCH monitoring windows.

Efficient delivery of msg4

As per WID, the other aspect on EDT enhancement is to support efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete. In last RAN2 meeting, a working assumption was made to support multiplexing of CB-msg4.

1. One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

In NR 2-step RACH, MsgB can include the feedbacks to multiple UEs by containing the following MAC subPDUs:

- a MAC subheader with Backoff Indicator only;

- a MAC subheader and fallbackRAR; (fallbackRAR is included in case that the MsgA payload is not successfully received by the NW. The UE fallbacks to msg3 (re-)transmission upon reception of fallbackRAR)

- a MAC subheader and successRAR; (successRAR is included in case the contention is resolved)

- a MAC subheader and MAC SDU for CCCH, DCCH or DTCH; (DL signalling/data can be included for the UE with successful contention resolution)

- a MAC subheader and padding.

By taking MsgB format as reference, we understand at least the following contents can be considered for the design of multiplexed CB-msg4.

- Backoff indication for contention control.

- Feedback to indicate a successful contention resolution, i.e., Contention resolution ID, C-RNTI.

- Feedback to indicate CB-msg3 retransmission, i.e., UL grant, RNTI for follow-up DL monitoring.

- RRC message to indicate the termination of CB-msg3 EDT, i.e., SDU for CCCH.

- Padding.

In multiplexed CB-msg4, at least the following information can be included:

  - Backoff indication.

  - Success contention resolution indication, e.g., contention resolution ID, optionally C-RNTI.

  - Re-transmission indication, e.g., UL grant, RNTI for follow-up DL monitoring.

  - RRC message.

  - Padding.

As agreed, the RNTI is derived based on the selected PUSCH occasion.

7. The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

To support CB-msg4 multiplexing, RNTI calculation should not include all the elements that determine a PUSCH occasion, i.e., partial elements are used for RNTI calculation, the rest is indicated in CB-msg4 payload to further distinguish different UEs. Given that we will have OCC UEs and non-OCC UEs in the same cell/CE level, it is observed that whether a common CB-msg4 format is used will have impact on the RNTI design, i.e., whether the OCC is considered in RNTI calculation or included in CB-msg4. Our consideration are as follows:

- If common CB-msg4 is used, OCC info can be either considered RNTI calculation or included in CB-msg4 since OCC UEs and non-OCCs will have a common understanding on how to perform CB-msg4 reception.

- If separate CB-msg4 is used, the OCC index does not need to be included in CB-msg4 since it is useless for non-OCC UEs. Further discussion is needed on how to differentiate the CB-msg4 with different formats, e.g., OCC is considered in RNTI calculation so that different RNTIs can be derived for OCC and non-OOC

RAN2 discuss whether common or separate CB-msg4 format is used for OCC UEs and non-OCC UEs.

In legacy EDT, after UE transmit UL data together with RRCEarlyDataRequest or RRCConnectionResumeRequest message in Msg3, the UE will not consider the EDT as successfully completed until it receives RRCEarlyDataComplete or RRCConnectionRelease message, i.e. only RRC message can be used to indicate the successful reception of the EDT transmission by the eNB.

For PUR, DL response optimization is specified for CP solution, i.e., eNB may terminate the PUR procedure by sending a L1 ACK optionally containing a Time Advance Command, a MAC Time advance Command or RRC response message with no data. By introducing L1/L2 ACK, the performance of resource efficiency, UE power consumption as well as MO latency can be further improved compared to EDT.

For CP-solution EDT, if there is no follow-up DL data, L1/L2 signalling can be considered for improvement of msg4 efficiency. The following solutions can be considered for further discussion:

- Solution1: If the NW is able to determine there is no follow-up DL data when responding msg4, the EDT termination indication can be delivered to the UE together with the contention resolution ID MAC CE.

- Solution2: If there is no follow-up DL data after contention resolution, L1 signalling scheduling by UE-dedicated RNTI  can be used to indicate EDT termination.

In case there is no follow-up DL data for CP-solution EDT/CB-msg3 EDT, the following solutions can be considered for msg4 enhancement:

  - L1 ACK scheduled by UE-dedicated RNTI.

  - A new MAC CE including contention resolution ID and EDT termination indication.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	Send an LS to RAN1/RAN4 to ask whether TA validation parameters are needed for the case of 15kHz NB-IoT and eMTC CE mode B.

Proposal 2	Separate CB-msg3 EDT resources are configured for OCC and non-OCC.

Proposal 3	TBS-threshold is configured per CE level, whether to configure the same value is up to the NW.

Proposal 4	For NB-IoT, the CB-msg3 EDT resources can be configured on anchor and/or non-anchor carriers for a CE level.

Proposal 5	For NB-IoT, the CB-msg3 EDT resources associated with a CE level can be configured with different numerologies (e.g. 3.75kHz SCS and 15kHz SCS).

Proposal 6	The processing time is considered for the start of PDCCH monitoring window. Up to RAN1 to determine the value.

Proposal 7	C-RNTI needs to be included in CB-msg4 if RRC message is not delivered together CB-msg4 or the RRC message together with CB-msg4 triggers state transition.

Proposal 8	For DSA, RNTI is calculated for each replica based on the selected CB-msg3 PUSCH occasion.

Proposal 9	For DSA, separate PDCCH monitoring window is started for each replica, during which the UE monitors the corresponding RNTI.

Proposal 10	When the contention is resolved, the UE stops all the PDCCH monitoring windows.

Proposal 11	In multiplexed CB-msg4, at least the following information can be included:

a.	Backoff indication.

b.	Success contention resolution indication, e.g., contention resolution ID, optionally C-RNTI.

c.	Re-transmission indication, e.g., UL grant, RNTI for follow-up DL monitoring.

d.	RRC message.

e.	Padding.

Proposal 12	RAN2 discuss whether common or separate CB-msg4 format is used for OCC UEs and non-OCC UEs.

Proposal 13	In case there is no follow-up DL data for CP-solution EDT/CB-msg3 EDT, the following solutions can be considered for msg4 enhancement:

a.	L1 ACK scheduled by UE-dedicated RNTI.

b.	A new MAC CE including contention resolution ID and EDT termination indication.