3GPP TSG-RAN WG2 Meeting #129-bis                                      R2-2502082

Wuhan, China, Apr 7th – 11th, 2025

Source:        ZTE Corporation

Title:           Discussion on random access procedure in SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The WID [1] of Rel-19 SBFD related to random access is given as below:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

This contribution provides our view on random access procedures in SBFD technique.

- Discussion

SBFD RACH configuration in NUL carrier

Legacy RACH configuration can be provided within UL BWP configuration, where the UL BWP can be allocated on NUL carrier or SUL carrier. For legacy RACH configuration in NUL, ROs in DL slot/symbol is considered as invalid; however, SUL carrier can always work even in D slot/symbol, i.e., UE can perform RA in SUL regardless of cell-level TDD pattern.

For SBFD, UL subband should only be configured within DL carrier, and SBFD symbol should only be legacy D or F slot/symbol. If UE performs random access in SBFD symbols, the ROs should occupy or overlap with both SBFD symbol (time domain) and UL subband (frequency domain). As the fact that SUL usually has non-overlapping lower frequency than NUL DL carrier to provide larger UL coverage, in order to occupy or overlap with resources in the UL subband, ROs should be configured within NUL carrier, not SUL carrier.

Figure 1. Example relationship between UL subband, DL carrier, NUL carrier in a TDD system

Proposal 1: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs

RAN2#127 has made the following agreements to discuss the RACH configuration in RRC signaling:

RAN2#127 Agreement RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

Wherein the SBFD RACH configuration in the SIB is for RRC_INACTIVE/IDLE UE to perform CBRA (or CFRA for SI request), and in dedicated RRC signaling is for RRC_CONNECTED UE to perform CFRA (or CBRA when fallback).

CBRA RACH resource is provided in SIB1 -> BWP-UplinkCommon for RRC_IDLE/INACTIVE UEs, or in RRCReconfiguration -> BWP-UplinkCommon for RRC_CONNECTED UEs. This section provides our thoughts on the RRC RACH configuration for SBFD-aware UEs in CBRA.

RACH configuration for option 2

In RAN1#118 meeting, the further agreement of RACH configuration option 2 is achieved as follows:

Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

This means, for option 2, only SBFD RO will be the valid RO type.

Observation 1: RACH configuration Option 2 can only provide valid SBFD ROs.

Separate RACH configuration provides independent basic RACH parameters compared with the legacy RACH configuration. So the valid RO of derived from separate RACH configuration and legacy RACH configuration can be totally in different pattern to increase the UL coverage, reduce the latency, etc.

According to the agreement in RAN2#127-bis, a new RACH configuration dedicated for SBFD-aware UE for option 2 will be needed:

Agreement in RAN2#127-bis For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. FFS dedicated RRC signalling detail.

We suggest to add the SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, like the following figure:

Figure 2. Recommended CBRA RACH configuration option 2

This way is to configure option 2 using similar principle of introducing 2-step RACH configuration, i.e., the new SBFD RACH configuration is also contained in each element in AdditionalRACH-ConfigList.

Proposal 2: For SBFD RACH configuration option 2 in CBRA, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

RO type selection

RAN2#129 has agreed to select RO type based on a NW indication or a RSRP threshold:

Agreement For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

For RSRP determination, we should strive to achieve an efficient and simple way, i.e., our view is that there is no need to design complicated criteria for RO selection. The main concern is the CLI interference on SBFD ROs, so only one RSRP threshold is enough, and only above the configured RSRP threshold can UE selects the SBFD RO to reduce the inference. If UE is at cell edge, the Msg1 repetition on legacy ROs can also achieve the benefit of using SBFD RO to increase coverage. Thus, no need to consider the RSRP-below-threshold case and no additional indicator is needed.

Furthermore, the spec will not have restriction that option 2 has to provide a larger preamble format. From spec perspective, option 2 just provides another way of SBFD RACH configuration with several/all RACH parameters to be different than legacy parameters. So there is also no need to associate the criteria with RRC configuration options.

Proposal 3: RAN2 to support only one configured RSRP threshold for RO selection, and UE selects SBFD RO when the measured RSRP is above the configured RSRP threshold. No additional indicator is needed.

The configured RSRP threshold should not be mandatory signaling to achieve the better flexibility. When the RSRP threshold is also absent and gNB provides SBFD RACH configuration, SBFD-aware UE should firstly select SBFD RACH configuration to balance the load, and to achieve early identification at gNB side, in order to reduce latency for the further Msg3/UL NAS message scheduling.

Proposal 4: When the RSRP threshold is absent, SBFD-aware UE firstly selects SBFD RO.

MAC order of RO type selection

Based on the analysis above, for CBRA, UE can select RO type based on some criteria. It is RAN2’s responsibility to study the impact of such selection on the current MAC RA procedure order/flow.

The current RA procedure in MAC spec is as follows:

Figure 3. Current MAC order of initiating RA procedure

The RACH configuration selection/RO type selection can happen before resource set selection, or after resource set selection.

In our view, selecting set according to feature/feature combination is more important than selecting RO type, since UE has to let gNB recognize some feature in Msg1/A (early identification), otherwise there will be problem in the next scheduling. For example, a RedCap UE should firstly select a set of feature ‘RedCap’ to transmit preamble, then gNB will schedule different UL resources for the RedCap UE in the upcoming Msg3. So, it is essential that UE selects a set corresponding to all the feature it wants as much as possible. Compared with this motivation, using SBFD RO or legacy RO is only different implement ways of initiating RA. Using SBFD RO is a ‘nice to have’ move to achieve low latency or larger coverage, but it is not as much as essential to the early identification.

If RO type selection is before set selection, there can be a case the selected RO type does not associate with any feature that the UE wants, since the RACH partition configuration given by RRC may be different on RACH configuration and SBFD RACH configuration. Therefore, selecting RO type before set selection may reduce the scope of the RACH resources corresponding to the wanted feature/feature combination.

The case of support Msg1 repetition is special since the validation of Msg1 repetition in CBRA needs to be evaluated by UE before set selection. Please see the analysis of the MAC order of Msg1 repetition in section 2.4.1.

Observation 2: Selecting RO type before set selection may reduce the scope of the RACH resources corresponding to the wanted feature/feature combination.

Figure 4. The example MAC order of RO type selection

Therefore, UE should first verify the feature. If there is only one set satisfy all the UE wanted features (expect the Msg1 repetition), no matter the set is SBFD or legacy, UE should select the set; only if there is a set associated with features that are applicable in 2 RO types, UE should select the set and UE should further selects RO type.

This also means, if a selected RACH resource set of the UE’s wanted feature/feature combination (expect the Msg1 repetition) only contains SBFD RO or only contains legacy RO, the UE can only select the SBFD RO or the legacy RO without additional criteria. If a selected RACH resource set contains two kinds of ROs, UE selects the RO type based on the NW indication or the RSRP threshold.

Since RAN1 has agreed not to support 2-step SBFD RO in Rel-19, the RO type selection should be after RA type selection, i.e., only the UE selects 4-step RA according to legacy rules can UE further select RO type. This way has minimum impact to the current MAC structure.

Based on above, we propose:

Proposal 5: RO type selection should be after the set selection and after RA type selection.

Proposal 6: If Msg1 repetition is not available for both RO types, UE should select one set among all the UE’s configured sets:

- If the selected set only contains SBFD RO or legacy RO, the UE further selects the SBFD RO or the legacy RO directly, regardless of any of the RO type selection criteria;

- Only if the selected set contains two kinds of ROs, UE further selects the RO type based on the RO type selection criteria.

Fallback from legacy RO to SBFD RO

Last meeting there is a discussion on whether to support legacy RO fallback to SBFD RO.

The issue is that, for a SBFD-aware UE supporting 2-step RACH configuration, it the UE initially selects 2-step RACH resource, after msgA-TransMax-r16 times of failure, UE will fall back to try 4-step RACH resource for the rest of times. If the number of preamble transmission failure before fallback from legacy RO to SBFD RO is also configurable, then it is unclear that whether the UE should fall back to 4-step RACH resource, or fall back to 4-step SBFD RACH resource. Additional spec impact will be needed to solve this issue.

Another spec impact would be that, if fallback from 2-step RACH resource -> 4-step RACH resource -> 4-step SBFD RACH resource is supported, the maximum preamble transmission number should also be increased, to ensure the same RA successful rate.

Figure 5. Fallback from legacy RO to SBFD RO when 2-step RACH is also configured

In summary, we see the benefit of supporting switching from legacy RO to SBFD RO, but issues mentioned above should be well solved. Therefore we propose:

Proposal 7: If switching from legacy RO to SBFD RO is supported, RAN2 to solve the issue that whether/how UE switches from 2-step RACH resource to the 4-step RACH resource or to the 4-step SBFD RACH resource.

Early identification of SBFD-aware UE

RAN2#127-bis meeting has agreed the following:

Agreement RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

That is to say Msg1 based early identification is not supported in legacy ROs. The question is whether to support Msg3 based early identification when legacy RO is selected.

Figure 6. Example of initial access timeline

Firstly, as the figure indicates, if early identification is achieved in Msg3, the benefit is only on Msg5 UL, NAS security UL and AS security UL (see green arrows), these UL transmission can be scheduled on UL subband. When gNB acquires UE capability, the support of SBFD feature will be directly known by gNB. However the NAS security and AS security set up procedure is only applicable in RRC_IDLE, i.e., even if UE resumes from RRC_INACTIVE, these two steps are not needed. So the scenario of scheduling UL transmission in UL subband becomes more limited.

Secondly, Msg3 based early identification will achieve less benefit than Msg1 based early identification. If companies thinks early identification is not an urgent feature to support for Msg1 based early identification, then Msg3 based early identification shouldn’t be supported at all due to the lower benefit.

Based on analysis 2.2.2, if the NW indication and RSRP threshold are not configured, SBFD-aware UE is better to use SBFD RO by default. So early identification can also be achieved when SBFD-aware UE always selects SBFD RO in Msg1/A, the motivation of enhancing Msg3 is further weakened.

Based on the above, we have the following proposal:

Proposal 8: Do NOT support Msg3 based early identification for SBFD-aware UE.

CFRA in SBFD ROs

RO type indication in CFRA

RAN2#128 has achieved the following agreement:

Agreement The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

The signaling is FFS. gNB configures different CFRA RACH resource for different CFRA scenarios, using RRC/MAC CE/DCI signaling. To be specific:

- For UE performing CFRA for BFR, the CFRA RACH configuration is provided in BWP-UplinkDedicated - BeamFailureRecoveryConfig;

- For UE performing CFRA for ReconfigurationWithSync, the CFRA RACH configuration is provided in ReconfigurationWithSync - rach-ConfigDedicated;

- For UE performing CFRA for LTM early sync, the CFRA RACH configuration is provided in RRCReconfiguration - LTM-Config -  LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig;

- For UE performing CFRA for PDCCH order, the corresponding DCI can also be used to indicate some CFRA resource;

- For UE performing CFRA for LTM switch, a LTM cell switch commend MAC CE is introduced to indicate CFRA resource in LTM:

TS38.321-i30: - 6.1.3.75 LTM Cell Switch Command MAC CE Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

Therefore, the RO type indication can be given via RRC signaling, DL MAC-CE or DCI:

- In RRC signaling, each CFRA RACH configuration is associated with a RACH-ConfigGeneric, the RACH-ConfigGeneric can naturally contain SBFD RO and non-SBFD RO, since the RO pattern is configured periodical and has nothing to do with the TDD pattern. So one simple way is to add an indication in each RRC-configured CFRA RACH configuration to say that, whether UE should only consider SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid, or UE should only consider non-SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid. To be specific:

  - For the BFR CFRA resource, the RO type indication can be configured in BWP-UplinkDedicated – BeamFailureRecoveryConfig;

  - For the ReconfigurationwithSync CFRA resource, the RO type indication can be configured in ReconfigurationWithSync - rach-ConfigDedicated;

  - For the LTM early sync CFRA resource, the RO type indication can be configured in RRCReconfiguration - LTM-Config - LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig. However the early sync is triggered by PDCCH order, and the PDCCH order has already contained a RO type indication. RAN2 should further dicsuss whether the corresponding RRC configuration needs the indication or not.

- For CFRA resource provided via LTM cell switch MAC CE, the RO type indication be added in the MAC CE that, whether the RA triggered by the MAC CE can only be performed on SBFD RO;

- For PDCCH order, RAN1#118-bis meeting has the following agreement to explicit indicate the RO type in PDCCH order:

Agreement For SBFD-aware UEs and PRACH mask indication for CFRA triggered by PDCCH order, discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used.

RAN2 should focus on RRC and MAC spec impact, and leave DCI/PDCCH order to RAN1 decision. So we have the following proposal:

Proposal 9: For RO type indication in CFRA, the following RRC and MAC spec impact should be considered:

- Add an RO type indication in BeamFailureRecoveryConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in rach-ConfigDedicated to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in LTM cell switch command MAC CE to indicate whether the RA triggered by the MAC CE can only be performed on SBFD RO.

FFS that whether to add an RO type indication in EarlyUL-SyncConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO.

CSI-RS used in SBFD RO in CFRA

Currently the CSI-RS beam can be used on legacy RO in BFR and ReconfigurationWtihSync kind of CFRA. We see no reason to block this for SBFD RO, i.e., when SBFD RO is indicated in CFRA, UE should also examine the availability of CSI-RSRP to determine whether the CSI-RS beam can be used or not. Further, there is no need to design separate SSB-RSRP or CSI-RSRP threshold than legacy, since UE measures same RS in advance.

Proposal 10: Support using CSI-RS beam for SBFD RO in BFR and ReconfigurationWtihSync kind of CFRA, and there is no need to design separate SSB-RSRP or CSI-RSRP threshold than legacy.

Fallback from CFRA to CBRA

In legacy RA mechanism, if CFRA resource is provided for the current RA event, UE should firstly try to use CFRA resource to initiate RA by comparing the measured RS-RSRP and configured power threshold. If the measured RS-RSRP is larger than the configured power threshold, UE should use the indicated CFRA resource; if not, UE should fall back to use the CBRA resource within the RACH resource set selected in advance. Each time when the current preamble transmission fails, UE should compare the measured RS-RSRP and configured power threshold to determine whether CFRA or CBRA should be used. The basic procedure flow is as following:

Figure 7. CFRA falls back to CBRA

Since there is no SBFD RO in legacy mechanism, UE keeps the same RO type when CFRA fallbacks to CBRA, i.e., in fallback CBRA, UE should select the same RO type as the RO type indicated by gNB for CFRA resource. We think the similar procedure should be applied here, i.e., if gNB indicates to use SBFD RO for the CFRA procedure, when CFRA falls back to CBRA, UE should also select SBFD RO for the CBRA procedure. That is to say, UE should NOT use the criteria to select between SBFD RO and legacy RO in CBRA, when the CBRA is a fallback from CFRA. The RO type selection criteria is only for pure CBRA case.

Proposal 11: When CFRA falls back to CBRA, if CFRA is indicated to perform on SBFD ROs, the fallback CBRA should also be performed on SBFD ROs.

Msg1 repetition in SBFD ROs

RAN1#118 has made the following agreement regarding Msg1 repetition in SBFD ROs:

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-Ros (not across additional-Ros and legacy Ros) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-Ros and legacy Ros) as in legacy releases.

It can be seen that Msg1 repetition in SBFD ROs is supported in RAN1, so RAN2 should discuss solutions and specification impacts on this aspect.

Msg1 repetition availability/repetition number determination on SBFD ROs

The rsrp-ThresholdMsg1-RepetitionNum2/4/8 is configured only for CBRA to determine whether Msg1 repetition should be applied, and which Msg1 repetition number should be applied.

As specified in TS38.321 5.1.1b, the basic procedure of Msg1 repetition on legacy ROs are:

Step 1: UE determines the Msg1 repetition feature availability and available Msg1 repetition number(s) on legacy ROs, based on one or more configured RSRP threshold rsrp-ThresholdMsg1-RepetitionNumX-r18:

- If current measured DL pathloss RSRP < any of configured rsrp-ThresholdMsg1-RepetitionNumX-r18, Msg1 repetition is applicable and X is an applicable Msg1 repetition number for current RA;

- If current measured DL pathloss RSRP ≥all of configured rsrp-ThresholdMsg1-RepetitionNumX-r18, Msg1 repetition is not applicable for current RA.

Step 2: UE selects a RACH resource set, according to the determined Msg1 repetition feature availability and determined available Msg1 repetition number(s):

- If there is only one configured set that is associated with all features that UE wants to apply, UE selects this set (in this case, UE does not care which Msg1 repetition number is associates with the set even though UE has determined a wanted Msg1 repetition number via RSRP threshold);

- If there are multiple sets with same feature/feature combinations that UE wants to apply, and each set is associated with different available Msg1 repetition numbers, UE selects the set that is associated with the highest repetition number;

- If there is no set that is associate with all features, but there are configured sets associated with subset of all features, UE selects one set according to priority rule in clause 5.1.1d.

Step 3: UE selects 4-step RA and UE performs Msg1 repetition on the selected set (with the determined Msg1 repetition number) and on legacy ROs.

RAN1#119 has further made following agreement regarding Msg1 repetition in SBFD ROs:

Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.  Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

When a new rsrp-ThresholdMsg1-RepetitionNumX is introduced for SBFD ROs, there can be two ways for determine the order of RO type + Msg1 repetition availability:

- Way 1: UE firstly determines the Msg1 repetition feature availability and Msg1 repetition number(s) on legacy ROs and on SBFD ROs separately, using separate RSRP threshold. For this approach, UE should select sets on both kinds of ROs, then after RA type selection (when UE selects 4-step RA), UE selects RO type and determine the final set based on RO type indication or RSRP.

- Way 2: UE firstly determine whether to perform SBFD RO or legacy RO, then determine the Msg1 repetition availability and number using the corresponding RSRP threshold.

Since Msg1 repetition derived from dedicated RSRP threshold should be performed on the corresponding RO type, for Way 1, UE can determine the same or different Msg1 repetition availability on different RO types, for example, based on current measured RSRP, UE needs Msg1 repetition in SBFD ROs, but does not need Msg1 repetition in legacy ROs. So the feature/feature combination on different RO type can be different, resulting in different sets within the scope of corresponding RACH configuration/SBFD RACH configuration.

For Way 2, UE can only select one set within all the sets configured for SBFD RACH configuration or within all the sets configured for legacy RACH configuration.

It can be seen, when Msg1 repetition is supported, if only select from the sets in the corresponding RACH configuration and only select from the sets in the corresponding SBFD RACH configuration, the final selected set derived from Way 1 and Way 2 contains same RACH resource or same SBFD RACH resource. That means if Msg1 repetition is to be verified/supported, Way 1 and Way 2 has same outcome. In addition to the analysis in 2.2.3, option 1 should be supported.

Observation 3: When Msg1 repetition is available, Way 1 ‘UE firstly select two sets then select RO type’ and Way 2 ‘UE firstly selects RO type then select one set’ will cause same RACH resource in the final selected set.

Proposal 12: In CBRA, for Msg1 repetition in SBFD ROs, the MAC order should be:

- Step 1: UE determines the Msg1 repetition feature availability and Msg1 repetition number(s) on legacy ROs and SBFD ROs separately (based on the separately configured RSRP threshold);

- Step 2: UE selects two sets of RACH resource corresponds to the determined Msg1 repetition feature availability and Msg1 repetition number(s) in two RO types, separately;

- Step 3: UE selects a RO type;

- Step 4: UE determines a final set, and performs Msg1 repetition using the determined Msg1 repetition number on the selected RO type on the final set.

Fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD RO

The other issue is whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA. In legacy mechanism, UE can fallback from lower repetition number to higher repetition number if several RA attempts fail, in order to increase the RA successful rate of the poor coverage UE. For the similar purpose, the UE should also be able to fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols.

The spec also restricts that UE should select a fallback set that has the same associated feature combination:

TS38.321-i40, 5.1.4: 2> if the Random Access procedure is not completed: 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

To reduce the specification effort, the fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols should also be associated the same feature combination, i.e., UE does not re-select RACH resource set when such fallback happens.

Proposal 13: Support fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols, and such fallback should be associated with the same feature/feature combination.

Fallback from SBFD RO to non-SBFD RO with Msg1 repetition

RAN2 has agreed fallback from SBFD RO to non-SBFD RO. RAN1/2 also agrees to perform Msg1 repetition on SBFD ROs. So RAN2 should further consider whether the two enhancement can work together or not, i.e., whether to allow UE firstly perform Msg1 repetition on SBFD ROs for several times and fallback from low Msg1 repetition number to higher Msg1 repetition number, then fallback to legacy ROs to continue Msg1 repetitions.

We think this is a reasonable and feasible solution to improve the RA successful rate. Therefore RAN2 should support this.

Proposal 14: Support UE firstly perform Msg1 repetition on SBFD ROs for several times and fallback from low Msg1 repetition number to higher Msg1 repetition number, then fallback to legacy ROs to continue Msg1 repetitions.

- Conclusion

In this contribution, we propose the following observation and proposals:

SBFD RACH configuration in NUL carrier

Proposal 1: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs

Observation 1: RACH configuration Option 2 can only provide valid SBFD ROs.

Observation 2: Selecting RO type before set selection may reduce the scope of the RACH resources corresponding to the wanted feature/feature combination.

Proposal 2: For SBFD RACH configuration option 2 in CBRA, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

Proposal 3: RAN2 to support only one configured RSRP threshold for RO selection, and UE selects SBFD RO when the measured RSRP is above the configured RSRP threshold. No additional indicator is needed.

Proposal 4: When the RSRP threshold is absent, SBFD-aware UE firstly selects SBFD RO.

Proposal 5: RO type selection should be after the set selection and after RA type selection.

Proposal 6: If Msg1 repetition is not available for both RO types, UE should select one set among all the UE’s configured sets:

- If the selected set only contains SBFD RO or legacy RO, the UE further selects the SBFD RO or the legacy RO directly, regardless of any of the RO type selection criteria;

- Only if the selected set contains two kinds of ROs, UE further selects the RO type based on the RO type selection criteria.

Proposal 7: If switching from legacy RO to SBFD RO is supported, RAN2 to solve the issue that whether/how UE switches from 2-step RACH resource to the 4-step RACH resource or to the 4-step SBFD RACH resource.

Proposal 8: Do NOT support Msg3 based early identification for SBFD-aware UE.

CFRA in SBFD ROs

Proposal 9: For RO type indication in CFRA, the following RRC and MAC spec impact should be considered:

- Add an RO type indication in BeamFailureRecoveryConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in rach-ConfigDedicated to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in LTM cell switch command MAC CE to indicate whether the RA triggered by the MAC CE can only be performed on SBFD RO.

FFS that whether to add an RO type indication in EarlyUL-SyncConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO.

Proposal 10: Support using CSI-RS beam for SBFD RO in BFR and ReconfigurationWtihSync kind of CFRA, and there is no need to design separate SSB-RSRP or CSI-RSRP threshold than legacy.

Proposal 11: When CFRA falls back to CBRA, if CFRA is indicated to perform on SBFD ROs, the fallback CBRA should also be performed on SBFD ROs.

Msg1 repetition in SBFD ROs

Observation 3: When Msg1 repetition is available, Way 1 ‘UE firstly select two sets then select RO type’ and Way 2 ‘UE firstly selects RO type then select one set’ will cause same RACH resource in the final selected set.

Proposal 12: In CBRA, for Msg1 repetition in SBFD ROs, the MAC order should be:

- Step 1: UE determines the Msg1 repetition feature availability and Msg1 repetition number(s) on legacy ROs and SBFD ROs separately (based on the separately configured RSRP threshold);

- Step 2: UE selects two sets of RACH resource corresponds to the determined Msg1 repetition feature availability and Msg1 repetition number(s) in two RO types, separately;

- Step 3: UE selects a RO type;

- Step 4: UE determines a final set, and performs Msg1 repetition using the determined Msg1 repetition number on the selected RO type on the final set.

Proposal 13: Support fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols, and such fallback should be associated with the same feature/feature combination.

Proposal 14: Support UE firstly perform Msg1 repetition on SBFD ROs for several times and fallback from low Msg1 repetition number to higher Msg1 repetition number, then fallback to legacy ROs to continue Msg1 repetitions.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06