3GPP TSG-RAN WG2 Meeting #129bis	R2-2502088

Wuhan, China, 7 – 11 April, 2025

Agenda item:	8.7.6

Source: 	Huawei, HiSilicon

Title: 	Discussion on XR rate control

Document for:	Discussion and Decision

1. Introduction

During the previous meetings, RAN2 agreed to support QoS flow level UL rate control for XR and to design a new table to meet the required range and granularity. In this contribution, we will further analyse the details for the QoS flow level rate control indication.

2. Discussion

2.1 Per QoS flow MAC CE v.s per DRB MAC CE

Although RAN2 has agreed on the QoS flow level rate control, there was controversy on the details of the indication signalling design. As shown by the following agreement, there are two options. One is to introduce per QoS flow MAC CE directly. The other is to indicate the QoS flows for rate control via RRC signalling and provide the recommended bit rate in per DRB MAC CE.

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

Comparing the two options, we think the first one is more aligned with the purpose of QoS flow level rate control, while the second one may have some problems. Specifically, if there are more than one QoS flow configured with rate control with the same DRB, when the gNB indicates the recommended bit rate in the DRB level MAC CE, the UE is not sure how to apply the recommended rate to the multiple QoS flows. It depends on the UE behaviour and may be out of network expectation. In the NR QoS mechanism, NG-RAN is responsible to ensure the QoS requirement on a per QoS flow level. If the gNB cannot control the rate of each QoS flow accurately, it would be hard to monitor and guarantee the QoS requirements.

Observation 1: Per DRB MAC CE is inaccurate for QoS flow level rate control and may cause some risk in QoS guarantee.

From the specification impact perspective, on the one hand, as responded by RAN3 in R3-250805 [1], both options have F1AP impacts. We suppose the impacts for the two options would be similar, i.e., the gNB-CU shall inform the gNB-DU about which QoS flows are subject to perform rate control. On the other hand, from the RAN2 point of view, both options would impact RRC (for configuration) and MAC (for dynamic indication) specification. In this way, we do not see any significant difference to implement the two options from the specification modelling perspective.

Excerption from R2-2501508: 1. Overall Description: RAN3 thanks RAN2 for the LS on XR UL bit rate control. RAN3 discussed it and agreed both options have an impact on the F1 interface. RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available.

Observation 2: Both options have similar specification impacts from both RAN2 and RAN3 perspective.

Some companies opposing the first option have concern on whether the gNB can provide the QoS flow level recommended bit rate in case of CU-DU split when multiple QoS flows are mapped to the same DRB. However, we think there is no issue. Actually, it should be feasible and up to the gNB-DU implementation to provide the QoS flow level recommended bit rate no matter how the QoS flows are mapped to the DRBs.

First, it is noted that F1AP is already able to transfer the QoS flow level QoS parameters to the gNB-DU via the UE context management messages, as the following content excerpted from TS 38.473 [2] shows. So that the gNB-DU can derive the recommended rate for each QoS flow based on the available resource and the QoS requirements. For example, assuming there are two QoS flows with their GFBR equal to 10Mbps and 20Mbps respectively, if the gNB-DU can provide up to 45Mbps for the QoS flows in total, then it can determine the recommended bit rate as 15Mbps and 30Mbps for each of the QoS flows.

### 9.2.2 UE Context Management messages #### 9.2.2.1 UE CONTEXT SETUP REQUEST This message is sent by the gNB-CU to request the setup of a UE context. Direction: gNB-CU gNB-DU. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. *** skipped part ***. >>>>>>QoS Flow Level QoS Parameters; M; 9.3.1.45; -. 9.2.2.7 UE CONTEXT MODIFICATION REQUEST This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU. Direction: gNB-CU gNB-DU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. *** skipped part ***. >>>>>>QoS Flow Level QoS Parameters; M; 9.3.1.45; -.

Another potential implementation is that the gNB-DU may monitor the actual bit rate of each QoS flow by taking statistics of the received DL or UL packets. Specifically, if there is only one QoS flow mapping to the DRB, the gNB-DU can measure the data rate for the DRB, which equals to the rate of the QoS flow. If there are multiple QoS flows mapping to the DRB, the gNB-DU can distinguish the packets belong to different QoS flows the SDAP header (for DL/UL) so that the rate of each QoS flow can be measured. It is worth noting that the CU can configure the DL/UL SDAP header to be included if N to 1 mapping will be performed for the QoS flow which supports bit rate control. With monitoring the actual rate for the QoS flows, the gNB-DU can determine the recommended rate for each QoS flow by taking the available resource, measured date rate per QoS flow and the QoS requirement of each QoS flow into account.

Observation 3: It is feasible and up to implementation for the gNB-DU to provide the QoS flow level recommended bit rate.

With the above analysis and observations, it should be confirmed that there is no technical difficulty to provide the QoS flow level recommended bit rate. So, it is reasonable to have per QoS flow MAC CE to indicate the rate directly, to achieve the purpose of QoS flow level rate control.

Proposal 1: RAN2 to introduce per QoS flow MAC CE for UL recommended rate indication.

2.2 Range and granularity for recommended bit rate

SA4 has replied the questions on the required range and granularity for XR rate control in the LS in S4-242182 [3]. According to the reply, the required granularity for XR rate control is 1% to 10% (where 1% is more preferred) with a range from 100kbps to 40Mbps. To meet the requirement, RAN2 agreed to specify a new table for XR rate control and check whether multipliers are needed.

- Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed

To design the new table, the distribution and the number of codepoints should be decided first. For the distribution, we think step size should be non-uniform since the required range covers very low bit rate as well as high bit rate. For the lower bit rate scope, the step size should be small so that the rate control is precise enough. While for the higher bit rate scope, the step size should be larger as it is meaningless to adjust the rate with a very small step compared to the average rate. Thus the exponential distribution should be adopted, instead of the linear distribution.

Besides, the logic to adapt the code rate at the application layer is also more aligned with the exponential distribution. For example, in some transport-wide congestion control algorithms, the encoder can adjust the sending bit rate based on the history bit rate and the network performance. For example, when the packet delay or the packet loss changes, the encoder may decrease or increase the rate Rt to p*Rt-1, where Rt-1 is the previous adopted rate and p is the adjustment factor, which can be static or dynamic related to the delay or loss rate. Such algorithms were introduced in TR 26.822, as mentioned by SA4 in the reply LS. To better coordinate with the upper layer bit rate adaptation mechanisms, RAN should support the same distribution when recommending the bit rate.

Proposal 2: Use exponential distribution for the new table for UL rate control.

To cover the range from 100kbps to 40Mbps with exponential distribution, different number of codepoints will be needed for different granularities, as listed in the following table.

Table 1. Needed number of codepoints corresponding to different granularities.

| Granularity | Number of codepoints |
| --- | --- |
| 1% | 604 |
| 5% | 124 |
| 10% | 64 |

Defining a table with more than 600 codepoints in the MAC specification is absolutely improper. Considering the overhead of the rate control MAC CE and the neatness of the specification, we should limit the number of codepoints. Currently, the largest table defined in the MAC specification is the table used for BSR, which has 256 codepoints. In our view, we should not introduce table with even larger size. It is better to have no more than 256 codepoints for the rate control table, which corresponds to 8-bit length indication.

Observation 4: It is better to limit the number of codepoints to no more than 2^8.

However, according to the above table, it is seen that the granularity cannot reach to 1% with 256 codepoints. Although the required granularity given by SA4 can be 1% to 10%, it is more preferred to support 1% considering the varieties of rate adaptation algorithms, as explained in S4-242182. To perform more effective and precise rate control for XR services, RAN2 should pay effort on providing the possibility to indicate the recommended bit rate with 1% granularity.

Excerption from S4-242182: The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

Observation 5: The limited number of codepoints cannot meet the 1% granularity requirement.

To resolve the contradiction between the limited number of codepoints and the required fine granularity, we think the multiplier can be used. Specifically, RAN2 can define an exponential table whose step size is 1% but the range is smaller than [100kbps, 40Mbps]. When the network wants to indicate the values out of the range of the table, a multiplier can be used to extend the indicatable range without effecting the granularity.

For example, a new recommended bit rate table with range from 100kbps to 1000kbps and with granularity of 1% can be defined, which costs about 230 code points considering exponential distribution (corresponding to 8 bits). And the range can be extended with some multipliers, e.g., to extend the upper limit to 40Mbps by multiplying 40. To support different range for bit rate recommendation, different multipliers can be used.

But when the multiplier is used, it would be unable to indicate the lower values with the required granularity. For example, assuming the first code point in the new table is 100kbps, when the multiplier 40 is used, the network cannot indicate the bit rates lower than 4Mbps with 1% granularity. To address the issue, dynamic multipliers can be considered. Specifically, in the above example, the network can dynamically use multipliers 1/10/40 to indicate the bit rate in ranges 100~1000kbps, 1000~10000kbps or 10000~40000kbps, while 1% granularity can be always satisfied. To reduce the signalling overhead, the network can pre-configure the multipliers, and indicate which one is used in the recommended bit rate MAC CE.

Proposal 3: Define a rate control table with a smaller range, e.g. 100kbps ~ 1000kbps, with 1% exponential distribution.

Proposal 4: Use dynamic multipliers to extend the range of the recommended rate indication to cover the whole range of rate from 100kpbs to 40Mbps.

Some companies may have concern on the complexity of the proposed solution. However, it can bring additional advantage, i.e., to provide future proof for the feature. Once new requirements for range or granularity emerge, the R19-specific table may be unsuitable. In this case, the solution can be well extended to new scenarios by setting new values for the multiplier. In our view, the additional complexity is not substantial and is justified by the extra flexibility that it brings.

2.3 MAC CE format

Some details of the MAC CE used for XR rate control can be considered based on the above proposals. First, to indicate per QoS flow level recommended bit rate, there should be an indication indicating the considered QoS flow within the MAC CE. Since the QoS flow ID is not unique within the UE, the PDU session ID should be combined to avoid the ambiguity. A simple method is to include the QoS flow ID and the PDU session ID together within the MAC CE, but it may cause some overhead. One way to optimize the overhead is to combine the QoS flow ID and the PDU session ID and indicate the combination using a few bits.

To indicate the recommended bit rate based on the proposed approach, several bits can be used to indicate the data rate with 1% granularity within a small range, and a multiplier can be included to extend the indicated bit rate. The multiplier can be an index of the pre-configured multipliers. For R19, values to extend the upper limit of the recommended bit rate to 40Mbps can be considered. The following figure shows an example of the MAC CE format as proposed, where the QoS Flow Indicator field indicates a pair of PDU session ID and QoS flow ID based on configuration.

| QoS Flow Indicator | Multiplier |
| --- | --- |
| Bit Rate |  |

Fig 1. Example of the UL rate control MAC CE.

Proposal 5: Recommended bit rate indication MAC CE for XR should consist of the following fields:

- Indicator corresponding to a certain combination of PDU session ID and QoS flow ID;

- Recommended bit rate indication for indicating bit rate with 1% granularity;

- Multiplier for recommended bit rate indication to extend the range.

2.4 Rate Query MAC CE

In RAN2#129 meeting, there was a working assumption on the query MAC CE for UL rate control, as shown below.

- Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this.

The query MAC CE can be useful for the UE to request for rate adaptation. For example, in case of congestion, the gNB may ask the application to decrease the source rate. After a period of time, the application may want to resume the rate, but it is not sure about whether the network congestion has disappeared or not. A proper way is the UE notifies the gNB about its desired bit rate, then the gNB can evaluate whether the desired rate can be provided and indicate a new recommended bit rate to the UE as the response.

The UE application may also want to change the bit rate due to some user behaviors, such as adjustments on the resolution of the uploaded video. To better coordinate with the network, the UE can indicate the desired rate to the gNB so that the gNB can prepare the scheduling accordingly, or further negotiate the bit rate with the UE.

Observation 6: UL MAC CE for UE desired bit rate can be applicable for UE-initiated change of data rate in the application layer.

From the perspective of feasibility, the desired bit rate can be determined by the application layer and indicated to the AS layer, since the application layer is aware of the service requirement and the UE behavior. It is up to the UE implementation to decide whether and when to send the query MAC CE.

With the benefit and feasibility, RAN2 should turn the WA into agreement to support rate query MAC CE.

Proposal 6: RAN2 to confirm the WA on rate query MAC CE.

3. Conclusion

In this contribution, we analyzed the potential enhancements for XR rate control. The following observations and proposals were made:

Per QoS flow v.s per DRB MAC CE:

Observation 1: Per DRB MAC CE is inaccurate for QoS flow level rate control and may cause some risk in QoS guarantee.

Observation 2: Both options have similar specification impacts from both RAN2 and RAN3 perspective.

Observation 3: It is feasible and up to implementation for the gNB-DU to provide the QoS flow level recommended bit rate.

Proposal 1: RAN2 to introduce per QoS flow MAC CE for UL recommended rate indication.

Range and granularity for recommended bit rate:

Observation 4: It is better to limit the number of codepoints to no more than 2^8.

Observation 5: The limited number of codepoints cannot meet the 1% granularity requirement.

Proposal 2: Use exponential distribution for the new table for UL rate control.

Proposal 3: Define a rate control table with a smaller range, e.g. 100kbps ~ 1000kbps, with 1% exponential distribution.

Proposal 4: Use dynamic multipliers to extend the range of the recommended rate indication to cover the whole range of rate from 100kpbs to 40Mbps.

MAC CE format:

Proposal 5: Recommended bit rate indication MAC CE for XR should consist of the following fields:

- Indicator corresponding to a certain combination of PDU session ID and QoS flow ID;

- Recommended bit rate indication for indicating bit rate with 1% granularity;

- Multiplier for recommended bit rate indication to extend the range.

UL MAC CE for UE desired bit rate:

Observation 6: UL MAC CE for UE desired bit rate can be applicable for UE-initiated change of data rate in the application layer.

Proposal 7: RAN2 to confirm the WA on rate query MAC CE.

4. Reference

[1]. R3-250805, TSG RAN WG3, Reply LS on XR UL Rate Control.

[2]. 3GPP TS 38.473: "NG-RAN F1 application protocol (F1AP) protocol specification".

[3]. S4-242182, TSG SA WG4, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications.

[4]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".