3GPP TSG-RAN WG2 Meeting #129bis	R2-2502097

Wuhan, China, April 07th – 11th, 2025

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the open issues related to LP-WUS considering the progress in RAN1/2/4.

# Discussion

## Issues for paging monitoring triggered by LP-WUS

### LP-WUS and PEI

When LP-WUS and PEI are both configured and used, there are two cases of potential subgrouping types used for LP-WUS and PEI.

Case 1 - Same type of subgrouping method used for LP-WUS and PEI: In this case, both LP-WUS and PEI use either CN assigned subgrouping or UE_ID based subgrouping. For CN assigned subgrouping, CN can ensure proper subgrouping of the LP-WUS and PEI by implementation to reduce the power consumption. For UE_ID based subgrouping, it is straightforward to support it without CN control.

Case 2 - Different type of subgrouping method used for LP-WUS and PEI: In this case, CN assigned subgrouping is applied for LP-WUS and UE_ID based subgrouping is applied for PEI and vice-versa. We do not see any functional issues other than a slight impact on the power saving gain. Hence what type of subgrouping to apply for LP-WUS and PEI can be left to NW implementation.

Proposal 1: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e., CN assigned subgrouping or UE_ID based subgrouping) are applied.

## Configuration Aspects of LP-WUS

### Dedicated configuration in RRC Signalling

RAN2 has reached the agreement that the LP-WUS related configuration for IDLE/INACTIVE state is provided at least via system information and whether dedicated signaling for configuration is still FFS.

The LP-WUS related configuration should be cell specific since it is for IDLE/INACTIVE state. The configuration includes both configuration for LP-SS and LP-WUS thus covering both OOK-based and OFDM-based receivers respectively. Providing dedicated configuration in RRC signalling for example in RRC Release limits the scenarios in which LP-WUS can be used. For example, when an LP-WUS UE is provided with dedicated configuration and if the UE selects a new cell due to mobility, the new cell is not aware of the dedicated configuration of the LP-WUS for the UE and thus the UE still needs to rely on the system information from the cell. Thus, there is no need for dedicated configuration.

Proposal 2: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

### Enabling of LP-WUS

If LP-WUS functionality is enabled for all the UEs supporting LP-WUS in a camped cell, for example, when a UE receives LP-WUS configuration via SIB from the camped cell, it can use LP-WUS functionality. It is simple but may lead to a lot of UEs using LP-WUS, which means the false paging probability increases and causes more transitions from LP-WUR to MR. This will reduce the power saving gain or even result in negative power saving gain considering the maximum number of subgroups for LP-WUS has been decided by RAN1 to be 31. Hence there can be UE/group specific control for enabling LP-WUS.

To reduce false paging probability, the network (CN or RAN) decides whether a LP-WUS capable UE is allowed to use the LP-WUS functionality based on the conditions in the network (e.g., radio resources) and/or the characteristics of the UEs (e.g., paging probability, shorter DRX cycle, etc.). For example, the CN uses NAS signaling to indicate whether a specific UE can use LP-WUS functionality, which is similar to Rel-17 CN-assigned subgrouping method. To minimize the impact for deployment, the UE by default can use LP-WUS functionality unless a specific indication to disable the LP-WUS functionality is received from CN, which implies that the absence of indication means UE is allowed to use LP-WUS functionality. In this case, if the CN does not implement this feature, LP-WUS can still be implemented as an AS solution; if CN prefers to have precise control, CN can implement the control indication in NAS signalling.

Proposal 3: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality.

### Configuration for different types of LP-WUR

RAN1 has been discussing to support both OOK-based and OFDM-based receivers for LP-WUS monitoring. Since the OFDM-based LP-WUR has stronger capability for LP-WUS signal reception and processing, it is possible for NW to configure a shorter LP-WUS to wake up only OFDM-based receiver to reduce the resource overhead. In this case, the UE with OFDM-based receiver can benefit from the low-complexity operation on LP-WUS, e.g., to cancel any OOK sequence/pattern detection or implement termination of LP-WUS procedure by early detection. The UE with OOK-based receiver will skip the current LP-WUS for obtaining more power saving gain. Thus, the NW should be able to support the scenario that only UEs with OFDM-based LP-WUR are paged, e.g., by sending a shorter LP-WUS that cannot be received by OOK-based receiver. This information can be implicitly indicated by the absence of LP-WUS configuration for OOK-based receiver.

Proposal 4: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

### LP-WUS entry/exit condition

In RAN2#127, RAN2 has agreed that the thresholds of LP-WUS entry/exit condition should be separately configured for different types of receiver. Considering the LP-WUS cell coverage cannot reach the full coverage of legacy cell as specified in the WID [1], the feasibility of LP-WUS feature may be greatly affected if the entry/exit condition is absent in the LP-WUS configuration, i.e., to leave when to enter/exit LP-WUS mode to UE implementation, since UE will spend plenty of time and resource overhead to decide whether LP-WUS can be reliably used. Therefore, it is beneficial that the thresholds of LP-WUS entry/exit condition are always carried in the LP-WUS related configuration as a reference for UE to determine the LP-WUS coverage with more precision. Thus, we propose that the thresholds of LP-WUS entry/exit conditions for a specific type of receiver should be always configured if supported by the network.

Proposal 5a: Thresholds of LP-WUS entry/exit conditions for OOK-based receivers are always configured when OOK-based LP-WUS configuration is provided by the network.

Proposal 5b: Thresholds of LP-WUS entry/exit conditions for OFDM-based receivers are always configured when OFDM-based LP-WUS configuration is provided by the network.

RAN1 made the following agreement that LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS.

Agreement in RAN-118b Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

The agreement implies that OFDM-based receiver can measure LP-SS. This may require separate thresholds for entry and exit conditions for OFDM-based receiver to measure LP-SS, in addition to thresholds for OOK-based receiver to measure LP-SS. However, RAN4 is still discussing the mapping between different sets of noise figure and different receiver types, e.g. whether noise figure can be decoupled with OOK-based/OFDM-based receiver. Thus, RAN2 can wait for further RAN4/1 conclusions to determine whether different thresholds distinguishing different sets of noise figure or different receiver types are needed.

Proposal 5c: RAN2 waits for further RAN4/1 conclusion to determine whether more thresholds are needed.

RAN2 has also discussed whether the entry condition can be also based on the measurement result of LP-WUR in RAN2#127 and RAN2#127bis, and made the following working assumption and agreements.

Working Assumption in RAN2#127 - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. Agreement in RAN2#127bis - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

UE usually performs RRM measurements by MR as per legacy behavior. Thus, the serving cell measurements by MR are always available and good enough. As the hardware capability of MR is better than that of LP-WUR, the measurements by LP-WUR do not provide any additional benefit for LP-WUS entry decision. On the other hand, using both MR and LP-WUS measurements can make the LP-WUS entry procedure complicated, introducing much specification impact. Hence, the measurement by LP-WUR is not needed for LP-WUS entry condition and it can be left to NW to decide whether the threshold based on LP-WUR is needed for UE to enter LP-WUS monitoring.

Proposal 6: For LP-WUS entry condition, the threshold based on MR is mandatorily configured, while the threshold based on LP-WUR is optional.

If the NW configures the LP-WUS entry thresholds for both MR and LP-WUR measurements, it should be up to UE implementation to obtain the measurement results on LP-WUR, i.e., the metrics and requirement of LR measurement should wait for RAN4’s discussion and there is no need to define in RAN2 spec the specific UE behavior on LP-WUR measurement for LP-WUS entry condition.

Proposal 7: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

# Conclusion

In this contribution, we discuss procedure and configuration of LP-WUS for IDLE/INACTIVE modes, and have the following proposals:

Issues for paging monitoring triggered by LP-WUS

Proposal 1: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e., CN assigned subgrouping or UE_ID based subgrouping) are applied.

Configuration Aspects of LP-WUS

Proposal 2: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Proposal 3: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality.

Proposal 4: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

Proposal 5a: Thresholds of LP-WUS entry/exit conditions for OOK-based receivers are always configured when OOK-based LP-WUS configuration is provided by the network.

Proposal 5b: Thresholds of LP-WUS entry/exit conditions for OFDM-based receivers are always configured when OFDM-based LP-WUS configuration is provided by the network.

Proposal 5c: RAN2 waits for further RAN4/1 conclusion to determine whether more thresholds are needed.

Proposal 6: For LP-WUS entry condition, the threshold based on MR is mandatorily configured, while the threshold based on LP-WUR is optional.

Proposal 7: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

# References

RP-241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode;

RP-242656 On LR and MR operating in different bands in release;