3GPP TSG-RAN WG2 Meeting #129bis		R2-2502154

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading

Agenda Item:	8.4.1

Document for:	Discussion and Decision

- Introduction

When drafting the RRC running CR [1], the rapporteur noticed that there are too many thresholds for the LP-WUS/WUR WI, including entry/exit condition for LP-WUS monitoring, serving cell measurement relaxation/offloading, and neighboring cell relaxation. This will create additional challenges for its implementation. This would mean operators will incur greater costs when setting or optimizing these parameters in the real field. In this contribution, we will discuss the number of thresholds for LP-WUS/WUR WI and try to analyze how to reduce the number of thresholds.

- Discussion

In the LP-WUS/WUR WI, the entry/exit conditions are introduced for the following 4 procedures:

- LP-WUS monitoring

- Serving cell RRM measurement offloading

- Serving cell RRM measurement relaxation

- Neighboring cell RRM measurement relaxation

And the corresponding thresholds will be configured for the above 4 procedures according to the discussion. Since this WI is for the UE equipped with LR, the serving cell measurement via LR may also need to be considered for the entry/exit conditions in the above 4 procedures in addition to the serving cell measurement via MR. Thus, it could be predictable the number of configured thresholds in this WI will be inconceivable, which will bring much more complexity and hurt the readability of the specification. In this paper, we will provide detailed analysis of the number of configured thresholds in this WI and propose some directions to reduce the number for simplicity.

  - Number of thresholds for LP-WUS/WUR

Number of thresholds of entry/exit conditions for LP-WUS monitoring

Firstly, for the entry/exit conditions for LP-WUS monitoring, RAN1 and RAN2 achieved the following agreements:

- RAN2 agreements Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. The LPWUS monitoring exit condition does not include MR measurements. RAN1 Agreement Confirm the following working assumption with the addition of the note at the bottom: Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

With the above agreements, it could be observed if entry/exit conditions for LP-WUS monitoring are configured:

- For entry conditions: MR-based thresholds (which include RSRP and optional RSRQ) are mandatorily configured while LR-based thresholds (which include (LP-)RSRP and optional (LP-)RSRQ) are optionally configured.

- For exit conditions: LR-based thresholds (which include (LP-)RSRP and optional (LP-)RSRQ) are mandatorily configured.

RAN1 and RAN2 both agreed separate entry/exit thresholds are configured for different types of LR. Besides, for the entry conditions, RAN1 only considered the MR thresholds and RAN2 agreed on the MR thresholds and LR thresholds finally, which means separate MR thresholds and LR thresholds for entry conditions will be introduced for different types of LR; for the exit conditions, RAN1 and RAN2 agreed only LR thresholds thus separate LR threshold for entry conditions will be introduced for different types of LR. Regarding the LR type, this WI introduces the following LR types:

- LRType#1: OOK-based LP-WUR measuring LP-SS with/without overlaid OFDM sequence

- LRType#2: OFDM-based LP-WUR measuring only LP-SS with and without overlaid OFDM sequence

- LRType#3: OFDM-based LP-WUR measuring SSB and may also measure LP-SS with and without overlaid OFDM sequence

In RAN4#114 meeting, RAN4 agreed the same requirement shall be applied for LRType#1 and LRType#2 based on the following conclusion. Thus, in our understanding, same entry/exit condition with same threshold should be configured for LRType#1 and LRType#2. At the same time, we also noticed that some companies think separate threshold should be configured LRType#1 and LRType#2. But anyway, this part needs further discussion.

Issue 1-1-3-1: On RRM requirements to be specified for OOK-based LP-WUR based on LP-SS with overlaid OFDM sequence Agreement: - In Rel-19, the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not. Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence Agreement: - Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS - Reuse the same requirement for LP-SS OOK based

With the above, we have the following observation on the threshold configuration for entry/exit conditions for LP-WUS monitoring.

Observation 1: Regarding the entry conditions for LP-WUS monitoring, the following 8 thresholds will be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_1: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_1: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_1: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_1: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_MR_RSRP_threshold5_1: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold6_1: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_1: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_1: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 2: Regarding the exit conditions for LP-WUS monitoring, the following 4 thresholds will be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Exit_LR_RSRP_threshold9_1: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold10_1: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_LR_RSRP_threshold11_1: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold12_1: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Besides, considering some companies also think that OFDM-based LP-WUR measuring only LP-SS should be configured with separate threshold for OOK-based LP-WUR measuring LP-SS, in this way, more thresholds (4 entry+2 exit) will be introduced.

Number of thresholds of entry/exit conditions for Serving Cell RRM measurement offloading

Secondly, for the entry/exit conditions for Serving cell RRM measurement offloading, RAN2 achieved the following agreements:

- RAN2 agreements For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

With the agreements, it could be observed there is entry/exit conditions for Serving cell RRM measurement offloading:

- For entry conditions: MR-based thresholds (which include RSRP and optional RSRQ) are mandatorily configured while LR-based thresholds (which includes (LP-)RSRP and optional (LP-)RSRQ) are optionally configured.

- For exit conditions: LR-based thresholds (which include (LP-)RSRP and optional (LP-)RSRQ) are mandatorily configured.

Although there is no conclusion on whether separate thresholds are configured for different types of LR, it is expected that the measurement performance of OFDM-based LP-WUR measuring SSB is better than the OFDM-based/OOK-based LP-WUR measuring LP-SS. Thus, it is reasonable to configure different thresholds for the UEs equipped with different LR types.

Following the similar principle as the thresholds of entry/exit conditions for LP-WUS monitoring, the following observations on entry/exit conditions for Serving cell RRM measurement offloading is achieved.

Observation 3: Regarding the entry conditions for Serving Cell RRM measurement offloading, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_2: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_2: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_2: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_2: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_MR_RSRP_threshold5_2: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold6_2: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_2: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_2: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 4: Regarding the exit conditions for Serving Cell RRM measurement offloading, the following 4 thresholds will be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Exit_LR_RSRP_threshold9_2: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold10_2: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_LR_RSRP_threshold11_2: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold12_2: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Similarly, if OFDM-based LP-WUR measuring only LP-SS is configured with separate threshold for OOK-based LP-WUR measuring LP-SS as some companies proposed, more thresholds (4 entry+2 exit) will be introduced.

Number of thresholds of entry/exit conditions for Serving cell RRM measurement relaxation

Thirdly, for the entry/exit conditions for Serving cell RRM measurement relaxation, RAN2 achieved the following agreements:

- RAN2 agreements The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

With the agreements, it could be observed there is entry/exit conditions for Serving cell RRM measurement relaxation:

- For entry conditions: MR-based thresholds (which include RSRP and optional RSRQ) are mandatorily configured while LR-based thresholds (which includes (LP-)RSRP and optional (LP-)RSRQ) are optionally configured.

- For exit conditions: It is still open on whether a separate exit condition than ‘not fulfilling the entry condition’ is needed. If a separate exit condition is needed, both the MR-based thresholds (which include RSRP and optional RSRQ) and LR-based thresholds (which include (LP-)RSRP and optional (LP-)RSRQ) will be configured since MR serving cell measurement and LR serving cell measurement are both performed in this case.

Similar to the Serving cell RRM measurement offloading, it is expected to configure different thresholds for the UEs equipped with the different LR types. Following the similar principle as above, we could have the following observations on entry/exit conditions for Serving cell RRM measurement relaxation.

Observation 5: Regarding the entry conditions for Serving Cell RRM measurement relaxation, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_3: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_3: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_3: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_MR_RSRP_threshold5_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold6_3: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_3: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_3: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 6: Regarding the exit conditions for Serving Cell RRM measurement relaxation, if it is not additionally introduced, no threshold will be introduced. Otherwise, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory?) Exit_MR_RSRP_threshold9_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold10_3: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_MR_RSRQ_threshold11_3: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold12_3: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory?) Exit_MR_RSRP_threshold13_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold14_3: LR LP-RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold15_3: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold16_3: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Number of thresholds of entry/exit conditions for Neighboring Cell RRM measurement relaxation

Lastly, for the entry/exit conditions for Neighboring cell RRM measurement relaxation, RAN2 achieved the following agreement:

- RAN2 agreements FFS if the entry condition for serving cell RRM measurement relaxation is the same as Neighboring cell RRM measurement relaxation.

With the agreement, if we agree the entry/exit condition for Neighboring Cell RRM measurement relaxation is the same as Serving Cell RRM measurement relaxation, no additional threshold will be introduced. On the other hand, the same number of entry/exit conditions for Serving Cell RRM measurement relaxation will be introduced for Neighboring Cell RRM measurement relaxation.

Observation 7: Regarding the entry conditions for Neighboring Cell RRM measurement relaxation, the following 8 thresholds are most likely to be configured if it is different from the Serving Cell RRM measurement relaxation:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_4: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_4: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_4: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory?) Entry_MR_RSRP_threshold5_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Entry_LR_RSRP_threshold6_4: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_4: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_4: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 8: Regarding the exit conditions for Neighboring Cell RRM measurement relaxation, if it is not additionally introduced, no threshold will be introduced. Otherwise, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory?) Exit_MR_RSRP_threshold9_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold10_4: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_MR_RSRQ_threshold11_4: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold12_4: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory?) Exit_MR_RSRP_threshold13_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold14_4: LR LP-RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold15_4: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold16_4: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Total number of thresholds

The above number of thresholds is based on the assumption that Low-Mobility criterion is not introduced for serving cell RRM relaxation relaxation/offloading and neighboring cell RRM measurement relaxation. If Low-Mobility criteria is introduced, more thresholds are needed.

With the above 6 observations, assuming the entry condition for Serving Cell RRM measurement relaxation is separate from the Neighboring Cell RRM measurement relaxation, the number of the thresholds for entry/exit conditions in LP-WUS/WUR WI is summarized as follows Table:

Table 1: Total number of thresholds for entry/exit conditions for LP-WUS/WUR WI

| Number of thresholds UE procedure | entry conditions | exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement offloading | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |
| Neighboring Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |

Thus, the number of entry/exit conditions in LP-WUS/WUR WI could be up to as high as 40 or 56. What an overwhelming number.  This will create additional challenges for the implementation of this feature. This would mean operators will incur greater costs when setting or optimizing these parameters in the real field.

Observation 9: The total number of thresholds for entry/exit conditions in LP-WUS/WUR WI could be up to as high as 40 or 56.

  - Directions to reduce the number of thresholds for LP-WUS/WUR

In order to make this feature easy to deploy in the real field, and reduce the workload for network optimization, it is important and beneficial to reduce the number of thresholds.

In our understanding, the following directions could be considered.

Direction 1: Remove the LR based thresholds for all entry conditions in the above RRM relaxation/offloading and LP-WUS monitoring procedures.

In our understanding, the reason to introduce the LR based thresholds is for the scenario that LR band is different from the MR band, in this case, the serving cell measurement performance via MR band couldn’t represent the LR band. However, since we only agreed with the intra-band case in Rel-19, the motivation to introduce the LR based threshold for entry conditions become weak. Besides, according to the following agreed RAN4 requirements:

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

It seems that the LR may not perform the measurement at the initial phase when UE accesses a cell since the LR is turned on based on UE implementation, for example when the MR measurement is better than a threshold. In this case, if the LR thresholds are configured for entry conditions, the time UE enters for the RRM relaxation or LP-WUS monitoring procedure may be delayed and the power saving gain will be impacted since even if the entry condition based on MR measurement is satisfied, UE has to wait for the entry condition based on LR measurement considering the time UE to turn on the LR may be late and the LR measurement is not obtained at this time.

Thus, we think this is a reasonable direction without the impact on LP-WUS power saving gain or will not bring other issues. With this direction, the number of thresholds could be reduced is 16 which includes:

- For LP-WUS monitoring:

  - Entry_LR_RSRP_threshold2_1 and Entry_LR_RSRQ_threshold4_1 for OFDM-based LP-WUR based on SSB;

  - Entry_LR_RSRP_threshold6_1 and Entry_LR_RSRQ_threshold8_1 for OOK-based LP-WUR based on LP-SS and OFDM-based LP-WUR based on only LP-SS

- For Serving Cell RRM measurement offloading:

  - Entry_LR_RSRP_threshold2_2 and Entry_LR_RSRQ_threshold4_2 for OFDM-based LP-WUR based on SSB;

  - Entry_LR_RSRP_threshold6_2 and Entry_LR_RSRQ_threshold8_2 for OOK-based LP-WUR based on LP-SS and OFDM-based LP-WUR based on only LP-SS

- For Serving Cell RRM measurement relaxation:

  - Entry_LR_RSRP_threshold2_3 and Entry_LR_RSRQ_threshold4_3 for OFDM-based LP-WUR based on SSB;

  - Entry_LR_RSRP_threshold6_3 and Entry_LR_RSRQ_threshold8_3 for OOK-based LP-WUR based on LP-SS and OFDM-based LP-WUR based on only LP-SS

- For Neighboring Cell RRM measurement relaxation:

  - Entry_LR_RSRP_threshold2_4 and Entry_LR_RSRQ_threshold4_4 for OFDM-based LP-WUR based on SSB;

  - Entry_LR_RSRP_threshold6_4 and Entry_LR_RSRQ_threshold8_4 for OOK-based LP-WUR based on LP-SS and OFDM-based LP-WUR based on only LP-SS

And the total number of thresholds is 24 or 40 with this reduction method in Direction 1:

Table 2: Total number of thresholds for entry/exit conditions with reduction Direction 1 for LP-WUS/WUR WI

| Number of thresholds UE procedure | entry conditions | exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | 8 -> 4 in which - 4 -> 2 configured for LR measures SSB - 4 -> 2 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement offloading | 8, -> 4 in which - 4 -> 2 configured for LR measures SSB - 4 -> 2 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement relaxation | 8 -> 4 in which - 4 -> 2 configured for LR measures SSB - 4 -> 2 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |
| Neighboring Cell RRM measurement relaxation | 8 -> 4 in which - 4 -> 2 configured for LR measures SSB - 4 -> 2 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |

Observation 10: With Direction 1, i.e, remove the LR based thresholds for all entry conditions in the RRM relaxation/offloading and LP-WUS monitoring procedures could save 16 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 24 or 40.

Direction 2: Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation

For the RRM relaxation/offloading in LP-WUS WI, it was agreed that it is only for LP-WUS UEs as below. Besides, serving cell relaxation is introduced. The power saving gain comes from the case that UE is in deep sleep or ultra deep sleep state. If the UE only performs serving cell relaxation or only performs neighboring cell relaxation, the power saving gain will be margin. The most benefit case is to perform RRM relaxation for serving cell and neighboring cell at the same time.

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

In summary, Direction 2 will achieve more power saving gain, since in this case UE could perform Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation simultaneously. With this direction, the number of thresholds could be reduced by 8 or 16 as below and the total number of thresholds is 32 or 40:

Table 3: Total number of thresholds for entry/exit conditions with reduction Direction 2 for LP-WUS/WUR WI

| Number of thresholds UE procedure | entry conditions | exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement offloading | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |
| Neighboring Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |

Observation 11: With Direction 2, i.e, merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation could save 8 or 16 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 32 or 40.

Direction 3: Merge the entry/exit conditions for Serving Cell RRM measurement offloading or Serving Cell RRM measurement relaxation or Neighboring Cell RRM measurement relaxation and LP-WUS monitoring

According to the current specification, the serving cell quality for neighboring cell RRM measurement relaxation criteria is Srxlev or Squal, for example:

The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

The definition of Srxlev and Squal is as follows:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

In this direction, assuming the serving cell quality for Serving Cell RRM measurement offloading or Serving Cell RRM measurement relaxation or Neighboring Cell RRM measurement relaxation have the same definition as LP-WUS monitoring, e.g. both the condition for relaxation/offloading and LP-WUS monitoring using the Srxlev and Squal for MR measurement, while both the condition for relaxation/offloading and LP-WUS monitoring using the LP-Srxlev and LP-Squal for LR measurement. In this case, the following sub-directions could be considered:

- Direction 3-1:  Entry/exit condition for Serving Cell RRM measurement offloading is the same as LP-WUS monitoring

- Direction 3-2:  Entry/exit condition for Serving Cell RRM measurement relaxation is the same as LP-WUS monitoring

- Direction 3-3:  Entry/exit condition for Neighboring Cell RRM measurement relaxation is the same as LP-WUS monitoring

For Direction 3-1, according to our analysis, since the number of entry thresholds for both LP-WUS monitoring and Serving Cell RRM measurement offloading may be 8, while the number of exit thresholds for both LP-WUS monitoring and Serving Cell RRM measurement offloading are 4, it is easily to combine them. In this scenario, the entry conditions for serving cell RRM relaxation or neighboring cell RRM relaxation will be easier to be satisfied than the entry conditions for LP-WUS monitoring, which is shown as following Fig.1.

With this direction, the number of thresholds could be reduced of 12 and the total number of thresholds is 28/44.

Table 4: Total number of thresholds for entry/exit conditions with reduction Direction3-1 for LP-WUS/WUR WI

| Number of thresholds UE procedure | entry conditions | exit conditions |
| --- | --- | --- |
| LP-WUS monitoring and Serving Cell RRM measurement offloading | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement offloading | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |
| Neighboring Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |

Observation 12: With Direction 3-1, i.e, merge the entry/exit conditions for Serving Cell RRM measurement offloading and LP-WUS monitoring could save 12 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 28 or 44.

For Direction 3-1 and Direction 3-2, according to our analysis, though the number of entry thresholds for both LP-WUS monitoring and Serving/Neighboring Cell RRM measurement relaxation may be 8, the number of exit thresholds for LP-WUS monitoring is 4 and the number of exit thresholds for Serving/Neighboring Cell RRM measurement relaxation is 0 or 8, it is not easy to combine them. However, we could consider the following solutions:

- Solution#1: When the exit conditions of LP-WUS are satisfied, the serving/neighboring cell RRM relaxation also exits.

- Solution#2: Introducing the exit condition for Serving/neighboring Cell RRM measurement relaxation which only includes the LR measurement similar to the LP-WUS monitoring.

Fig.1 example of threshold for entry/exit condition for LP-WUS monitoring and RRM relaxation/offloading

Anyway, it could be achieved to combine the entry/exit conditions for Serving Cell RRM measurement offloading or Serving/Neighboring Cell RRM relaxation, and in this case, the UE behaviour will be simplified. With this direction, the number of thresholds could be reduced by 8/16 and the total number of thresholds is 32/40.

Table 5: Total number of thresholds for entry/exit conditions with reduction Direction3-2/3 for LP-WUS/WUR WI

| Number of thresholds UE procedure | entry conditions | exit conditions |
| --- | --- | --- |
| LP-WUS monitoring and Serving (or Neighboring) Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement offloading | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | 4 include only LR thresholds - 2 configured for LR measures SSB - 2 configured for LR only measures LP-SS |
| Serving Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |
| Neighboring (or Serving) Cell RRM measurement relaxation | 8, in which - 4 configured for LR measures SSB - 4 configured for LR only measures LP-SS | - If only ‘not fulfilling the entry condition’: 0 - Otherwise: 8 |

Observation 13: With Direction 3-2 and Direction 3-3, i.e, merge the entry/exit conditions for Serving Cell RRM relaxation or Neighboring Cell RRM relaxation and LP-WUS monitoring could save 8/16 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 32 or 40.

Direction 4: Merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement

In current measurement procedure, UE will not perform neighboring cell measurement when serving cell measurement is above the SIntraSearchP/SnonIntraSearchP. In details, when the serving cell fulfils Srxlev > SIntraSearchP and Squal > SintraSearchQ, intra-frequency measurement won’t be performed, and when the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, inter-frequency measurement won’t be performed (except the frequency with high priority).

To achieve the most power saving gain, both neighboring cell measurement and serving cell measurement could be stopped simultaneously. The corresponding UE behaviour is shown as below:

Fig.2 example of the entry/exit conditions for Serving Cell RRM measurement offloading is same as SIntraSearchP/SnonIntraSearchP

In this case, we could combine the entry/exit conditions for Serving Cell RRM measurement offloading with current threshold for stopping neighboring cell measurement, i.e., no additional threshold for Serving Cell RRM measurement offloading will be introduced, and the UE enters Serving Cell RRM measurement offloading when the serving cell measurement is above the threshold SIntraSearchP/SnonIntraSearchP, and exits Serving Cell RRM measurement offloading when the serving cell measurement is not above the threshold SIntraSearchP/SnonIntraSearchP. With this direction, the number of thresholds could be reduced by 12 and the total number of thresholds is 28/44, which is same as table 4.

Observation 14: With Direction 4, i.e, merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement, i.e. SIntraSearchP/SnonIntraSearchP could save 12 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 28 or 44.

With the above analysis, it could be observed that all the directions could reduce the total number of thresholds for entry/exit conditions, and the directions are not mutually exclusive, we could combine them for further simplification.

Proposal 1: RAN2 to discuss whether/how to reduce the number of thresholds for LP-WUS/WUR WI considering the following one or more directions:

- Direction 1: Remove the LR based thresholds for all entry conditions in the above 4 RRM relaxation/offloading and LP-WUS monitoring procedures

- Direction 2: Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation

- Direction 3: Merge the entry/exit conditions for Serving Cell RRM measurement offloading or Serving Cell RRM measurement relaxation or Neighboring Cell RRM measurement relaxation and LP-WUS monitoring

- Direction 4: Merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement

- Conclusion

In this contribution, we discussed whether/how to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading. Based on the discussion, we have the following observations and proposals:

Observations:

Observation 1: Regarding the entry conditions for LP-WUS monitoring, the following 8 thresholds will be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_1: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_1: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_1: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_1: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_MR_RSRP_threshold5_1: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold6_1: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_1: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_1: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 2: Regarding the exit conditions for LP-WUS monitoring, the following 4 thresholds will be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Exit_LR_RSRP_threshold9_1: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold10_1: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_LR_RSRP_threshold11_1: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold12_1: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 3: Regarding the entry conditions for Serving Cell RRM measurement offloading, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_2: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_2: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_2: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_2: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_MR_RSRP_threshold5_2: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold6_2: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_2: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_2: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 4: Regarding the exit conditions for Serving Cell RRM measurement offloading, the following 4 thresholds will be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Exit_LR_RSRP_threshold9_2: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold10_2: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_LR_RSRP_threshold11_2: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold12_2: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 5: Regarding the entry conditions for Serving Cell RRM measurement relaxation, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_3: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_3: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_3: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory) Entry_MR_RSRP_threshold5_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold6_3: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_3: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_3: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 6: Regarding the exit conditions for Serving Cell RRM measurement relaxation, if it is not additionally introduced, no threshold will be introduced. Otherwise, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory?) Exit_MR_RSRP_threshold9_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold10_3: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_MR_RSRQ_threshold11_3: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold12_3: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory?) Exit_MR_RSRP_threshold13_3: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold14_3: LR LP-RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold15_3: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold16_3: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 7: Regarding the entry conditions for Neighboring Cell RRM measurement relaxation, the following 8 thresholds are most likely to be configured if it is different from the Serving Cell RRM measurement relaxation:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory) Entry_MR_RSRP_threshold1_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRP_threshold2_4: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Entry_MR_RSRQ_threshold3_4: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold4_4: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory?) Entry_MR_RSRP_threshold5_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Entry_LR_RSRP_threshold6_4: LR LP-RSRP threshold for serving cell measurement based on LP-SS

  - (optional) Entry_MR_RSRQ_threshold7_4: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Entry_LR_RSRQ_threshold8_4: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 8: Regarding the exit conditions for Neighboring Cell RRM measurement relaxation, if it is not additionally introduced, no threshold will be introduced. Otherwise, the following 8 thresholds will be/most likely to be configured:

- For OFDM-based LP-WUR measuring SSB and may also measure LP-SS

  - (mandatory?) Exit_MR_RSRP_threshold9_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold10_4: LR RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_MR_RSRQ_threshold11_4: MR RSRQ threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold12_4: LR RSRQ threshold for serving cell measurement based on SSB

- For OOK-based LP-WUR measuring LP-SS (and maybe OFDM-based LP-WUR measure only LP-SS?)

  - (mandatory?) Exit_MR_RSRP_threshold13_4: MR RSRP threshold for serving cell measurement based on SSB

  - (optional?) Exit_LR_RSRP_threshold14_4: LR LP-RSRP threshold for serving cell measurement based on SSB

  - (optional) Exit_LR_RSRQ_threshold15_4: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

  - (optional) Entry_LR_RSRQ_threshold16_4: LR LP-RSRQ threshold for serving cell measurement based on LP-SS

Observation 9: The total number of thresholds for entry/exit conditions in LP-WUS/WUR WI could be up to as high as 40 or 56.

Observation 10: With Direction 1, i.e, remove the LR based thresholds for all entry conditions in the RRM relaxation/offloading and LP-WUS monitoring procedures could save 16 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 24 or 40.

Observation 11: With Direction 2, i.e, merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation could save 8 or 16 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 32 or 40.

Observation 12: With Direction 3-1, i.e, merge the entry/exit conditions for Serving Cell RRM measurement offloading and LP-WUS monitoring could save 12 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 28 or 44.

Observation 13: With Direction 3-2 and Direction 3-3, i.e, merge the entry/exit conditions for Serving Cell RRM relaxation or Neighboring Cell RRM relaxation and LP-WUS monitoring could save 8/16 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 32 or 40.

Observation 14: With Direction 4, i.e, merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement, i.e. SIntraSearchP/SnonIntraSearchP could save 12 thresholds, in this case, the total number of thresholds for entry/exit conditions in LP-WUS/WUR WI is 28 or 44.

Proposal:

Proposal 1: RAN2 to discuss whether/how to reduce the number of thresholds for LP-WUS/WUR WI considering the following one or more directions:

- Direction 1: Remove the LR based thresholds for all entry conditions in the above 4 RRM relaxation/offloading and LP-WUS monitoring procedures

- Direction 2: Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation

- Direction 3: Merge the entry/exit conditions for Serving Cell RRM measurement offloading or Serving Cell RRM measurement relaxation or Neighboring Cell RRM measurement relaxation and LP-WUS monitoring

- Direction 4: Merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement

- References

- R2-2502153, RRC Running CR for LP-WUS/WUR, vivo