3GPP TSG-RAN WG2 Meeting #129		R2-2502165

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on RLC enhancement for XR

Agenda Item:	8.7.5

Document for:	Discussion and Decision

- Introduction

In RAN2#129, the following agreements were made for RLC enhancement [1]:

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

In this paper, we will discuss the remaining issues for the RLC enhancement of XR.

- Discussion

  - Enhancements to ensure timely RLC retransmissions

In the RAN2#129 meeting, there was an FFS on how to determine the remaining time for triggering the autonomous retransmission and/or polling, i.e., based on discardTimer at PDCP or new timer at RLC.

In our view, using a new RLC timer to track the remaining time of packets may not be accurate because packets first arrive at PDCP, and the PDCP entity may buffer them for a period of time before delivering them to RLC. If the new RLC timer starts only when the RLC layer receives the packet from PDCP, the actual remaining time for the packet would be shorter than the time determined by the new RLC timer. If the new RLC timer starts when the packets arrive at PDCP, some indication from PDCP is still needed to inform RLC to start the timer.

Observation 1: Using a new RLC timer to calculate the remaining time may be not accurate, because the PDCP entity may buffer the packet for a period of time before delivering it to RLC, or some indication is still needed from PDCP to inform RLC to start the timer when the packets arrive at PDCP.

In addition, introducing a new RLC timer would bring much more specification impact compared to reusing the legacy discardTimer in PDCP. For example, we would need to define behaviors for the starting, stopping, and expiration of the new RLC timer. Conversely, by reusing the legacy discardTimer in PDCP, only two cross-layer indications (for autonomous retransmission and polling enhancement) are required.

Observation 2: Determining the remaining time based on a newly introduced RLC timer incurs more specification impacts compared to utilizing the existing discardTimer in PDCP.

Proposal 1: Remaining time for triggering autonomous retransmission and polling enhancement should be determined based on the existing discardTimer in PDCP.

    - Autonomous retransmission

According to the current RLC specification, AMD PDUs for retransmission are prioritized over AMD PDUs for initial transmission.

TS 38.322

5.2.3.1	Transmit operations

5.2.3.1.1	General

The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

When autonomous retransmission is introduced, the priority between AMD PDU(s) for autonomous retransmission and AMD PDU(s) for ARQ-based retransmission (triggered by STATUS report) needs to be discussed. Autonomous retransmission is triggered when the remaining time is below a threshold, indicating that AMD PDU(s) for autonomous retransmission are more delay-critical than those for ARQ-based retransmission. Therefore, AMD PDU(s) for autonomous retransmission are expected to have higher priority and should be transmitted first to ensure timely retransmission of delay-critical data.

Additionally, since only one instance of autonomous retransmission may be triggered per RLC SDU, no additional special handling is required to manage the coexistence of autonomous retransmission and the ARQ procedure.

Proposal 2: The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs for autonomous retransmission over AMD PDUs for ARQ-based retransmission. No additional special handling is required to manage the coexistence of autonomous retransmission and the ARQ procedure.

2.2.2. Polling enhancement

According to the agreement in RAN2#129, polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. However, whether polling can be triggered only once per RLC SDU or multiple times has not been discussed.

In RAN2#129, it was agreed that a single autonomous retransmission would be triggered per RLC SDU. Similar to autonomous retransmission, polling should also be limited to one trigger per RLC SDU. On one hand, there may not be sufficient time left to trigger multiple pollings for a delay-critical RLC SDU. On the other hand, if multiple polling were allowed, mechanisms such as a prohibit timer would need to be introduced to avoid excessive triggering of polls, which would add unnecessary complexity without providing significant benefits.

Proposal 3: Similar to autonomous retransmission, polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold.

2.2.3. Coexistence of autonomous retransmission and polling enhancement

According to the agreement in RAN2#128, the network can configure either or both of autonomous retransmission and polling enhancement.

RAN2#128 Agreement

- Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In RAN2#129, it was agreed that separate thresholds for autonomous retransmission and polling be introduced.

RAN2#129 Agreement

- We have separate thresholds for autonomous reTx and for polling

Therefore, autonomous retransmission and polling can work independently according to the agreement, it is quite straightforward that separate UE capabilities should be introduced.

Proposal 4: Separate UE capabilities are introduced for autonomous retransmission and polling enhancement.

When both polling enhancement and autonomous retransmission are configured for the same Tx RLC entity, under reasonable network configuration, the threshold for enabling autonomous retransmission is expected to be set lower than that for enabling polling enhancement. Otherwise, the configuration for enabling polling enhancement will become useless since autonomous retransmission would have already been triggered before the UE polls the Rx RLC entity to request the STATUS report.

Additionally, unnecessary autonomous retransmission can reduce overall system capacity. Therefore, it should only be utilized when the RLC SDU(s) are particularly delay-critical. For RLC SDU(s) that are not delay-critical, polling enhancement should be preferred to avoid resource wastage caused by unnecessary autonomous retransmission.

Consequently, when both autonomous retransmission and polling enhancement are configured by the network, the threshold for enabling autonomous retransmission should be lower than the threshold for enabling polling enhancement (refer to Figure 4 below).

Figure 4. Different thresholds for enabling polling enhancement and autonomous retransmission

Proposal 5: The threshold for enabling autonomous retransmission is expected to be lower than the threshold for enabling polling enhancement when both are configured to the UE for the same RLC entity.

  - Enhancements to avoid unnecessary retransmissions

Indication is sent from Tx to Rx when the Tx decides the outdated SDU(s)

In RAN2#128, the Tx to Rx indicator for the combined approach was discussed, and the following agreements were made.

RAN2#128 Agreement

- There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer

- Unless critical issue is identified, no Tx to Rx indication will be introduced

In our view, the Tx to Rx indication is critical for keeping the timely synchronization between the Tx window and the Rx window. Without the indication, stalling of the Tx and Rx window may occur in some situations.

For example, when PDU set discard is enabled, all PDUs in the entire PDU set will be discarded if any one of the PDUs expires. At this time, the Tx RLC entity may have already stopped (re)transmission for amounts of obsolete RLC SDUs but has not received the STATUS report from the Rx side for a long time. This is because the local timer on the Rx side has not started due to no gap detected (refer to Figure 3 below). In this scenario, both the Tx window and the Rx window may stall and delay the transmission of the subsequent packets. With the indication from Tx to Rx, we can avoid this window stalling.

Figure 3. Window stalling due to without Tx to Rx indicator

Observation 3: Without Tx to Rx indication, stalling of the Tx and Rx window may occur, particularly in cases where PDU set discard is enabled.

Additionally, when the Tx side decides to stop (re)transmission of the RLC SDUs, the Rx side may still send STATUS reports with NACK for the corresponding RLC SDUs before the local timer on the Rx side expires. In practice, the Tx side will ignore these NACK indications for those RLC SDUs and will not perform (re)transmission. Therefore, these STATUS reports waste radio resources and should be avoided. With the Tx to Rx indication, the Rx can promptly identify the outdated SDUs based on the indication and send STATUS report with positive ACK without waiting for the local timer on the Rx side to expire.

Observation 4: Without the Tx to Rx indication, unnecessary STATUS reports with NACK indication for obsolete RLC SDUs will be sent to the Tx side, leading to resource wastage.

Proposal 6: Indication about obsolete SDUs from Tx side to Rx side is needed for the combined approach.

As for the format of the indication from the Tx to the Rx side, we believe a simple RLC control PDU carrying the sets of SNs or SN range of the outdated RLC SDUs is enough.

Proposal 7: The indication from the Tx side to the Rx side should include the sets of SNs or the SN range associated with the outdated RLC SDUs

Rx RLC indicates Rx PDCP when t-RxDiscard expires

When t-RxDiscard (the new RLC timer introduced to determine obsolete RLC SDUs) expires, the RLC entity stops waiting to receive the obsolete RLC SDUs and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, since this is a fake ACK, the transmitting PDCP entity will consider these RLC SDUs as successfully received based on the STATUS report, while the receiving PDCP entity will still be waiting to receive the RLC SDUs from the RLC entity until the t-reordering timer expires. In this case, HFN asynchronization between the transmitting PDCP entity and the receiving PDCP entity may occur. For example, the transmitting PDCP entity may consider the current HFN for a PDCP SDU to be X +1 based on the fake ACK from the STATUS report, while the receiving PDCP entity may consider it to be X. This problem has also been discussed in Nokia’s contribution [2].

Observation 5: HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary retransmission is configured.

To solve this problem, a simple approach is to make the receiving RLC entity indicate the SN of obsolete RLC SDUs when the t-RxDiscard expires, so that the receiving PDCP entity can update its state variable to maintain HFN synchronization with the transmitting PDCP entity.

Proposal 8: The receiving RLC entity should indicate to the receiving PDCP entity the SN of obsolete RLC SDUs when t-RxDiscard (the new Rx RLC timer to determine obsolete RLC SDUs) expires.

New RLF triggering approach

According to the current RLC and RRC specification, RLF will be triggered if the retransmission count of an RLC SDU reaches maxRetxThreshold. The purpose of this mechanism is to enable the UE to trigger RLF and initiate RRC re-establishment when it determines that uplink transmission is unavailable/unreliable.

However, when stopReTxObsoleteSDU (the feature of avoiding unnecessary retransmission) is configured to the Tx RLC entity, maxRetxThreshold might never be reached. This is because the UE will stop retransmitting the corresponding RLC SDU based on the discard indication from PDCP before reaching the maxRetxThreshold. To ensure that RLF can still be triggered in cases where uplink transmission becomes unavailable/unreliable, a new approach for triggering RLF needs to be introduced. For example, RLF could be triggered when:

- The RLC entity receives N consecutive SDU discard indications from the PDCP entity; or

- The RLC entity receives a total of N SDU discard indications from the PDCP entity within a configured time window.

Proposal 9: RAN2 should discuss the following approaches for triggering RLF when avoiding unnecessary retransmission is enabled:

- Approach 1: Triggering RLF when the RLC entity receives consecutive N SDU discard indications from the PDCP entity.

- Approach 2: Triggering RLF when the RLC entity receives a total of N SDU discard indications from the PDCP entity within the configured time window.

- Conclusion

In this contribution, we discuss the two directions of RLC AM enhancements, and we have the following observations and proposals:

Enhancements to ensure timely RLC retransmissions

Observation 1: Using a new RLC timer to calculate the remaining time may be not accurate, because the PDCP entity may buffer the packet for a period of time before delivering it to RLC, or some indication is still needed from PDCP to inform RLC to start the timer when the packets arrive at PDCP.

Observation 2: Determining the remaining time based on a newly introduced RLC timer incurs more specification impacts compared to utilizing the existing discardTimer in PDCP.

Proposal 1: Remaining time for triggering autonomous retransmission and polling enhancement should be determined based on the existing discardTimer in PDCP.

Proposal 2: The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs for autonomous retransmission over AMD PDUs for ARQ-based retransmission. No additional special handling is required to manage the coexistence of autonomous retransmission and the ARQ procedure.

Proposal 3: Similar to autonomous retransmission, polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold.

Proposal 4: Separate UE capabilities are introduced for autonomous retransmission and polling enhancement.

Proposal 5: The threshold for enabling autonomous retransmission is expected to be lower than the threshold for enabling polling enhancement when both are configured to the UE for the same RLC entity.

Enhancements to avoid unnecessary retransmissions

Observation 3: Without Tx to Rx indication, stalling of the Tx and Rx window may occur, particularly in cases where PDU set discard is enabled.

Observation 4: Without the Tx to Rx indication, unnecessary STATUS reports with NACK indication for obsolete RLC SDUs will be sent to the Tx side, leading to resource wastage.

Observation 5: HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary retransmission is configured.

Proposal 6: Indication about obsolete SDUs from Tx side to Rx side is needed for the combined approach.

Proposal 7: The indication from the Tx side to the Rx side should include the sets of SNs or the SN range associated with the outdated RLC SDUs

Proposal 8: The receiving RLC entity should indicate to the receiving PDCP entity the SN of obsolete RLC SDUs when t-RxDiscard (the new Rx RLC timer to determine obsolete RLC SDUs) expires.

Proposal 9: RAN2 should discuss the following approaches for triggering RLF when avoiding unnecessary retransmission is enabled:

- Approach 1: Triggering RLF when the RLC entity receives consecutive N SDU discard indications from the PDCP entity.

- Approach 2: Triggering RLF when the RLC entity receives a total of N SDU discard indications from the PDCP entity within the configured time window.

- References

- R2_129_ChairNotes.

- R2-2500490, RLC Enhancements, Nokia, Nokia Shanghai Bell.