3GPP TSG-RAN WG2 Meeting #129bis		R2-2502169

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Discussion on indication from MAC to RRC layer in RACH-based LTM

Agenda Item:	7.2

Document for:	Discussion and Decision

- Introduction

In this paper, we discuss the issue brought by the MAC indication to RRC layer when LTM is completed in current MAC specification and propose corresponding solutions.

- Discussion

  - Indication from MAC to RRC layer in RACH-based LTM

According to the latest MAC specification [1] on the completion of RACH-based MCG/SCG LTM:

### 5.1.6 Completion of the Random Access procedure Upon completion of the Random Access procedure, the MAC entity shall: 1> discard any explicitly signalled contention-free Random Access Resources for 2-step RA type and 4-step RA type except the 4-step RA type contention-free Random Access Resources for beam failure recovery request, if any; 1> flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer and the MSGA buffer. Upon successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall: 1> indicate the successful completion of the Random Access procedure to the upper layers. Upon successful completion of the Random Access procedure initiated for LTM cell switch, the MAC entity shall: 1> indicate the successful completion of the LTM cell switch to upper layers.

It could be observed that upon completion of the RACH-based MCG/SCG LTM cell switch, MAC layer needs to indicate to the RRC layer the successful completion of the LTM cell switch.

Observation 1: The completion of RACH-based LTM triggers the MAC to send the indication to RRC layer.

And according to current TS 38.331[2], it has the following description on the conditions of stopping the timer T304.

### 5.3.5.3	Reception of an RRCReconfiguration by the UE

#### ------omit-------

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1>	if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or,

1>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

2>	stop timer T304 for that cell group if running;

2>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

2>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

3>	release dedicated preambles provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

#### ------omit-------

### 5.3.5.3	Reception of an RRCReconfiguration by the UE

#### ------omit-------

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1>	if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or,

1>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

2>	stop timer T304 for that cell group if running;

2>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

2>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

3>	release dedicated preambles provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

#### ------omit-------

The highlighted part in yellow corresponds to the cases for cell change based on RACH procedure, i.e., it includes HO, CHO, MCG/SCG LTM, CPAC, and PScell change, that is to say, it includes the case for RACH-based LTM. While for the highlighted part in green, according to the observation 1, it also includes the RACH-based MCG/SCG LTM scenario, which results in the duplication with the highlighted in yellow.

And for the release of RACH resource, it should be only applied to the RACH-less LTM scenario for the procedure captured in RRC specification considering the RACH resource release procedure for RACH based LTM scenario has been captured in MAC specification upon the completion of the RACH procedure. However, the highlighted part in green also includes the RACH-based LTM scenario, which will result the RACH resource to be released twice for RACH-based LTM.

Observation 2: The completion of RACH-based LTM triggering the MAC to send the indication to RRC layer will result in the duplication to stop the timer T304 or release of RACH resource.

In our understanding, the initial intention of the green part above is only corresponding to the RACH-less MCG/SCG LTM scenario similar to the RACH-less HO procedure.

Observation 3: For the conditions to stop the T304 specified in current TS 38.331, the description intends to be divided into the following parts:

- RACH-based cell switch procedure, includes HO, CHO, MCG/SCG LTM, CPAC and PSCell change (highlighted in yellow);

- RACH-less based HO;

- RACH-less based MCG/SCG LTM (highlighted in green).

To achieve this purpose, we suggest to remove this cross-layer interaction when completion of the RACH-based LTM procedure. Furthermore, since for legacy RACH procedure, there is no explicit indication from MAC to RRC layer and RRC layer could also know the completion of RACH procedure, there is no need to introduce the explicit indication for RACH-based LTM procedure also. And if there is no such indication, the procedure in RRC specification is clear and correct.

Observation 4: No explicit indication from MAC to RRC layer for the completion of legacy RACH procedure.

Observation 5: Remove the indication from MAC to RRC layer for the completion of RACH-based LTM could make current RRC specification clearer.

Proposal 1: Remove the indication from MAC to RRC layer for the completion of RACH-based LTM.

The corresponding TP is provided in Annex 1.

Proposal 2: If P1 is agreeable, RAN2 to adopt the TP provided in Annex1.

- Conclusion

In this contribution, we discussed for indication in RACH-based LTM procedure. The following observations and proposals are proposed:

Observation 1: The completion of RACH-based LTM triggers the MAC to send the indication to RRC layer.

Observation 2: The completion of RACH-based LTM triggering the MAC to send the indication to RRC layer will result in the duplication to stop the timer T304 or release of RACH resource.

Observation 3: For the conditions to stop the T304 specified in current TS 38.331, the description intends to be divided into the following parts:

- RACH-based cell switch procedure, includes HO, CHO, MCG/SCG LTM, CPAC and PSCell change (highlighted in yellow);

- RACH-less based HO;

- RACH-less based MCG/SCG LTM (highlighted in green).

Observation 4: No explicit indication from MAC to RRC layer for the completion of legacy RACH procedure.

Observation 5: Remove the indication from MAC to RRC layer for the completion of RACH-based LTM could make current RRC specification clearer.

Proposal 1: Remove the indication from MAC to RRC layer for the completion of RACH-based LTM.

Proposal 2: If P1 is agreeable, RAN2 to adopt the TP provided in Annex1.

- References

- TS 38.321 v18.5.0, Medium Access Control (MAC) protocol specification.

- TS 38.331 v18.5.1, Radio Resource Control (RRC) protocol specification.

Annex – TP to TS 38.331

### 5.1.6	Completion of the Random Access procedure

Upon completion of the Random Access procedure, the MAC entity shall:

1>	discard any explicitly signalled contention-free Random Access Resources for 2-step RA type and 4-step RA type except the 4-step RA type contention-free Random Access Resources for beam failure recovery request, if any;

1>	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer and the MSGA buffer.

Upon successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall:

1>	indicate the successful completion of the Random Access procedure to the upper layers.