3GPP TSG-RAN2#129bis									          R2-2502202

Wuhan, China, April . 7 – 11, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	LCP enhancements for XR

Agenda item:	8.7.4.1

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for specify additional Logical Channel priority handling using delay/deadline information of packets to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

In RAN2#126 meeting, the following related agreements has been made [2]:

- LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial - The solutions should not disallow non-delay critical data from using an UL grant. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In RAN2#127 meeting, the following related agreements has been made [3]:

LCP Enhancement - RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. - FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

In RAN2#127bis meeting, the following related agreements has been made [4]:

- As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. For now, we will use “LCH priority-adjusted data”.

In RAN2#128 meeting, the following agreements have been made[5]:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In RAN2#129 meeting, the following agreements have been made[7]:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact. - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

In this contribution we will further investigate the possible scheduling enhancements per this objective.

# Discussion

It has been agreed to introduce an independent per-LCH remaining time threshold for applying delay-critical priority in RAN2, and LCP prioritization within a logical channel will not be considered in RAN2 discussions, based on the threshold, “LCH priority-adjusted data” can be determined per LCH, e.g. if there is data in one logical channel with remaining time less than or equal to the threshold, all the data in the logical channel will be prioritized for transmission, and the data in the logical channel will be called “LCH priority-adjusted data”.

Proposal 1: RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel having data with remaining time less than or equal to the remaining time threshold configured for LCP.

Considering that the remaining time for “LCH priority-adjusted data” determination is the he remaining time till discardTimer expiry, the “LCH priority-adjusted data” differentiation is used in RLC entity, but the discardTimer is maintained only in PDCP, either PDCP entity notify RLC entity that the remaining time of each RLC SDU, or PDCP entity notify RLC entity the remaining time of an SDU is below the remaining time threshold for LCP. Considering that the remaining time threshold is configured per LCH, and LCH is not visible in PDCP(e.g. only DRB is visible in PDCP), we prefer that PDCP entity notify RLC entity the remaining time of each RLC SDU, and RLC entity determines whether the remaining time of an SDU is below the remaining time threshold for LCP.

Proposal 2: PDCP entity notifies RLC entity the remaining time of each RLC SDU, and RLC entity determines whether the remaining time of an SDU is below the remaining time threshold for LCP.

In RAN2#129 meeting, there is the following FFS:

- FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination.

Although the WI objective is to “Specify additional Logical Channel priority handling using delay/deadline information of packets”, considering SR priority is also considered in LCP procedure for Intra-UE prioritization, the additional LCP priority for “LCH priority-adjusted data” can also used for SR priority determination.

Proposal 3: For “LCH priority-adjusted data”, the additional LCP priority is also used for SR priority determination.

In RAN2#129 meeting, there is the following FFS:

7. FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Considering that unfairness will occur for non LCH priority-adjusted data if allowing an LCH with an upgraded priority to be transmitted even if Bj is negative. e.g. the LCH with small discardTimer will always be prioritized, and all the LCH will be scheduled with large delay when the discardTimer is same(e.g. always prioritize the data with remaining time less than the threshold). So, in the first LCP round, an LCH with an upgraded priority should not be allowed to be transmitted when Bj is negative.

Proposal 4: Same as legacy, in the first round of LCP, only logical channels with Bj > 0 are allocated resources in a decreasing priority order, whatever the adjusted priority or legacy priority is used.

# Conclusion and proposals

The following proposals are made:

Proposal 1: RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel having data with remaining time less than or equal to the remaining time threshold.

Proposal 2: PDCP entity notifies RLC entity the remaining time of each RLC SDU, and RLC entity determines whether the remaining time of an SDU is below the remaining time threshold for LCP.

Proposal 3: For “LCH priority-adjusted data”, the additional LCP priority is also used for SR priority determination.

Proposal 4: Same as legacy, in the first round of LCP, only logical channels with Bj > 0 are allocated resources in a decreasing priority order, whatever the adjusted priority or legacy priority is used.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- 3GPP TS 38.321, V18.4.0 (2024-12), NR;Medium Access Control (MAC) protocol specification.

- Draft_RAN2_129_Meeting_Report_v2