3GPP TSG-RAN2#129bis									         R2-2502204

Wuhan, China, April  07 – 11, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	DSR enhancements for XR

Agenda item:	8.7.4.2

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG, to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

In RAN2#126 meeting, the following related agreements has been made [2]:

DSR enhancements – multiple thresholds and grouping of data - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 meeting, the following related agreements has been made [3]:

Enhanced DSR contents - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127bis meeting, the following related agreements has been made [4]:

- We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In RAN2#128 meeting, the following related agreements has been made [5]:

- =>Let the network configure the triggering and reporting thresholds without constraints. - RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR. - The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In RAN2#129 meeting, the following related agreements has been made [6]:

- One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size.

In this contribution we will further investigate the possible DSR enhancements per this objective.

# Discussion

It has been agreed in RAN2#127 meeting that network should be able to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes per LCG, but there will be a single triggering threshold. The value range of remainingTimeThreshold-r18 is INTEGER (1..64) ms. Very small remaining time (e.g. 1ms, 2ms etc ) are not useful as there is not enough time to report these such that network can receive the DSR, process this and respond with scheduling. Considering the useful range of the remaining times, at most four remaining time thresholds seem a reasonable choice of split and this. Only these configured remaining time thresholds along with the buffered data associated with these remaining time will be reported. For each of the portions reported, the eDSR should report the corresponding buffer size and the shortest remaining time among the PDCP SDUs in this portion, which is shown in the following figure. And for flexibility, variable number of remaining time thresholds (e.g.1, 2, 3 or 4 thresholds) should be allowed. The total number of remaining time thresholds to be used can be configurable.

Proposal 1: For DSR enhancement, at most four remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Considering that the data volume with remaining time larger than the largest reporting threshold can be reported by BSR, it is not necessary to report data volume with remaining time larger than the largest reporting threshold in the DSR.

Proposal 2: UE shall not report data volume with remaining time larger than the largest reporting threshold in the DSR.

In RAN2#129 meeting, there is a “FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds”. Considering that it has been agreed in RAN2#129 meeting that “One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE”, based on this agreement, the new DSR MAC CE is compatible with the legacy DSR MAC CE, e.g. the UE can identify whether the DSR MAC CE is a new DSR MAC CE or legacy DSR MAC CE based on the value of the reserved bit if the UE supports the new DSR MAC CE and at least one LCG is configured with multiple thresholds, new LC-ID is not necessary for the new DSR MAC CE.

Observation 1: The new DSR MAC CE is compatible with the legacy DSR MAC CE. The UE can identify whether the DSR MAC CE is a new DSR MAC CE or legacy DSR MAC CE based on the value of the reserved bit if the UE supports the new DSR MAC CE, no new LC-ID is necessary for the new DSR MAC CE.

Proposal 3: New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds, and not define new LC-ID for the new DSR MAC CE.

In the email discussion <[POST129][511][XR] PDCP running CR (LGE)>, the following option issues have been identified[7]:

Discuss the following issues in the RAN2#129bis meeting. - Whether the RAN2 agreement “non-delay critical data ahead of delay critical data” applies non-delay-reporting data ahead of delay-reporting data. - Whether to use COUNT to describe the non-delay-critical data ahead of delay-critical data. - Whether to use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set. - Whether to keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the PDCP specification. - Whether to specify delay-reporting indication in the PDCP specification or leave it up to UE implementation. - Whether to specify update of delay-reporting indication in the PDCP specification or leave it up to UE implementation.

The multiple reporting thresholds for one LCG should be configured in ascending order(e.g. the first entry corresponds to the smallest threshold, and the last entry corresponds to the largest threshold), but it is enough to describe this in RRC specification(e.g. in the field description), not in PDCP specification.

Proposal 4: The multiple DSR reporting thresholds for one LCG are configured in ascending order(e.g. the first entry corresponds to the smallest threshold, and the last entry corresponds to the largest threshold), which is described in RRC specification.

Considering that the delay-reporting data is the data that the remaining time below the remaining time threshold, which is similar as the delay critical data defined in rel-18, the rules of “non-delay critical data ahead of delay critical data” should apply to the non-delay-reporting data ahead of delay-reporting data.

Proposal 5: The rules of “non-delay critical data ahead of delay critical data” applies to the non-delay-reporting data ahead of delay-reporting data.

We understand that “non-delay-critical data ahead of delay-critical data” is to describe the case that there are non-delay-critical data ahead of delay-critical data in RLC queue, it should be described in RLC specification.

Proposal 6: Describe “non-delay-critical data ahead of delay-critical data” in RLC specification only.

In TS 38.323, it has been specified as follows:

When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU.

It means that if pdu-SetDiscard is configured, the discardTimers of all PDCP SDUs belonging to the PDU Set will expire if one of the discardTimer of one SDUs belonging to the PDU Set expires, no PDU set discardTimer is defined. And the remaining time is defined as “the remaining time till discardTimer expires”. Thus, similar as in rel-18, it is enough to describe “The shortest remaining time till discardTimer expiry among the PDCP SDU, and not define “PDU Set remaining time”.

Proposal 7: Similar as in rel-18, it is enough to describe “The shortest remaining time till discardTimer expiry among the PDCP SDU...”, and not define “PDU Set remaining time”.

Since DSR is only used in UL, and the delay-reporting indication is only used in UE internal, it can be based on UE implementation to determine delay-reporting information.

Proposal 8: delay-reporting indication and update of delay-reporting indication are up to UE implementation”.

# Conclusion and proposals

The following observations/proposals are made:

Observation 1: The new DSR MAC CE is compatible with the legacy DSR MAC CE. The UE can identify whether the DSR MAC CE is a new DSR MAC CE or legacy DSR MAC CE based on the value of the reserved bit if the UE supports the new DSR MAC CE, no new LC-ID is necessary for the new DSR MAC CE.

Proposal 1: For DSR enhancement, at most three remaining time thresholds can be configured for one LCG, and variable number of remaining time thresholds are allowed.

Proposal 2: UE shall not report data volume with remaining time larger than the largest reporting threshold in the DSR.

Proposal 3: New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds, and not define new LC-ID for the new DSR MAC CE.

Proposal 4: The multiple DSR reporting thresholds for one LCG are configured in ascending order(e.g. the first entry corresponds to the smallest threshold, and the last entry corresponds to the largest threshold).

Proposal 5: The rules of “non-delay critical data ahead of delay critical data” applies to the non-delay-reporting data ahead of delay-reporting data.

Proposal 6: Describe “non-delay-critical data ahead of delay-critical data” in RLC specification only.

Proposal 7: Similar as in rel-18, it is enough to describe “The shortest remaining time till discardTimer expiry among the PDCP SDU...”, and not define “PDU Set remaining time”.

Proposal 8: delay-reporting indication and update of delay-reporting indication are up to UE implementation”

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_R2_126_Meeting_Report_v2

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- Draft_RAN2_129_Meeting_Report_v2

- [POST129][511][XR] PDCP running CR (LGE)