3GPP TSG-RAN WG2 Meeting #129bis	R2-2502216

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.2.3

Source:		Futurewei

Title:		Further discussions on A-IoT random access

Document for:		Discussion and Decision

# Introduction

In TR 38.769 v19.0.0 [1], the general framework of slotted-ALOHA for A-IoT random access procedure is shown as in Figure 1 below, which is same as Figure 6.3.4-1 in TR 38.769.

Figure 1. General framework of slotted-ALOHA for A-IoT random access procedure

After the SI phase is ended, RAN2 has further agreed, in meeting#129, to “assume that device randomly selects among FDMA occasions as the baseline” [2], as an addition to the earlier agreement, reached in meeting#127bis, that “at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA” [3].

In this contribution, we discuss some detailed issues related to the framework for A-IoT random access.

# Discussions

According to TR 38.769, an access occasion (AO) is an opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device). Each column of square boxes in Figure 1 represents multiple AOs that are FDM’ed within a same access period. One or more columns of AOs may be scheduled as a result of a paging message. However, due to the poor clocking drifting performance at the devices, another R2D transmission is needed once a while to allow devices to correct their clocks using the synchronization signal in the R2D transmission. The AOs between the paging message and the first R2D transmission or between two consecutive R2D retransmissions can be considered an AO Set. One question is whether all AO Sets between the AIoT paging and the subsequent AIoT paging always have the same number of AOs or each AO Set can have a different number of AOs. If always the same number of AOs, it may make the job easier for the device to figure out the AO that it should respond, without a need for counting the number of AOs in each AO Set passed. RAN2 could make it an assumption that all AO Sets between the AIoT paging and the subsequent AIoT paging have the same number of AOs and ask RAN1 if they see any issue in it.

Proposal 1. RAN2 assumes that all AO Sets between the AIoT paging and the subsequent AIoT paging have the same number of AOs and ask RAN1 if they see any issues in it.

In the paging message and the subsequent paging message shown in Figure 1, the reader should include information about the size of random number to be drawn by the devices, which is similar to the Q value in RFID, for the device to generate a random number to be used in selecting its AO.

Proposal 2. In each paging message sent, the reader should include information about the size of random number to be drawn by the devices.

Proposal 3. The device generates a random number of the size as indicated and uses the random number for determining the AO where it should send in its response during the random-access procedure.

# Conclusions

We propose the following:

Proposal 1. RAN2 assumes that all AO Sets between the AIoT paging and the subsequent AIoT paging have the same number of AOs and ask RAN1 if they see any issues in it.

Proposal 2. In each paging message sent, the reader should include information about the size of random number to be drawn by the devices.

Proposal 3. The device generates a random number of the size as indicated and uses the random number for determining the AO where it should send in its response during the random-access procedure.

# References

TR 38.769 v19.0.0.

R2_129_ChairNotes_25-02-21_final.

R2_127bis_ChairNotes_24-10-18_13-00.