3GPP TSG-RAN WG2 Meeting #129bis			     R2-2502224

Wuhan, China, 7-11 April, 2025

Agenda Item:	8.7.6 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining issues on rate control signaling for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses remaining issues on XR UL rate control signalling based on RAN2 agreements and the post email discussions for R19 XR.

Agreements on XR rate control in RAN2#129 - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

Discussion

Even though the following was agreed in RAN2#128 meeting, RAN2 cannot discuss this issue in the RAN2#129 meeting due to lack of discussion time and this issue is still open from RAN2 point of view.

Agreements in RAN2#128 - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

For per QoS flow indication, two options can be considered based on the FFS above.

- Option 1. QoS flows are indicated by RRC while MAC CE is per DRB

- Option 2. QoS flows are indicated by MAC CE

In the option 1, multiple QoS flows can be mapped to one DRB and the gNB configures which QoS flow can be throttled in one DRB by RRC. Then if the UE receives a MAC CE with per DRB indication for XR UL rate control, the UE can throttle only QoS flows allowed for throttling by RRC configuration among all QoS flows in one DRB. We think that this option may have limitation to support finer QoS flow throttling since the MAC CE for XR UL rate control only has per DRB information with one bit rate filed applied to multiple QoS flows which may not throttle each QoS flow differently. In addition, if QoS flow which can be throttled in one DRB is changed, RRC signalling is always needed to change this configuration in one DRB. On the other hand, the option 2 has full flexibility to indicate which QoS flow should be throttled and RRC signalling is not needed even when QoS flow is changed to be throttled in one DRB. Note that even though the option 1 uses a MAC CE with per DRB, the legacy RBR MAC CE cannot be reused and separate new MAC CE with new LCID should be defined to make the UE recognize that this MAC CE with per DRB information needs QoS flow throttling based on RRC configuration. Considering aforementioned reason, it would be better to have new MAC CE with per QoS flow indication for XR UL rate control signalling.

We think that This issue is also related to two editor’s notes in the MAC running CR. If the per QoS flow indication is indicated by the MAC CE, it is obvious to introduce a new MAC CE for UL rate control and bit rate per QoS flow should be included in the new MAC CE.

- Editor’s Note: FFS granularity of the bit rate indicated in the MAC CE (e.g. either for a logical channel or a QoS flow in a logical channel.

- Editor’s Note: FFS whether a new MAC CE will be used for UL rate control or the legacy Recommended Bit Rate MAC CE can be reused.

Proposal 1. Per QoS flow with the bit rate is indicated by a new MAC CE for UL XR rate control.

Another editor’s note in the MAC running CR is as follow.

- Editor’s Note:	FFS whether this bit rate is physical-layer bit rate (as in the legacy recommended bit rate procedure) or some other type of bit rate.

In our understanding, the RBR MAC CE is targeted for VoIP and one QoS flow for VoIP may be associated with one DRB and the physical-layer bit rate would be sufficient to associate/control one QoS flow in one DRB/LCH for VoIP. However, one XR service can have multiple QoS flows, e.g., video, audio, and haptic, etc., and each of them may have different bit rate/QoS requirement. Each QoS flow can be mapped to a different DRB since multiple QoS flows only having similar bit rate/QoS requirement can be mapped to a same DRB. In this sense, it is not sure how the physical-layer bit rate can be associated with one QoS flow among multiple QoS flows in one DRB. The situation for UL XR rate control seems different from the legacy recommended bit rate and it would be better to use other term for bit rate of a QoS flow instead of physical-layer bit rate.

Proposal 2. Physical-layer bit rate (as in the legacy recommended bit rate procedure) is not used for UL XR rate control, i.e., just bit rate or other term for bit rate is defined for UL XR rate control.

RAN2 agreed to specify a new bit rate table for XR rate control in the last RAN2 meeting and now we can discuss details of a new table and MAC CE format.

SA4 reply LS in R2-2500056 The SA4 reply is as follows: The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps. The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

For a new table and MAC CE format, the followings should be decided.

- distribution, e.g., exponential, linear, or piecewise linear;

- the number of codepoint;

- multiplier;

- LCID space, i.e., eLCID;

The most important point is how many codepoints are defined for UL XR rate control. According to the RAN2 discussion in the last meeting, some companies want to have 6bit codepoint as in legacy RBR MAC CE to avoid overhead and other companies indicate that a new table may not need a multiplier. However, the SA4 reply clearly said that the required bit range is from 100kbps to 40Mbps and they prefer to have 1% granularity. It is obvious that 6bit codepoint cannot provide the new table with 1% granularity and if 6bit codepoint is adopted for UL XR rate control, multiplier may be needed.

Observation 1. 6bit codepoint is insufficient to support 1% granularity for range from 100kbps to 40Mbps for UL XR rate control and multiplier may be needed.

With the observation 1, as per the analysis in [1], at least 10bit codepoint is needed to support 1% granularity if exponential distribution is used, and 12bit codepoint is needed if piecewise linear distribution is used. Actually, we are fine with any distribution for the new table, but we wonder whether less bit codepoint, i.e., 8bit codepoint, is feasible to satisfy SA4 request since SA4 reply mentioned typical uplink data rate as shown yellow highlight above box and coarser granularity may also be sufficient, e.g., 10% granularity. Need of multiplier depends on how the new table look like, so this can be revisited after defining the new table for UL XR rate control as RAN2 already agreed in the last meeting.

Proposal 3. Exponential distribution with at least 8bit codepoint is used for the new table.

For LCID space, we don’t see any reasons to use reserved bits in LCID space for this new MAC CE and it should be sufficient to use eLCID space.

Proposal 4. eLCID value is used to indicate the new MAC CE for UL XR rate control.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. 6bit codepoint is insufficient to support 1% granularity for range from 100kbps to 40Mbps for UL XR rate control and multiplier may be needed.

Proposal 1. Per QoS flow with the bit rate is indicated by a new MAC CE for UL XR rate control.

Proposal 2. Physical-layer bit rate (as in the legacy recommended bit rate procedure) is not used for UL XR rate control, i.e., just bit rate or other term for bit rate is defined for UL XR rate control.

Proposal 3. Exponential distribution with at least 8bit codepoint is used for the new table.

Proposal 4. eLCID value is used to indicate the new MAC CE for UL XR rate control.

- Reference

[1] R2-2500095	Discussion on XR rate control	Qualcomm Incorporated	discussion