3GPP TSG-RAN WG2 Meeting #129bis			         R2-2502229

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.5.3

Source: 	LG Electronics Inc.

Title:         	Remaining issues on OD-SIB1

Document for: 	Discussion and Decision

Introduction

In this contribution we will discuss remaining issues on the on-demand SIB1 operation.

Discussion

- Issue 1. SIB X validity while T311 is running

In the last meeting, RAN2#129, the followings were agreed to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - The UE follows the legacy validity principle of stored SIB.

If UE transmits WUS according to invalid WUS configuration while T311 is running, the UE will fail to acquire OD-SIB1 from the NES Cell and try to find another cell for RRC connection re-establishment. Since it takes quite a long time to detect the OD-SIB1 request failure, this may lead to unacceptable delay in the RRC connection recovery. Therefore, UE should be able to determine whether the stored WUS configuration is still valid or not while T311 is running. For this purpose, RAN2 agreed to follow the legacy validity principle for stored WUS configuration.

However, the legacy SIB validity check works only when the UE has a serving cell, because it is based on comparing the first PLMN identity, SI area ID, and the value tag of the SIB received from the current serving cell and those values associated with the stored SIB. Since UE has no serving cell while T311 is running, UE cannot check the validity of the stored SIB X using the existing method.

TS 38.331 V18.4.0 3> else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell;

Observation 1	Since UE has no serving cell when T311 is running, the UE cannot check the validity of the stored SIB X using the existing method.

Since the cell selection will be performed immediately after an RLF occurs in the PCell, the SIB X that was valid in the most recent serving cell is highly likely to still be valid when T311 is running. Therefore, it seems reasonable when T311 running, UE checks the validity of the stored SIB X by comparing the PLMN-Identity, systemInformationAreaID and valueTag of the last serving cell and those of stored SIB X.

Proposal 1	When T311 is running, if the first PLMN-Identity, systemInformationAreaID and valueTag received from the last serving cell are identical to those of the stored SIB X, UE considers the stored SIB X as valid.

- Issue 2. Paging reception interruption due to OD-SIB1

After transmitting WUS to a neighbor NES cell, the UE should monitor the neighbor NES cell during RAR and following OD-SIB1 window to successfully acquire OD-SIB1. However, the RAR window or OD-SIB1 window can be overlapped with the UE’s paging occasion. Then, the UE should select one cell between them to monitor during the overlapping period. If UE monitors the NES cell during the overlapping period, the UE will miss Paging.

Figure 1 Paging reception interruption

Observation 1	The UE may miss Paging while monitoring neighbor cell during the RAR window or OD_SIB1 window.

Since the paging reception from serving cell is much more critical than acquiring SIB1 of the neighbor cell, the receiving RAR and monitoring OD-SIB1 window on neighbor cell should not interfere with the paging reception. If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, UE should monitor the serving cell during the overlapping period.

Proposal 2	If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, the UE monitors the serving cell during the overlapping period.

Conclusion

Proposal 1	When T311 is running, if the first PLMN-Identity, systemInformationAreaID and valueTag received from the last serving cell are identical to those of the stored SIB X, UE considers the stored SIB X as valid.

Proposal 2	If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, the UE monitors the serving cell during the overlapping period