3GPP TSG-RAN WG2 Meeting #129bis	R2-2502231

Wuhan, China, Apr. 7th – 11th , 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on LCM and specification impact for AI/ML mobility

Agenda Item:	8.3.5

Document for: 	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper will discuss the potential specification impacts of AI/ML for mobility enhancements, considering RRM measurement prediction and measurement event prediction.

- Potential specification impacts of RRM measurement prediction

At the RAN2 #129 meeting, RAN2 decided to kick off the specification impact discussions based on the following agreements[5],

1. RLF prediction will not be studied in Rel-19

2.Further RRM prediction simulations are not expected.  For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction.   FFS if L3 beam level needs to be considered.  For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

We discuss the potential specification impacts of RRM measurement prediction use case in this section. Based on the experiences of Rel-18 and Rel-19 AI4AI studies and normative work, the specification impacts of an AI/ML for air interface use case usually focus on the three categories of aspects,

- Aspects related to the data, including configurations and collections for model inference, training, or performance monitoring. Ensuring the consistency of these data, especially the consistency between inference and training data, is also an issue to be addressed.

- Aspects related to model management, including the LCM framework for model applicability, activation/deactivation, and fallback.

Both NW- and UE-sided models are under the scope of this study according to SID [1]. There are significant differences between NW- and UE-sided models regarding the impact of specifications.

For the NW-sided model, the AI/ML model is implemented by the NW, and it is transparent to UEs. It is not necessary to indicate to UE whether the NW is using an AI/ML model. But the UE should provide necessary inference, training, or performance monitoring data to the NW. Therefore, there may still be potential specification impacts for aspects related to the data.

For the UE-sided model, both aspects should be studied, as AI/ML models are no longer transparent to the UE.

Therefore, we have the following observations and proposals,

Observation 1

- NW-sided model: The AI/ML model is transparent to UEs. The study should focus on the data-related aspects without UE awareness of the existing AI/ML models.

- UE-sided model: All three aspects should be studied.

Proposal 1

- For RRM measurement prediction, study the specification impacts for NW-sided and UE-sided models, respectively.

  - For the NW-sided model, the study focuses on data-related aspects.

  - For the UE-sided model, the study focuses on data-related and model management aspects.

As RAN2 agreed in the last meeting [5], the impact of measurement prediction on the temporal, frequency, and spatial domains will be studied. For NR RRM measurement, all the temporal, frequency, and spatial domain behaviors are configured under the NR measurement framework. They have similar specification impacts. Therefore, a unified design for the prediction over all three domains is possible and should be pursued in RAN2 studies.

Proposal 2

- For RRM measurement prediction, strive for a unified design for temporal, frequency, and spatial domain predictions.

In the following, we will discuss the details for NW- and UE-sided models, respectively.

  - NW-sided model

UE should provide the following data to the NW for the NW-sided model based on the configurations from the NW,

- Data for NW side model inference;

- Data for performance monitoring on the NW side;

- Data for training of the NW side model.

Following the design principle of NR RRM measurement, NW should fully control the UE measurements and reports. The UE does not need to know the purpose and connections of the data. Therefore, for inference and NW-side performance monitoring, UE measurements and reports should be incorporated into the existing RRM measurement framework. UE measures and reports the necessary data to the NW for these purposes. For the NW-side model training, the outcome of the NW-side data collection framework, as discussed in RAN2 Rel-19 AI/ML for air interface, should serve as a baseline. Our proposal is then,

Proposal 3

- For RRM measurement prediction, study the specification impacts for the NW-sided model regarding data for NW-side inference, performance monitoring, and training.

  - For NW-sided inference and performance monitoring, consider the existing RRM measurement framework as a baseline.

  - For NW-sided training, consider the NW-side data collection framework being studied in RAN2 Rel-19 AI4AI as a starting point, and examine the necessary case-specific enhancements, e.g., adding Layer 3 measurement results as a data item that the UE needs to collect and report.

  - UE-sided model

Both data-related and model-related aspects should be studied for the specification impacts of UE-sided models. Because the AI/ML models are running on the UE side, the UE capabilities in terms of computation power and storage should also be studied. Then, the potential specification impacts regarding UE-sided model can include the following,

Data-related aspects

- Input and output for the UE-side inference prediction, and the corresponding reports.

  - For inference input and output, it is necessary to study whether enhancing the existing framework to configure the UE predictions and support prediction reports is necessary.

- Tags and metrics for performance monitoring, and the corresponding reports.

  - The first issue to be addressed is whether the performance monitoring is conducted at the UE or the NW side.

  - If the monitoring is conducted on the NW side, the specification impacts may be similar to the NW side model. However, for UE-side performance monitoring, the UE needs to derive some KPIs for the monitoring and report the monitoring KPIs or results to the NW, which has significant specification impacts.

- Input and training tags for UE-side training.

  - Investigating how to configure the UE to collect this data is recommended.

- The consistency among these data, especially between training and inference.

  - RAN2 should identify whether the inconsistency in NW-side conditions has a significant impact on prediction performance. If it does, the mechanism under discussion in Rel-19 AI/ML for the air-interface topics, such as the associated ID, can be adopted.

Model management aspects

- General procedures have been studied in RAN2 Rel-19 AI/ML for the air interface topics, which can serve as a good reference.

UE capability aspects

- About UE capabilities on AI/ML processing and storage.

  - UEs usually have limited capabilities for executing AI/ML models and/or storing the necessary AI/ML models and data. Those capabilities may be shared among all AI/ML for the air-interface use cases. This issue is under investigation in Rel-19 AI/ML for the air interface. If this use case also shares the same UE capabilities, it should be discussed as well.

Proposal 4

- For RRM measurement prediction, study the specification impacts of the following listed issues for the UE-sided model.

  - Data-related aspects

    - Input and output for the UE-side inference prediction, and the corresponding reports.

    - Tags and metrics for performance monitoring, and the corresponding reports.

    - Input and training tags for UE-side training.

    - The consistency among these data, especially between training and inference.

  - Model management aspects

    - For model management, use the procedures of Rel-19 AI/ML for the air-interface use cases as the baseline.

  - UE capability aspects

- Potential specification impacts of measurement event prediction

At least the indirect measurement event prediction is closely tied to the RRM measurement prediction, as it is based on the RRM measurement predictions. The following are potential differences between the two use cases, which may have additional specification impacts at least on inference and monitoring results reports, and data collection.

- Different types of prediction: Predicted events vs. predicted measurement values. For inference, whether the measurement event should be enhanced to support the predicted measurement events should be studied. For training and performance monitoring, the ground-truth tags of the measurement events should be collected by the UE, which also has specification impacts.

- Different performance monitoring: Metric about miss/false prediction (e.g., F1-score) vs. Metric about prediction error (e.g., RSRP gap). Those differences may impact the UE behaviors for performance monitoring.

Following the discussions and agreements from previous meetings, the direct measurement event prediction differs from the indirect one in the following ways, which may have additional specification impacts.

- Direct prediction may utilize the L1-filtered measurement results as inputs to the model, while also considering some additional inputs. Whether these differences impact the specifications should be studied.

- Direct prediction reports a probability or binary prediction of the event occurrence within a time window in the current RAN2 study, where the definition of the window may differ from that defined for the indirect prediction.

Because the measurement event prediction is UE-sided, the differences in the model input may not have specification impacts. At the same time, we can strive for a common framework to accommodate the window definitions. Therefore, a unified design for both indirect and direct measurement event predictions is possible and should be studied to reduce the diverse implementations and the NW operation burden for handling different implementations.

Proposal 5

- For measurement event prediction, study the additional specification impacts on the following issues based on the outcomes from UE-side RRM measurement prediction.

  - Data collection.

  - Inference report.

  - Performance monitoring.

- For measurement event prediction, study whether different specification supports are necessary for indirect and direct predictions.

- Conclusion

In this contribution, we have the following observations and proposals,

Observation 1

- NW-sided model: The AI/ML model is transparent to UEs. The study should focus on the data-related aspects without UE awareness of the existing AI/ML models.

- UE-sided model: All three aspects should be studied.

Proposal 1

- For RRM measurement prediction, study the specification impacts for NW-sided and UE-sided models, respectively.

  - For the NW-sided model, the study focuses on data-related aspects.

  - For the UE-sided model, the study focuses on data-related and model management aspects.

Proposal 2

- For RRM measurement prediction, strive for a unified design for temporal, frequency, and spatial domain predictions.

Proposal 3

- For RRM measurement prediction, study the specification impacts for the NW-sided model regarding data for NW-side inference, performance monitoring, and training.

  - For NW-sided inference and performance monitoring, consider the existing RRM measurement framework as a baseline.

  - For NW-sided training, consider the NW-side data collection framework being studied in RAN2 Rel-19 AI4AI as a starting point, and examine the necessary case-specific enhancements, e.g., adding Layer 3 measurement results as a data item that the UE needs to collect and report.

Proposal 4

- For RRM measurement prediction, study the specification impacts of the following listed issues for the UE-sided model.

  - Data-related aspects

    - Input and output for the UE-side inference prediction, and the corresponding reports.

    - Tags and metrics for performance monitoring, and the corresponding reports.

    - Input and training tags for UE-side training.

    - The consistency among these data, especially between training and inference.

  - Model management aspects

    - For model management, use the procedures of Rel-19 AI/ML for the air-interface use cases as the baseline.

  - UE capability aspects

Proposal 5

- For measurement event prediction, study the additional specification impacts on the following issues based on the outcomes from UE-side RRM measurement prediction.

  - Data collection.

  - Inference report.

  - Performance monitoring.

- For measurement event prediction, study whether different specification supports are necessary for indirect and direct predictions.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #128 Chair Notes”, Nov., 2024.

- 3GPP TR36.839 v11.1.0, Mobility enhancements in heterogeneous networks (Release 11), Dec., 2012.

- 3GPP TR 38.843 v2.0.0, Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface (Release 18), Dec., 2023.

- RAN2 Chair (InterDigital), “RAN2 #129 Chair Notes”, Feb., 2025.