3GPP TSG-RAN WG2 Meeting #129bis	R2-2502235

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.6.2

Source: 	China Telecom

Title:	Discussion on Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last few RAN2 meetings, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding inter-CU LTM, the following agreements were reached in last RAN2 meeting.

Agreements on inter-CU LTM:

- For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID.

- Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

- The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

- UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG.

- FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

- The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

- The release of the LTM configuration (MCG or SCG) follows the Rel-18 principles.

- A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

- Add extension mark “…” to LTM-ConfigSCG-r19, adopt the principle that fields only applicable to inter-CU SCG change should be defined in the root of LTM-ConfigSCG-r19, not LTM-Config-r18.

- Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s).

- Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- How to support shared RACH resources for inter-CU LTM is left to RAN3

However, there are still some open issues on the details of inter-CU LTM in both non-DC and DC scenarios. In this contribution, we would like to share our opinions on the following remaining issues for inter-CU LTM.

- Security key handling for inter-CU LTM in non-DC case;

- Fast recovery handling for inter-CU LTM;

- Coexistence of one-shot LTM and subsequent LTM;

- Inter-CU MCG LTM.

- Discussion

Security key handling for inter-CU LTM in non-DC case

In Rel-18, subsequent LTM for intra-CU scenario is supported and security key is maintained upon an LTM cell switch. For inter-CU LTM, the handover parameters exchanged between gNBs, and then needs to be transmitted over Uu interface. Frequent update of handover parameters results in high inter-node singling overhead. By supporting subsequent LTM, the signalling overhead for over Xn and Uu interface can be reduced to a certain extent. Thus, subsequent LTM for inter-CU scenario will also be supported in Rel-19.

To support inter-CU subsequent LTM, how to handling the security key update is the most essential issue to be solved, which also involves coordination with RAN3/SA3. In RAN2#128 meeting, RAN2 sent a reply LS R2-2411136 on the security handling for inter-CU LTM in non-DC cases to request SA3 to avoid RRC configuration between cell switches for subsequent inter-CU LTM in solution selection. In reply LS R2-2501756, the following information is provided by SA3.

Copied from R2-2501756

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.

In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

Based on SA3’s reply, to support the delivery of NCC, further enhancement on the LTM Cell Switch Command MAC CE is needed. Regarding how to deliver the NCC value, potential solutions are listed as follows based on the previous discussion in RAN2.

- Option 1: the real NCC value is delivered in LTM CSC MAC CE;

- Option 2: the index of NCC value is delivered in LTM CSC MAC CE.

Option 1 may have less spec impact, while Option 2 can give some kind of protection on the real NCC value, since only the index is transmitted to the UE in plaintext via MAC CE. From network operator’s perspective, to avoid security threats is very important. In Option 2, the mapping of the index and the real NCC value could be preconfigured by RRC message, which is secure. Thus, we prefer to go for option 2.

Proposal 1: Regarding the security key handling for inter-CU LTM in non-DC case, an index of NCC value could be delivered in LTM CSC MAC CE.

Proposal 2: The mapping of the index and the real NCC value could be preconfigured by RRC message. The details can be discussed in CR drafting phase.

In current spec, the IE NextHopChainingCount is used to update the KgNB key and corresponds to parameter NCC and the IE keySetChangeIndicator indicates whether UE shall derive a new KgNB. No matter which option is accepted, similar information needs to be indicated to the UE. To indicate whether UE shall derive a new KgNB, a 1-bit field could be introduced in LTM CSC MAC CE. To indicate the NCC-related field, 3 bits is enough. Since there are 5 Reserved bits in current LTM CSC MAC CE, there is no need to introduce a new LTM CSC MAC CE for R19 inter-CU LTM.

Proposal 3: The Reserved bit in LTM CSC MAC could be used to deliver the information for security key update:

- a 1-bit field could be introduced to indicate whether the UE shall derive a new KgNB;

- a 3-bit field could be introduced to indicate the NCC or the index of NCC to be used.

Fast recovery handling for inter-CU LTM

In previous RAN2 meetings, the following agreements on fast recovery handling for inter-CU LTM were reached.

Agreements in RAN2#127bis

1. The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

Agreements in RAN2#128

1. RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

In the reply LS from SA3, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message. Based on SA3’s conclusion, we can reopen the fast recovery for inter-CU LTM discussion in RAN2. Since the mix configuration of intra-CU and inter-CU candidate cell is supported in R19, there are the following fast recovery handling cases to be considered.

- Case 1: After an intra-CU LTM failure, the UE selects an intra-CU LTM candidate cell;

- Case 2: After an intra-CU LTM failure, the UE selects an inter-CU LTM candidate cell;

- Case 3: After an inter-CU LTM failure, the UE selects an intra-CU LTM candidate cell;

- Case 4: After an inter-CU LTM failure, the UE selects an inter-CU LTM candidate cell.

Case 1 is actually what we have already agreed in RAN2#127bis, and the Rel-18 handling on failure recovery is reused. For Case 2 and 4, since the NCC is delivered by the LTM CSC MAC CE, there is no NCC upon the fast recovery towards an inter-CU candidate cell. The fast recovery will be failed again. For Case 3, since the first LTM is an inter-CU LTM, PDCP has already been re-established during the first LTM procedure. After the LTM failure, the fast recovery towards an intra-CU LTM may not work either. It is obvious that the R18 fast recovery cannot work for other cases expect Case 1. To support the other cases, certain enhancement is needed. However, considering the limited TU left, we suggest not to support fast recovery for those cases at all, since the fast recovery is a best effort way for the radio link recovery. Even if we have perfect enhancement towards those cases, it is still possible to fail again. For those cases, we can just rely on the RRC re-establishment as usual.

Proposal 4: RAN2 confirms to only support the fast recovery towards an intra-CU candidate cell after an intra-CU LTM failure, which is an R18 case.

Proposal 5: The fast recovery towards an inter-CU candidate cell after an intra-CU LTM failure is not supported.

Proposal 6: The fast recovery after an inter-CU LTM failure is not support.

Considering the mix configuration of intra-CU and inter-CU candidate cells in R19, the Rel-19 Set ID could be used for identifying whether the selected candidate cell is inter-CU or intra-CU.

Proposal 7: The Rel-19 Set ID is used for the UE to identify whether the selected candidate cell is inter-CU or intra-CU during the fast recovery.

Coexistence of one-shot LTM and subsequent LTM

In Rel-16/17, the feature of CPAC is introduced. To reduce signalling overhead, subsequent CPAC is introduced in Rel-18. Since the procedure of CPAC and subsequent CPAC are introduced separately in different release, the coexistence issue was discussed in Rel-18. It is agreed that coexistence of CPAC and subsequent CPAC for the same candidate SN is not supported.

For LTM, one-shot LTM and subsequent LTM may influence the signalling interaction among the network nodes. It is better to clarify the support of subsequent LTM is mandatory or optional for a certain candidate gNB or SN.

Taking inter-SN SCG LTM as an example, Fig.1 illustrates a potential stage-2 signalling flow for SN-initiated inter-SN SCG LTM based on the agreements made in previous RAN2 meetings. In step 5~6, the MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. Then the source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN. If the support of subsequent LTM is mandatory, step 7~8 shall be mandatory to transfer the common CSI resource configuration and the collected information of candidate cells to each candidate SN. The candidate SN(s) responds with the updated candidate SCG configuration to the MN. If the support of subsequent LTM is optional, step 7~8 could be optional as well.

Fig.1 Stage-2 signalling flow for SN-initiated inter-SN LTM without MN changed

Proposal 8: RAN2 clarifies whether the support of subsequent LTM is mandatory or optional for a certain candidate gNB or candidate SN.

Inter-CU MCG LTM

In previous RAN2 meeting, the following scenarios were agreed to be supported for inter-CU MCG LTM.

Agreements in RAN2#127

SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

Agreements in RAN2#128

RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

Regarding the details on inter-CU MCG LTM in DC scenarios, further discussion might be needed. For inter-CU MCG LTM in DC, the interaction between the UE and the MN is the same as that in non-DC case and the stage-2 signalling flow in draftCR of TS 38.300 can be referred to. For the interaction between network nodes, the procedure of CHO with SN as illustrated in Figure 10.19.2-1 of TS 37.340 could be followed as baseline. In the running CR for 37.340 in R2-250xxxx, the following Stage-2 signalling flow for inter-CU MCG LTM with SCG is provided as an example.

Fig.2 An example flow chart for inter-MN MCG LTM with Secondary Node

As illustrated above, for an inter-CU MCG LTM without SN change, the source SN and the target SN shown in Fig.2 are the same node. For an inter-CU MCG LTM with SN addition, the source SN and steps involving the source SN in Fig.2 are ignored. For the case of inter-CU MCG LTM with SCG release, it is also covered in Fig.2. Since similar procedure of Conditional Handover with SN could be followed, the spec impact seems not too much. Thus, we suggest RAN2 confirm to support inter-CU MCG LTM with SCG addition in Rel-19.

Proposal 9: RAN2 confirms to support inter-CU MCG LTM with SCG addition in Rel-19.

Although similar procedure as CHO with SN could be followed for inter-CU MCG LTM with SCG, it is better to have a separate stage-2 signalling flow and corresponding step descriptions in TS 37.340. Fig.2 could be used as a baseline and details of the signalling flow and procedure descriptions can be discussed in CR drafting phase with a tight coordination of RAN3.

Proposal 10: Fig.2 could be taken as baseline for stage-2 signalling flow of inter-CU MCG LTM with SCG. The details can be discussed in CR drafting phase with a tight coordination of RAN3.

- Conclusion

In this contribution, we discuss inter-CU LTM related aspects in Rel-19. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding the security key handling for inter-CU LTM in non-DC case, an index of NCC value could be delivered in LTM CSC MAC CE.

Proposal 2: The mapping of the index and the real NCC value could be preconfigured by RRC message. The details can be discussed in CR drafting phase.

Proposal 3: The Reserved bit in LTM CSC MAC could be used to deliver the information for security key update:

- a 1-bit field could be introduced to indicate whether the UE shall derive a new KgNB;

- a 3-bit field could be introduced to indicate the NCC or the index of NCC to be used.

Proposal 4: RAN2 confirms to only support the fast recovery towards an intra-CU candidate cell after an intra-CU LTM failure, which is an R18 case.

Proposal 5: The fast recovery towards an inter-CU candidate cell after an intra-CU LTM failure is not supported.

Proposal 6: The fast recovery after an inter-CU LTM failure is not support.

Proposal 7: The Rel-19 Set ID is used for the UE to identify whether the selected candidate cell is inter-CU or intra-CU during the fast recovery.

Proposal 8: RAN2 clarifies whether the support of subsequent LTM is mandatory or optional for a certain candidate gNB or candidate SN.

Proposal 9: RAN2 confirms to support inter-CU MCG LTM with SCG addition in Rel-19.

Proposal 10: Fig.2 could be taken as baseline for stage-2 signalling flow of inter-CU MCG LTM with SCG. The details can be discussed in CR drafting phase with a tight coordination of RAN3.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

3GPP TS 38.300 18.1.0: " NR; NR and NG-RAN Overall Description; Stage 2".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB