3GPP TSG-RAN WG2 Meeting #129bis	           R2-2502241

Wuhan, China, April 7th – April 11th, 2025

Agenda item:	8.13.2

Source:	China Telecom

Title:	Multi-hop relay discovery and reselection

Document for:	Discussion

- Introduction

In this paper, we would like to further discuss the potential enhancements to relay discovery and (re)selection to support additional hops, such as discovery conditions for the intermediate relay UEs, relay (re)selection radio criteria, relay reselection trigger, backward compatibility and so on.

- Discussion

2.1	General

Regarding backward compatibility, in our understanding, in order to make more Remote UEs located in poor coverage able to communicate with the network via Relay UE(s). RAN2 should try to enable Rel-17 U2N Remote UE to communicate with the network via Rel-19 U2N Relay UE(s) when designing multi-hop relay mechanism.

Proposal 1: Support Rel-17 U2N Remote UE to communicate with the network via Rel-19 U2N Relay UE(s) when designing multi-hop relay mechanism.

2.2	Relay discovery

RAN2 has the following agreements on discovery conditions for multi-hop Relay UE:

RAN2#127: A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage. FFS if acting as a relay requires it to transition to RRC_CONNECTED. Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage. If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). FFS discovery conditions for the intermediate relay UEs. RAN2#128: A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound. RAN2#129： The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell. FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive. Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay. For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient). For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied. FFS if the PC5 link is not already established.

The Rel-17 U2N Relay UE uses a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, which are configured by the network, to determine if it can transmit Relay discovery messages to U2N Remote UE(s). Obviously, this is also applicable for last relay UE (i.e. the U2N relay UE directly connected to the gNB) in Rel-19, which was agreed during RAN2#127 meeting. However, according to the agreements, some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage. If intermediate Relay UE is out of coverage, the Uu RSRP measurement is unavailable for intermediate Relay UE, such as first relay and second relay. Thus the Rel-17 Uu RSRP threshold can’t be used for OOC intermediate Relay UE.

Take one additional hop relay scenario for example, first relay UE out of coverage can use a maximum SD-RSRP/SL-RSRP threshold, a minimum SD-RSRP/SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s). The maximum SD-RSRP/SL-RSRP threshold, and minimum SD-RSRP/SL-RSRP threshold can be configured by the network via RRC signalling. With the extended threshold conditions, the interference introduced by first relay UE is limited while close enough to last relay UE. In addition, the extended threshold conditions also apply to two additional hops relay scenario.

Proposal 2: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SD-RSRP threshold, or a minimum SD-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 3: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SL-RSRP threshold, or a minimum SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

2.3	Relay selection/reselection

RAN2#129 meeting has the following agreements on multi-hop Relay UE (re)selection:

For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE. For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition: - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold. - When U2N Remote UE receives a PC5-S link release message from current first relay UE. - When U2N Remote UE detects PC5 RLF with the current first relay UE. - Indicated by upper layer. L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification. FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger)

Rel-17 defined the following cases triggering U2N Relay UE reselection:

-	PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold;

-	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling;

-	When U2N Remote UE receives a PC5-S link release message from U2N Relay UE;

-	When U2N Remote UE detects PC5 RLF;

-	Indicated by upper layer.

However, in terms of the second bullet, for intermediate relay UE, there may be some new cases. Single-hop U2N Relay UE communicates with the network directly, which can be considered as last relay UE in multi-hop relay. The intermediate relay UE may perform path switch, such as switch from indirect path to direct path, indirect path to indirect path. In that case, the intermediate relay UE may need to indicate path switch to the connected remote UE, which may trigger the remote UE to perform relay UE reselection. In another case, the intermediate relay may reselect its relay UE, and the reselected relay UE’s serving cell may be different from or the same as the former serving cell. Thus we think Rel-19 need to introduce new indication type to trigger U2N Relay UE reselection, such as path switch. Upon intermediate relay UE performing path switch, it can notify its child node.

Proposal 4: Upon intermediate relay UE performing path switch, it can notify its Rel-19 child node.

On the other hand, as we indicated in 2.1, RAN2 should try to support Rel-17 Remote UE to communicate with the network via Rel-19 U2N Relay UE(s) when designing multi-hop relay mechanism in order to make more Remote UEs located in poor coverage able to communicate with the network. However, Rel-17 Remote UE can’t understand new causes (such as path switch, relay selection, relay reselection) in notification message. In case of that, the intermediate relay UE can send PC5-S link release message to the Rel-17 remote UE.

Proposal 5: Upon intermediate relay UE performing path switch or relay (re)selection, it sends PC5-S link release message to the Rel-17 remote UE.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: Support Rel-17 U2N Remote UE to communicate with the network via Rel-19 U2N Relay UE(s) when designing multi-hop relay mechanism.

Proposal 2: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SD-RSRP threshold, or a minimum SD-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 3: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SL-RSRP threshold, or a minimum SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 4: Upon intermediate relay UE performing path switch, it can notify its child node.

Proposal 5: Upon intermediate relay UE performing path switch or relay (re)selection, it can send PC5-S link release message to the Rel-17 remote UE.