3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502259

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.2.1

Source:	Huawei, HiSilicon

Title:	Initial Text Proposal for A-IoT MAC specification

Document for: Discussion and agreement

# 1	Introduction

RAN#107 decided to create a new TS to specify the MAC protocol layer of Ambient IoT, i.e., TS38.391. Considering the practice for a new specification is to endorse the skeleton first, a skeleton of clause headings is provided in R2-2502258. On top of that, this document provides the text proposals to implement the current RAN2 agreements in the A-IoT MAC specification, which can be taken as a starting point for the A-IoT MAC running CR discussion.

# Initial Text Proposal for A-IoT MAC specification:

| 3GPP TS 38.391 V0.1.0 (2025-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Ambient IoT Medium Access Control (MAC) Protocol specification (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	General	7

4.1	Introduction	7

4.2	A-IoT MAC architecture	8

4.3	Services	8

4.3.1	Services provided to upper layers	8

4.3.2	Services expected from physical layer	8

4.4	Functions	8

5	A-IoT MAC procedures	9

5.1	General	9

5.2	A-IoT paging	9

5.3	A-IoT random access procedure	10

5.3.1	Initialization	10

5.3.2	Selection of random access type 	10

5.3.3	Contention-Based Random Access procedure	10

5.3.3.1	Selection of the access occasion for the D2R transmission of Random ID message	10

5.3.3.2	Transmission of Random ID message	10

5.3.3.3	Reception of Random ID Response message	10

5.3.4	Contention-Free Random Access procedure	11

5.4	A-IoT upper layer data transmission	11

5.4.1	D2R message transmission	11

5.4.2	R2D message reception	11

5.4.3	Segmentation	11

5.5	Failure detection	12

6	Protocol Data Units, formats and parameters	12

6.1	Protocol Data Units	12

6.1.1	General	12

6.2	A-IoT MAC messages	13

6.2.1	A-IoT Paging message	13

6.2.2	Random ID message (Msg1)	13

6.2.3	Random ID Response message (Msg2)	14

6.2.4	D2R Data Transfer messages	14

6.2.4.1	D2R Upper Layer Data Transfer message	14

6.2.5	R2D Data Transfer messages	14

6.2.5.1	R2D Upper Layer Data Transfer message	14

6.2.5.2	[D2R Scheduling message]	15

6.2.6	[Failure Indication message]	15

Annex <X> (informative): Change history	16

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the Medium Access Control (MAC) protocol of Ambient IoT.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[38291]	3GPP TS 38.291: "Ambient IoT Physical layer".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Device: [A-IoT device xxxx]

Reader: [A-IoT reader xxxx]

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-IoT	Ambient IoT

D2R	Device to reader

IoT	Internet of Things

PDRCH	Physical device-to-reader channel

PRDCH	Physical reader-to-device channel

R2D	Reader to device

# 4	General

## 4.1	Introduction

The objective of this clause is to describe the A-IoT MAC architecture and the A-IoT MAC entity of the device from a functional point of view.

## 4.2	A-IoT MAC architecture

Figure 4.2-1 illustrates a model of the A-IoT MAC entity; and it does not restrict implementations.

The A-IoT MAC entity of the device handles the data received or to be transmitted via the physical channels, i.e., PRDCH and PDRCH, as specified in TS 38.291 [38291].

Figure 4.2-1: A-IoT MAC structure overview

## 4.3	Services

### 4.3.1	Services provided to upper layers

The A-IoT MAC layer provides the following services to upper layers:

-	data transfer;

### 4.3.2	Services expected from physical layer

The A-IoT MAC layer expects the following services from the physical layer:

-	data transfer;

## 4.4	Functions

The A-IoT MAC layer supports the following A-IoT MAC functions:

-	construct MAC PDUs onto D2R blocks to be delivered to the physical layer;

-	process MAC PDUs from R2D blocks delivered from the physical layer;

-	message type determination;

-	paging;

-	random access;

-	transfer of upper layer data;

-	D2R segmentation;

-	failure detection;

-	interaction with upper layers.

# 5	A-IoT MAC procedures

## 5.1	General

The objective of this clause is to describe the A-IoT MAC procedures.

When the device is powered on, the device shall monitor the R2D messages via PRDCH, as specified in TS 38.291 [38291], in order to perform the corresponding A-IoT MAC procedures.

## 5.2	A-IoT paging

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include one [Paging ID] to select a specific device or a group of devices, or include no [Paging ID] to select all devices.

The device monitors the A-IoT Paging message, and determines if it is selected and shall initiate the random access procedure.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

1>	if the [Transaction ID] included in the A-IoT Paging message is the same as the value stored by the device if any:

2>	if the previous procedure associated with the received value indicated in the [Transaction ID] is determined as a failure as specified in clause 5.5:

3>	consider the device is selected by this A-IoT Paging message;

1>	else:

2>	store the received value of [Transaction ID] or replace the previously stored the [Transaction ID] with the current received value;

2>	if the A-IoT Paging message includes no [Paging ID]:

3>	consider the device is selected by this A-IoT Paging message and indicate to the upper layers;

2>	else:

3>	forward the [Paging ID] to the upper layers;

3>	if the indication received from the upper layers indicates that the [Paging ID] is matched:

4>	consider the device is selected by this A-IoT Paging message;

1>	if the device is selected by this A-IoT Paging message:

2>	apply [the access related parameters];

2>	initiate the random access procedure as specified in clause 5.3;

## 5.3	A-IoT random access procedure

### 5.3.1	Initialization

Based on the parameters received in the A-IoT Paging message as defined in clause 5.2, the A-IoT MAC entity determines the random access type, i.e., Contention-Based Random Access (CBRA) or Contention-Free Random Access (CFRA). Subsequently, the A-IoT MAC entity performs the actions corresponding to the determined random access type, with the aim of acquiring the radio resources for the D2R upper layer data transmission as specified in clause 5.4.

### 5.3.2	Selection of random access type

If the random access procedure is initiated according to clause 5.2, the A-IoT MAC entity shall:

1>	if [the number of access occasions configured in A-IoT Paging message is larger than one]:

2>	perform Contention-Based Random Access procedure as specified in clause 5.3.3;

1>	else:

2>	perform Contention-Free Random Access procedure as specified in clause 5.3.4;

### 5.3.3	Contention-Based Random Access procedure

#### 5.3.3.1	Selection of the access occasion for the D2R transmission of Random ID message

The A-IoT MAC entity shall:

1>	draw a random integer number 'i' with equal probability in the range: 0 ≤ i < n, where n is the number of access occasions configured in A-IoT Paging message;

1>	select the ‘i’th access occasion for transmission of the Random ID message to perform the transmission of Random ID message, as specified in clause 5.3.3.2, where the access occasion is triggered by [Access Occasion Trigger message, i.e. paging-like message or QueryRep-like message].

#### 5.3.3.2	Transmission of Random ID message

The A-IoT MAC entity shall:

1>	draw a random 16-bit integer number 'j' with equal probability in the range: 0 ≤ j < 216;

1>	set [Random ID] to the ‘j’ in the Random ID message;

1>	process the received [D2R Grant] of the selected access occasion in A-IoT Paging message and indicate it to the physical layer;

1>	instruct the physical layer to transmit the Random ID message using the selected access occasion.

#### 5.3.3.3	Reception of Random ID Response message

Once the Random ID message is transmitted, and if the Random ID Response message is received, the A-IoT MAC entity shall:

1>	if the value indicated in [ACK] in Random ID Response message is identical to the [Random ID] in the transmitted Random ID message:

2>	consider this Random Access procedure is successfully completed;

[2>	FFS AS ID handling;]

2>	process the received [D2R Grant] in Random ID Response message and indicate it to the physical layer;

2>	initiate the D2R message transmission as specified in clause 5.4.1.

### 5.3.4	Contention-Free Random Access procedure

The A-IoT MAC entity shall:

1>	select the access occasion, which is indicated by A-IoT Paging message, for A-IoT upper layer data transmission as specified in clause 5.4.1 and indicate it to the lower layers;

1>	process the received [D2R Grant] of the selected access occasion in A-IoT Paging message and indicate it to the physical layer;

1>	initiate the D2R message transmission as specified in clause 5.4.1.

## 5.4	A-IoT upper layer data transmission

### 5.4.1	D2R message transmission

Once the [D2R Grant] for D2R upper layer data transmission is received in the A-IoT Paging message or the Random ID Response message or the R2D Upper Layer Data Transfer message, the A-IoT MAC entity shall:

1>	if the upper layer data is available to be transmitted:

2>	if the resource size given by the [D2R Grant] is equal or larger than the size of the MAC PDU that contains the entire upper layer data SDU to be transmitted:

3>	generate the D2R Upper Layer Data Transfer message, including:

4>	include the [Data SDU];

4>	set [More Data Indication] to value 0;

3>	instruct the physical layer to transmit the generated D2R Upper Layer Data Transfer message;

2>	else (the resource size given by the [D2R Grant] is smaller than the size of MAC PDU that contains the entire upper layer data SDU to be transmitted):

3>	the upper layer data SDU is to be segmented according to clause 5.4.3;

### 5.4.2	R2D message reception

Once a R2D message is received, the A-IoT MAC entity shall:

1>	upon a R2D Upper Layer Data Transfer message addressed with the device’s AS ID is received:

2>	forward the upper layer data SDU to upper layers;

2>	process the [D2R Grant] and indicate it to the physical layer for the following D2R message transmission, as specified in 5.4.1;

1>	upon a [D2R Scheduling] message addressed with the device’s AS ID is received:

2>	process the [D2R Grant] and indicate it to the physical layer for the following segment transmission, as specified in 5.4.3;

### 5.4.3	Segmentation

When an upper layer data SDU is to be segmented according to D2R upper layer data transmission procedure in clause 5.4.1, the A-IoT MAC entity performs this segmentation procedure for the original upper layer data SDU.

The A-IoT MAC entity shall:

1>	if the [D2R Grant] is received in the A-IoT Paging message or the Random ID Response message or the R2D Upper Layer Data Transfer message (i.e., the first segment):

2>	generate the D2R Upper Layer Data Transfer message for this segment according to the [D2R Grant], including:

3>	consider the segment starts from the first byte of the original upper layer data SDU;

3>	include the [Data SDU];

3>	set [More Data Indication] to value 1;

2>	instruct the physical layer to transmit the above generated D2R Upper Layer Data Transfer message.

1>	else if [D2R Grant] is received in the [D2R Scheduling] message (i.e., not the first segment):

2>	generate the D2R Upper Layer Data Transfer message for this segment according to the [D2R Grant], including:

3>	consider that the segment starts from the (x+1)th byte, indicated by the [Received Data Indicator, i.e., received x bytes], of the original upper layer data SDU;

3>	include the [Data SDU];

3>	if the segment is the last segment of the original upper layer data SDU:

4>	set [More Data Indication] to value 0;

3>	else:

4>	set [More Data Indication] to value 1;

2>	instruct the physical layer to transmit the above generated D2R Upper Layer Data Transfer message.

## 5.5	Failure detection

The A-IoT MAC entity shall:

1>	if CBRA procedure fails [(i.e., the device does not successfully complete the CBRA procedure before reception of the next A-IoT Paging message)]; or

1>	if the [Failure Indication] message including the AS ID of the device is received:

2>	consider that the current procedure associated with the stored [Transaction ID] failed.

# 6	Protocol Data Units, formats and parameters

## 6.1	Protocol Data Units

### 6.1.1	General

An A-IoT MAC Protocol Data Unit (PDU) is the data unit format in which the A-IoT MAC message is encapsulated for transmission through the lower layer of the A-IoT protocol stack. An A-IoT MAC PDU is a bit string. The contents of each A-IoT MAC message are specified in clause 6.2 using tables to specify the fields in the message. In the tables, the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within a MAC PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

An A-IoT MAC SDU is a bit string [that is byte aligned (i.e., multiple of 8 bits) in length]. A MAC SDU is included into a MAC PDU from the first bit onward.

[The A-IoT MAC entity shall ignore the value of the Reserved bits in R2D MAC PDUs.]

The R2D message type represents the set of A-IoT MAC messages that are sent from the reader to the device on the PRDCH. The value of R2D message type is specified in Table 6.1-1.

Table 6.1-1: R2D Message Type

| R2D Message Type value | R2D message name |
| --- | --- |
| 001 | A-IoT Paging message |
| 002 | [FFS Access Occasion Trigger message (i.e., Query-like message)] |
| 003 | Random ID Response message |
| 004 | R2D Upper Layer Data Transfer message |
| 005 | D2R Scheduling message |
| 006 | Failure Indication message |
| 007 | Reserved |
| 000 | Reserved |

The D2R message type is the set of A-IoT MAC messages that sent from the device to the reader on the PDRCH. The value of D2R message type is specified in Table 6.1-2.

Table 6.1-2: D2R Message type

| D2R Message Type value | D2R message name |
| --- | --- |
| NA | Random ID message |
| 01 | D2R Upper Layer Data Transfer message |
| 02 | Reserved |
| 03 | Reserved |
| 00 | Reserved |

## 6.2	A-IoT MAC messages

### 6.2.1	A-IoT Paging message

Figure 6.2.1-1 shows the format of the A-IoT Paging message.

The definition of each field:

-	R2D Message Type: xxx

-	Paging ID: xxx

-	Transaction ID: xxx

-	Number of access occasions: xxx

[Figure 6.2.1-1 is to be updated]

### 6.2.2	Random ID message (Msg1)

Figure 6.2.2-1 shows the format of the Random ID message.

The definition of each field:

-	Random ID: xxx

[Figure 6.2.2-1 is to be updated]

### 6.2.3	Random ID Response message (Msg2)

Figure 6.2.3-1 shows the format of the Random ID Response message.

The definition of each field:

-	R2D Message Type: xxx

-	ACK: xxx

[Figure 6.2.3-1 is to be updated]

### 6.2.4	D2R Data Transfer messages

#### 6.2.4.1	D2R Upper Layer Data Transfer message

Figure 6.2.4.1-1 shows the format of the D2R Upper Layer Data Transfer message.

The definition of each field:

-	[D2R Message Type: xxx ]

-	[More Data Indication]: xxx

-	Data SDU: xxx

-	[MAC padding:]

[Figure 6.2.4.1-1 is to be updated]

### 6.2.5	R2D Data Transfer messages

#### 6.2.5.1	R2D Upper Layer Data Transfer message

Figure 6.2.5.1-1 shows the format of the R2D Upper Layer Data Transfer message.

The definition of each field:

-	R2D Message Type: xxx

-	Data SDU: xxx

-	[D2R Grant]: xxx

[Figure 6.2.5.1-1 is to be updated]

#### 6.2.5.2	[D2R Scheduling message]

Figure 6.2.5.2-1 shows the format of the D2R Scheduling message.

The definition of each field:

-	R2D Message Type: xxx

-	Received Data Indicator: xxx

-	[D2R Grant]: xxx

[Figure 6.2.5.2-1 is to be updated]

### 6.2.6	[Failure Indication message]

Figure 6.2.6-1 shows the format of the Failure Indication message.

The definition of each field:

-	R2D Message Type: xxx

[-	AS ID: xxx]

[Figure 6.2.6-1 is to be updated]

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |