3GPP TSG-RAN WG2 Meeting #129bis	 R2-2502261

Wuhan, China, April 7-11, 2025

Source:	KDDI Corporation

Title:	Discussion on On-demand SIB1

Tdoc Type:	Discussion

Agenda Item:	8.5.3

Document for:  Discussion

Release:       Rel-19

1	Introduction

In RAN2#127b and RAN2#129, the following agreements were made.

Agreements on OD-SIB1@RAN2#127b

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Agreements on OD-SIB1@RAN2#129

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we will provide our further consideration on on-demand SIB1.

2	Discussion

2.1 whether SIB-x in cell A can be set “broadcasting/not-broadcasting”

It is agreed in RAN2 that the WUS configuration for OD-SIB1 to be included in SIB-x, a new system information block.

In legacy system, except SSB and SIB1, other system information can be configured as “not-broadcasting” and only transmitted upon UE request via SI-Request. As the new SIB-x,which convey the WUS configuration to the UE, can be considered as other system information, we are wondering if SIB-x follows the legacy behaviour. If the SIB-x is always broadcasted, UE can obtain the WUS configuration immediately, ensuring quick access to OD-SIB1, while it will cause extra energy consumption of Cell A. If the SIB-x can be configured as “not-broadcasting”, Cell A needs to indicate the list of WUS configurations it can provide within its SIB1 and the UEs need to request it explicitly via legacy SI-request, which introducing additional signalling overhead.

Proposal 1: RAN2 clarify whether the new SIB-x is mandatory broadcast or can follow the legacy behaviour and be configured as “broadcasting/not-broadcasting”.

2.2 NES cell SIB1 monitoring

In RAN2 #127b, it was agreed that the UE must check whether SIB1 is currently being broadcast or provided on demand for a given cell before requesting SIB1, even if the UE has the SIB1 request configuration for that cell. However, the method by which the UE checks the SIB1 status is unclear and is still under discussion in RAN1. Therefore, RAN2 can wait for RAN1 to determine the UE's behavior for monitoring SIB1 before it sends the WUS signal to the NES cell.

Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following: - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission. - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy. Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements Note: The above cases are for SSB on the sync raster.

Proposal2: RAN2 waits for RAN1's decision on how the UE monitors SIB1 before requesting SIB1 from the NES cell.

2.3 Automatic Neighbour Relation

gNB manages the NCR(neighbor cell relation) information to facilitate handover (HO) between gNBs. There are two main methods for creating the NCR: one is manual creation by the operator, and the other is automatic creation through ANR (Automatic Neighbor Relations).

For the automatic creation of NCR through ANR, UE needs to obtain the NCGI information of the target cell and report it to the source cell. In the on-demand SIB1 scenario, there is no periodic transmission of SIB1 from NES cell, so UE cannot acquire SIB1 for NES cell.

As a result, ANR cannot function properly, leading to the issue of being unable to automatically create NCR. Manually creating all NCRs would significantly increase operational costs, so it is necessary to consider the issues in the on-demand SIB1 scenario.

Proposal3: RAN2 discuss ANR issue in the on-demand SIB1 scenario.

To achieve the coexistence of on-demand SIB1 and ANR, it is necessary to discuss specific methods. For example, One approach is that when gNB detects ANR activity, it could coordinate with other gNB or higher-level network equipment to stop the SIB1-less mode of NES cells through network interface and have NES Cell transmit SIB1 periodically at all times. Another approach could be for UE to obtain SIB1 information of the NES cell in advance before executing ANR operations. In the latter case, if the UE is to acquire the necessary SIB1 information for ANR in advance, it must first receive and store the required information internally, which would lead to a significant burden on UE's implementation. Therefore, it is considered more straightforward to first consider solution like stopping the SIB1-less mode on the network side in RAN3, rather than considering solution in RAN2.

Proposal 4: If RAN2 discuss the coexistence of ANR and on-demand SIB1, RAN2 agree that the solution should be considered within RAN3

2.4 Contention between paging and OD-SIB1 process

In OD-SIB1 process, the acquisition of SIB1 is initiated by sending UL WUS to the NES cell. During this process, CellA may send paging to UE. For example, if the timing of receiving paging overlaps with the timing of receiving RAR, SIB, or the timing of sending UL WUS, there is a possibility that paging could be lost. Since this issue of contention between the on-demand SIB1 acquisition process and paging has been raised in RAN1[1], RAN2 proposes to wait for RAN1's discussion.

Issue 10-F: UL-WUS on NES cell overlapping with paging occasion on normal cell MTK Proposal 5: A potential issue of paging dropping may arise if UL-WUS on NES cell overlapped with paging occasion on Cell A as shown in Figure 1. RAN1 can further discuss how to solve this issue.

Proposal 5: RAN2 wait for RAN1's discussion on the contention between paging and OD-SIB1 process.

3	Conclusion

We make the following observations and proposals:

Proposal 1: RAN2 clarify whether the new SIB-x is mandatory broadcast or can follow the legacy behaviour and be configured as “broadcasting/not-broadcasting”.

Proposal 2: RAN2 waits for RAN1's decision on how the UE monitors SIB1 before requesting SIB1 from the NES cell.

Proposal 3: RAN2 discuss ANR issue in the on-demand SIB1 scenario.

Proposal 4: If RAN2 discuss the coexistence of ANR and on-demand SIB1, RAN2 agree that the solution should be considered within RAN3

Proposal 5: RAN2 wait for RAN1's discussion on the contention between paging and OD-SIB1 process.

3	　Reference

[1]　 R1-2501379	FL summary 2 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)