3GPP TSG-RAN WG2 Meeting #129bis                                               	 R2-2502279

Wuhan, China, April 7th – 11st, 2025

Agenda Item:	8.11.1 Organizational

Source:	NEC

Title:	TS38.304 impacts on supporting Rel-19 SBFD

Document for:	Discussion, Decision

Introduction

During RAN2#127bis meeting, following Spec editor assignment suggested by WI Rapporteur was agreed:

38.300 => CATT

38.321 => Samsung

38.331 => Huawei

38.306 => Ericsson

38.304 => NEC

This contribution discusses the TS38.304 impacts on supporting Rel-19 SBFD.

Discussion

TS38.304 specifies the user equipment (UE) procedures in Idle mode and RRC Inactive state. It consists of following clauses:

4. General description of RRC_IDLE state and RRC_INACTIVE state

5. Process and procedure descriptions

5.1	PLMN selection and SNPN selection

5.2	Cell selection and reselection

5.3	Cell Reservations and Access Restrictions

5.4	Tracking Area registration

5.5	RAN Area registration

6	Reception of broadcast information

7	Paging

8	Sidelink Operation

9	Tracking Reference Signal

- After carefully checking the agreements achieved from RAN2#127 to RAN2#129 meeting (summarized in APPENDIX), we observed that there are no TS38.304 impacts on supporting Rel-19 SBFD.

After carefully checking the agreements achieved from RAN2#127 to RAN2#129 meeting (summarized in APPENDIX), we observed that there are no TS38.304 impacts on supporting Rel-19 SBFD.

RAN2 confirms that there are no TS38.304 impacts on supporting Rel-19 SBFD, i.e., no TS38.304 running CR is needed.

Conclusion

This contribution discussed the TS38.304 impacts on supporting Rel-19 SBFD.

- After carefully checking the agreements achieved from RAN2#127 to RAN2#129 meeting (summarized in APPENDIX), we observed that there are no TS38.304 impacts on supporting Rel-19 SBFD.

Proposal 1 RAN2 confirms that there are no TS38.304 impacts on supporting Rel-19 SBFD, i.e., no TS38.304 running CR is needed.

- References

[1] R2-2407572 Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

[2] R2-2409212	Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD

[3] R2-2410912	Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD

[4] R2-2501332	Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD

APPENDIX

- Agreements at RAN2#127

Random access in SBFD

·	Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

·	RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

·	RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

·	FFS whether/how early indication is used during a SBFD RA procedure.

·	RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

Other aspects

·	Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress

- Agreements at RAN2#127bis

Random access in SBFD

·	RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

·	From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

·	Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

·	For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

·	The following two RACH configuration options are considered for SBFD based random access:

·	Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

·	Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

·	For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

·	The RACH configuration for SBFD is transmitted via SIB1.

·	FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

Other aspects

·	For UL resource muting for PUSCH, the configuration of time and frequency location for UL resource muting should be introduced based on R1 agreement.

·	For L1 based UE-to-UE CLI measurement mechanism, the configuration of periodic, semi-persistent or aperiodic UE-to-UE CLI measurement resource (set) should be introduced based on R1 agreement.

·	For L1 based UE-to-UE CLI reporting mechanism, the configuration of report quantities should be introduced based on R1 agreement.

- Agreements at RAN2#128

Random access in SBFD

·	CFRA

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

·	CBRA

FFS on the following options

·	Option 1

Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the

prioritization of the additional ROs over legacy RO.

If there is no such indication from the NW, FFS on the following mechanism

-	UE select legacy RO or SBFD RO based on SSB RSRP, or

-	UE select the legacy RO, or

-	UE select the SBFD RO, or

-	Other metrics than SSB RSRP.

·	Option 2

UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

·	RACH configuration

Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Other aspects

Prioritization of SBFD cells / frequencies during cell reselection is not considered.

RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

- Agreements at RAN2#129

Random access in SBFD

·	When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell.

·	When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration.

·	For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

·	If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

·	FFS whether RO type selection is performed before or after the RA type selection.

·	FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

Other aspects

·	A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.