3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502283

Wuhan, China, Apr 7th – Apr 11th, 2025

Agenda item:	8.3.5

Source:	Qualcomm Incorporated

Title:	LCM and specifications impact for AI/ML for mobility

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #129 meeting [1], the focus was on drawing conclusions from the simulation results provided by companies on RRM measurement prediction, Model generalization for RRM measurement prediction, and Measurement Event prediction and System Level Simulations (SLS). Many agreements were made on the observations and conclusions from the simulation studies; see the Chair notes [1]. It was also agreed that in the upcoming meeting RAN2 #129-bis, the focus will be on the simulation results for Model Generalization for RRM measurement prediction and Measurement Event prediction and SLS, and on a study of the specification impacts for AI/ML for mobility.

### 8.3.5     LCM and spec impact for AI/ML mobility

Contributions should focus on the LCM framework/signaling for RRM measurement prediction and measurement event prediction, and the gap from the currently agreed LCM framework for the beam management use case under the AI/ML for NR air interface work item.

Contributions can address required signaling and protocol to enable reporting of inference outcome for RRM measurement prediction and measurement event prediction for a UE sided model, or the required signaling and protocol aspects to enable reporting of UE assistance for inference operation of a network sided model for RRM measurement prediction.

In the agenda for the upcoming meeting, the Chairlady provided the following scope for the study on specification impacts.

Accordingly, in this contribution, we discuss the following:

- LCM for AI/ML for mobility,

- Reporting RRM measurement prediction and Measurement Event prediction for a UE-side model.

- Reporting UE assistance and network assistance for inference operation of a network-side and a UE-side model for RRM measurement prediction, respectively.

In the discussion on LCM for AI/ML for mobility, we leverage the developments in the LCM framework for the Rel-19 AI/ML for Beam Management (BM) prediction use case.

# LCM gap analysis for AI/ML for mobility

Over the past two years, RAN1/RAN2 have worked on the AI/ML for NR air interface study and work item. The most advanced discussions have centred on the beam management prediction feature. This section discusses what beam management prediction agreements and concepts can be adopted for the AI/ML for mobility study.

The following aspects were studied for AI/ML for the Air Interface in Rel-18 and Rel-19, and should be studied for AI/ML for mobility as well:

-	Data collection,

-	Model training,

-	Model delivery/transfer,

-	Model inference operation,

-	Functionality selection, activation, deactivation, switching, and fallback operation,

-	Model monitoring, and

-	UE capabilities.

## Data consistency

RAN2 #126 [2] agreement:

For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.

During the Rel-19 AI/ML for BM prediction discussion, the issue of network-side additional conditions was discussed and in the RAN2 #126 meeting [2], an agreement was made on the signaling of such conditions for consistency of inference and data collection/training configurations provided by the network to the UE, for UE-side models.

A similar situation can happen with prediction at the cell level. Indeed, an operator’s network can change with time. Cells are split. Frequencies are added or removed. Beams (precoding weights) are reconfigured. Transmission power, antennae tilt, and antennae direction can be adjusted. Even the inputs (what to measure) and outputs (what to predict, which cells and which events) may change with time. As a result, models trained in past environments may not perform as expected in new environments, if enough consistency is not maintained.

We conclude that, for AI/ML for mobility, for UE-side models, the network needs to ensure consistency between the environment and configurations used at the data collection phase and those that will be used when the feature is enabled, configured and activated.

Proposal 1. For UE-side models for AI/ML for mobility, there should be consistency between the inference and data collection/training configurations provided by the network. The exact solution can be discussed during the WI phase.

## Supported, Applicable and Activated functionalities

In Rel-19 AI/ML for BM prediction (RAN2 #127 [3]), the following terminologies were introduced to facilitate the discussion: supported functionalities, applicable functionalities, and activated functionalities. These terminologies can be adopted for AI/ML for mobility, as well as their definitions.

Proposal 2. The below definitions on supported functionalities, applicable functionalities, and activated functionalities, from Rel-19 AI/ML for BM prediction, can be adopted for the AI/ML for mobility discussion.

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information.

- Applicable functionalities refer to functionalities that the UE is ready to activate.

- Activated functionalities refer to functionalities enabled for performing inference.

## Signaling flow

In RAN2 #127 [3], the signaling flow for inference configuration for the UE-side model for BM prediction was agreed; see Figure 1. This signaling flow can be taken to be the baseline for AI/ML for mobility.

Proposal 3. Based on agreements for the beam management prediction part of the AI/ML for NR air interface work item, this is a proposed list of specification impacts to implement AI/ML for mobility for both RRM measurement prediction and Measurement Event prediction for UE-side models. The work item phase can finalize and refine these proposals:

- In the UE capabilities, the UE lists its supported functionalities, e.g., whether the UE supports AI/ML models for RRM measurement prediction and/or for Measurement Event prediction.

- To enable consistent UE-side Data collection, the UE can request a configuration from the network. Details of data collection needed for Measurement Event prediction and RRM measurement prediction can be discussed in the WI phase.

- The network provides the UE with a feature configuration ensuring consistency between the data collection and feature execution phases.

- The network signals indication for activation/deactivation for an AI/ML for mobility functionality at the UE.

- The signaling flow for inference configuration (see Figure 1) for Rel-19 AI/ML for BM prediction can be adopted as the baseline.

- The three types of performance monitoring for the Rel-19 AI/ML for BM prediction – UE-side performance monitoring, NW-side performance monitoring, and UE-assisted performance monitoring – may be considered for AI/ML for mobility.

Figure 1: Signalling flow for inference configuration for UE-side AI/ML model for BM prediction

For network-side AI/ML model for RRM measurement prediction, network configures the UE to provide measurement reports.

Proposal 4. The following is a specification impact for Network-side model for AI/ML for mobility:

- For network-side data collection and inference, the network configures the UE to provide reports of cell and beam measurements. Details can be discussed during the WI phase.

# Reporting of RRM measurement prediction, Measurement Event prediction, UE assistance, and network assistance

For a UE-side model, the network configures the UE for performing Cell-level and L3 Beam-level RRM measurement prediction and reporting. The network provides the configuration for temporal domain prediction, frequency domain prediction, and spatial domain prediction. For Cell-level RRM prediction, spatial domain prediction is the case where the UE predicts the measurements for a serving cell or a candidate cell by measuring a subset of configured SSBs, or for a candidate cell (on which measurements are not performed by the UE) based on measurements of other cells. For L3 Beam-level RRM prediction, spatial domain prediction is the case where the UE predicts the L3 Beam-level measurements for one or more beams of a serving cell or a candidate cell (on which measurements are not performed by the UE) based on measurements of other beams on the same cell or other cells.

Proposal 5. For a UE-side model for RRM measurement prediction, the network configures the UE to report Cell-level and L3 Beam-level measurement predictions. The detailed signaling, protocol, and contents of the reports, can be discussed during the WI phase.

Proposal 6. For a UE-side model for Measurement Event prediction, the network configures the UE to report Measurement Event predictions. The detailed signaling, protocol, and contents of the reports, can be discussed during the WI phase.

Regarding reporting of UE assistance for inference operation of network-side model for RRM measurement prediction, our understanding is that this refers to UE location or UE speed reporting.

RAN2 #129 [1] agreements on generalization over UE speed:

1 For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across all UE speeds.

2 Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size).

3 For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline.

The simulation results for the study on Generalization over UE speeds for RRM measurement prediction showed that the models generalize well for intra-frequency, temporal domain prediction in FR1/FR2. So, there is no need to provide reports of UE speed.

Regarding reporting UE location, the benefits of providing UE location have not been reported by most companies in their simulation results for RRM measurement prediction or for Measurement Event prediction. Consequently, we see no need to provide reports of UE location.

Proposal 7. For supporting inference of network-side model for RRM measurement prediction or Measurement Event prediction, the UE does not provide reports of UE location or UE speed, as UE assistance information.

For UE-side RRM measurement prediction, it is essential for the UE to know the cells that are activated or deactivated. For cells that are deactivated, UE does not need to perform measurements or prediction on them for data collection/training or inference. The network should keep the UE updated about the activation/deactivation status of cells that are configured for the UE-side model of AI/ML for mobility.

Proposal 8. For the UE-side model for RRM measurement prediction, the network provides the activation/deactivation status of cells configured to be measured or predicted.

# 4 Conclusions

Based on the above discussions, we recommend that RAN2 discuss the following proposals.

Proposal 1. For UE-side models for AI/ML for mobility, there should be consistency between the inference and data collection/training configurations provided by the network. The exact solution can be discussed during the WI phase.

Proposal 2. The below definitions on supported functionalities, applicable functionalities, and activated functionalities, from Rel-19 AI/ML for BM prediction, can be adopted for the AI/ML for mobility discussion.

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information.

- Applicable functionalities refer to functionalities that the UE is ready to activate.

- Activated functionalities refer to functionalities enabled for performing inference.

Proposal 3. Based on agreements for the beam management prediction part of the AI/ML for NR air interface work item, this is a proposed list of specification impacts to implement AI/ML for mobility for both RRM measurement prediction and Measurement Event prediction for UE-side models. The work item phase can finalize and refine these proposals:

- In the UE capabilities, the UE lists its supported functionalities, e.g., whether the UE supports AI/ML models for RRM measurement prediction and/or for Measurement Event prediction.

- To enable consistent UE-side Data collection, the UE can request a configuration from the network. Details of data collection needed for Measurement Event prediction and RRM measurement prediction can be discussed in the WI phase.

- The network provides the UE with a feature configuration ensuring consistency between the data collection and feature execution phases.

- The network signals indication for activation/deactivation for an AI/ML for mobility functionality at the UE.

- The signaling flow for inference configuration (see Figure 1) for Rel-19 AI/ML for BM prediction can be adopted as the baseline.

- The three types of performance monitoring for the Rel-19 AI/ML for BM prediction – UE-side performance monitoring, NW-side performance monitoring, and UE-assisted performance monitoring – may be considered for AI/ML for mobility.

Proposal 4. The following is a specification impact for Network-side model for AI/ML for mobility:

- For network-side data collection and inference, the network configures the UE to provide reports of cell and beam measurements. Details can be discussed during the WI phase.

Proposal 5. For a UE-side model for RRM measurement prediction, the network configures the UE to report Cell-level and L3 Beam-level measurement predictions. The detailed signaling, protocol, and contents of the reports, can be discussed during the WI phase.

Proposal 6. For a UE-side model for Measurement Event prediction, the network configures the UE to report Measurement Event predictions. The detailed signaling, protocol, and contents of the reports, can be discussed during the WI phase.

Proposal 7. For supporting inference of network-side model for RRM measurement prediction or Measurement Event prediction, the UE does not provide reports of UE location or UE speed, as UE assistance information.

Proposal 8. For the UE-side model for RRM measurement prediction, the network provides the activation/deactivation status of cells configured to be measured or predicted.

# References

[1] Chair notes, RAN2 #129 meeting.

[2] Chair notes, RAN2 #126 meeting.

[3] Chair notes, RAN2 #127 meeting.