3GPP TSG-RAN WG2 Meeting #129bis	R2-2502285

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.6.2

Source: 	Fujitsu

Title:	Discussion on inter-CU LTM

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to provide our considerations on open issues for intra-CU and inter-CU LTM.

- Discussions

  - Intra-CU LTM with bearer type change

In RAN2#129 meeting, L2 reset operation for both inter-CU LTM and intra-CU LTM with bearer type change was discussed [1].

Proposal 8: For MCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB with termination point change:

- The UE performs PDCP re-establishment.

- For the associated MCG bearer that maintains unchanged if any, the UE performs RLC re-establishment.

Proposal 9: For MCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB without termination point change:

- For the associated MCG bearer that maintains unchanged if any, UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- If it is AM DRB, the UE performs PDCP data recovery when there is RLC bearer released/re-established.

Proposal 10: For SCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB with termination point change:

- The UE performs PDCP re-establishment.

- If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, RLC re-establishment is required for the associated RLC bearer that maintains unchanged if any.

- If there is no bearer type change, the UE performs re-establishment for the associated RLC bearer(s).

Proposal 11: For SCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB without termination point change:

- If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, the UE compares the ltm-ServingCellNoResetID and the ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18 for the associated SCG bearer that maintains unchanged if any.

- If there is no bearer type change, the UE compares the ltm-ServingCellNoResetID and the ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18 for the associated SCG bearer if any.

- If it is AM DRB, the UE performs PDCP data recovery when there is RLC bearer released/re-established.

Will be discussed as part of RRC running CR preparation (including what is UE intended behaviours for different cases and whether/how to capture them into spec).

As summarised above, support of PDCP termination point change and RLC bearer type change is required as a prerequisite, however it is not yet concluded whether they are supported for intra-CU LTM so far. RAN2 already supports bearer type change during inter-CU LTM and SCPAC in Rel-18, then it can be applied to intra-CU LTM as well because there is no reason to limit it in inter-CU LTM only. We propose to make it clear that both PDCP termination point change and RLC bearer type change are supported during intra-CU LTM in Rel-19.

Proposal 1: RAN2 to confirm PDCP termination point change and RLC bearer type change during intra-CU LTM are supported in Rel-19.

  - Fast LTM recovery for inter-CU LTM

In RAN2#127bis meeting and RAN2#128 meeting, RAN2 agreed to postpone the discussion on fast LTM recovery involving inter-CU LTM [2][3].

- Agreement in RAN2#127bis meeting

Proposal 3 (modified): The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

Agreed.

- Agreement in RAN2#128 meeting

RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

For security aspects, RAN2 has received reply LS from SA3 on security handling for inter-CU LTM in non-DC cases [4], so RAN2 can resume the discussion.

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.

According to this SA3’s decision, the following observations are derived.

Observation 1: NCC is transmitted via MAC CE.

Observation 2: The UE cannot find out the NCC before it receives the MAC CE.

RAN2 should consider all cases of fast LTM recovery involving inter-CU LTM listed below.

- Fast LTM recovery to an inter-CU cell after RLF

- Fast LTM recovery to an inter-CU cell after L3 HO/CHO failure

- Fast LTM recovery to an inter-CU cell after intra-CU LTM failure

- Fast LTM recovery to an inter-CU cell after inter-CU LTM failure

- Fast LTM recovery to an intra-CU cell after inter-CU LTM failure

First, cases (1) – (4), in which a cell selected during RRC re-establishment procedure for fast LTM recovery is an inter-CU LTM candidate cell are discussed. After LTM cell switch failure, RLF, or L3 HO/CHO failure, the UE performs cell selection and if the UE is configured with fast LTM recovery and the selected cell is one of the LTM candidate cell(s), the UE performs LTM cell switch to the selected cell. In this case, if the selected cell belongs to a different CU from the serving cell, NCC is needed to perform cell switch.

As the NW does not know which cell is selected for fast LTM recovery, the NW cannot send the MAC CE containing NCC to the UE. Considering this together with observation 2, fast LTM recovery to an inter-CU cell is not feasible in practical. In addition, fast LTM recovery after L3 HO/CHO is already not supported in Rel-18, which was agreed in RAN2#125bis meeting [5].

- Agreement in RAN2#125bis meeting

LTM recovery is not supported for T304 expiry after HO (L3 HO) and CHO, but supported for T304 expiry after LTM cell switch (and RLF as earlier agreed). This agreement supersedes earlier agreements.

Therefore, fast LTM recovery should not be supported if the cell selected during RRC re-establishment procedure is one of the inter-CU LTM candidate cell(s).

Proposal 2: RAN2 to not support fast LTM recovery if the cell selected during RRC re-establishment procedure is one of the inter-CU LTM candidate cell(s).

Second, case (5) fast LTM recovery to an intra-CU cell after inter-CU LTM failure is discussed. When the UE performed an inter-CU LTM but it failed, PDCP entities and RLC entities are both re-established. PDCP re-establishment and RLC re-establishment include the actions which cannot be reverted, e.g., reset of EHC/ROHC protocol, discard of all RLC SDUs, SDU segments and PDUs, and reset of all state variables in RLC entities. This is pointed out in [6] for fast LTM recovery after L3 HO or CHO failure.

Observation In [6]

Observation 2: HO/CHO execution may incur PDCP and RLC re-establishment, which prohibits UE from reverting back to configuration and L2 state used in the source cell before HO/CHO attempt.

With the reasons, fast LTM recovery after L3 HO or CHO was not agreed to support in RAN2#125bis meeting as mentioned above [5].

Given that, case (5) fast LTM recovery to an intra-CU cell after inter-CU LTM failure is not possible because the UE cannot revert the configuration back to that of the source PCell. Thus, the same principle as fast LTM recovery after L3 HO or CHO should be applied, i.e., RAN2 should also not support case (5). Besides, case (4) should not be supported as mentioned above, consequently fast LTM recovery after inter-CU LTM failure should not be supported regardless of whether the selected cell is an intra-CU cell or an inter-CU cell.

Proposal 3: RAN2 to not support fast LTM recovery after inter-CU LTM failure.

- Conclusion

We have the following observations and proposals.

Observation 1: NCC is transmitted via MAC CE.

Observation 2: The UE cannot find out the NCC before it receives the MAC CE.

Proposal 1: RAN2 to confirm PDCP termination point change and RLC bearer type change during intra-CU LTM are supported in Rel-19.

Proposal 2: RAN2 to not support fast LTM recovery if the cell selected during RRC re-establishment procedure is one of the inter-CU LTM candidate cell(s).

Proposal 3: RAN2 to not support fast LTM recovery after inter-CU LTM failure.

# References

[1] R2-2501331, “Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung)

[2] R2-2409211, “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung)

[3] R2-2410911, “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung)

[4] R2-2501756, “Reply LS on security handling for inter-CU LTM in non-DC cases”, SA3

[5] R2-2403735, “Report from session on Mobility Enh and Mobile IAB”, Session Chair (MediaTek Johan)

[6] R2-2402235, “Fast Recovery with LTM Candidates”, MediaTek Inc.