3GPP TSG-RAN WG2 #129bis	R2-2502288

Wuhan, China, April 7th – 11th, 2025

Source:		NEC

Agenda item:		8.10.2.1 LTM

Title:	MRO enhancement for LTM

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, for MRO of LTM, RAN3 agreed to send LS to RAN3 to asks them to consider whether to specify NW based solution for the scenario of beam failure recovery shortly after successful LTM [1]:

Agreements Send an LS to RAN3 indicating scenario of beam failure recovery shortly after successful LTM (“near failure”), also say that we will not specify a UE based solution for this, but that RAN3 can consider a NW based solution if they want. RAN3 can see if/how the solution (if specified) could also be used for the case of LTM failure.

In this paper, we provide our further consideration on MRO for LTM.

- Discussion

For RACH-less LTM, both dynamic grant and configured grant can be used. And the UE shall apply the TCI state indicated in the LTM cell switch command for dynamic grant reception or configured grant resource selection.

Bad TCI indication may result in LTM cell switch failure, to analysis whether the failure of RACH-less LTM is due to improper TCI state indication, it is beneficial to record the TCI state information for the failed LTM cell switch. A network-based solution (e.g., the source DU stores the TCI state information for all cell switch executions) for all UEs would bring big burden to the network device. For this reason, a network based solution is not a good way. Therefore, we propose to include the TCI state information in the RLF-report associated with the failed LTM cell switch procedure.

Proposal 1: Include in the RLF-report the TCI state information included in the LTM cell switch command for the LTM cell switch failure.

For RACH-based LTM, both CFRA and CBRA can be performed. And CFRA can be based on either CFRA configuration in RRC configuration, or CFRA resource in LTM cell switch command. During RAN2 #125bis meeting, it was agreed that RAN2 considers RA report for MCG-LTM. However, it is not clear yet, whether the RA-report can be supported for a RA procedure for early sync purpose.

To support RACH-less based LTM, early synchronization can be performed, during which the UE can be triggered by PDCCH order from the network to perform RA to the LTM candidate cell. More specifically, UE would transmit preamble to the LTM candidate cell, and the RA procedure is considered as completed. No RAR would be received by the UE, instead, the TA information would be transmitted from the candidate DU to the source DU via CU, and the TA value is sent together in the LTM cell switch command to the UE by the source DU. The UE is not aware of whether the RA procedure for early UL sync is successful or not, and the retransmission of the preamble is indicated by the source cell. The RA resource is allocated per candidate cell per gNB-DU by the candidate DU, so it is beneficial to record RA information for better usage of RA resource for early sync, however, whether there is the worth to work on the related function, and whether this is done by the UE or network needs further discussion.

Observation: The UE is not aware of whether the random access for early UL sync is successful or not, and cannot provide more information than the NW could.

Proposal 2: Support a solution for RA optimization of early UL sync for LTM.

Proposal 3: Discusses possible solutions including network-based solution and UE-based solution for RA optimization of early UL sync for LTM.

# 3. Conclusion

In this paper, we discuss on further consideration on MRO for LTM, and have the following observation and proposals:

Proposal 1: Include in the RLF-report the TCI state information included in the LTM cell switch command for the LTM cell switch failure.

Observation: The UE is not aware of whether the random access for early UL sync is successful or not, and cannot provide more information than the NW could.

Proposal 2: Support a solution for RA optimization of early UL sync for LTM.

Proposal 3: Discusses possible solutions including network-based solution and UE-based solution for RA optimization of early UL sync for LTM.

# Reference

[1] Meeting report of RAN2 #129