3GPP TSG-RAN WG2 Meeting #129bis		R2-2502289

Wuhan, China, Apri 7th – 11st, 2025

Source:	Baicells

Title:		Discussion on Conditional intra-CU LTM remaining issues

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

The following agreements to support the Conditional intra-CU LTM are made in the RAN2#128[1] and RAN2#129[2]:

RAN2#128 Agreements on C-LTM: - The triggering condition of conditional LTM can be based on L3 measurement. - CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. - The L1 execution condition of a candidate cell is associated to only one triggering event. - For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. - To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. - The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. - For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer. - For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

RAN2#129 Agreements on C-LTM: 1. From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. 2. The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. 3. For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. 7. Do not support DG-based first UL transmission for RACH-less conditional LTM. 8. For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed. 9. For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions. 10. Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA)

In this contribution, we will discuss the remaining open issues and further details of conditional LTM, including:

- The details for configuring the L1 execution conditions

- How to handle the TA timers upon CLTM cell switch

- Which beam to perform the CLTM cell switch for L3 based C-LTM

- Discussion

  - CLTM preparation

L1 executions

It has been agreed that it is DU to generate the L1 execution condition in RAN2#127bis. In R18 intra-CU LTM, DU has already generated the LTM-CSI-ReportConfig for the LTM. Thus in R19 intra-CU CLTM, DU can directly configure the L1 execution condition in the LTM-CSI-ReportConfig too. In this way, the UE can find the L1 measurement report configuration for intra-CU LTM and the L1 execution condition for intra-CU CLTM at the same time without decoding another IE.

What’s more, in RAN2#129, the following agreements are achieved for L1 event triggered measurement reporting:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

Also, in RAN2#129, the following agreements is achieved for CLTM:

- Re-use existing L1 events for CLTM.

From the above agreements, we can see that the L1 events will be configured in the LTM-CSI-ReportConfig IE. To re-used existing L1 events for CLTM, it’s simple to configure the L1 execution along with the L1 events in LTM-CSI-ReportConfig.

Observation1: In R18 LTM, DU has already generated the LTM-CSI-ReportConfig for the LTM.

Observation2: In R19 LTM, for measurement resource configuration, R18 LTM CSI resource configuration is reused.

Observation3: In R19 LTM, for measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused.

Observation4: Re-use existing L1 events for CLTM.

Proposal1: DU can configure the L1 execution condition in the LTM-CSI-ReportConfig.

  - Conditional LTM Execution

How to handle the TA timers upon CLTM cell switch

There are two kinds of TA timers to be handled upon the CLTM cell switch. One kind is the TA timer of the target cell and another kind is the TA timer of the candidate cell. Regarding how to handle the TA timer of the target cell, two options have been achieved at the last meeting as follow:

-	Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18;

-	Option 2 (keep the offline agreement): discuss the options below in the MAC running CR

o	Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR;

o	Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running.

o	Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer.

In R18 intra-CU LTM, it’s gNB to maintain the validity of the TA and UE has no information about the validity of the TA. In this case, the UE can only use the PTAG timer configured by gNB upon the LTM cell switch. However, it’s UE to maintain the TA timer in intra-CU conditional LTM and UE knows the validity of the TA timer better than the gNB. Thus, the above option 2 is preferred in CLTM. Among the three sub-options, the option 2.3 is the simplest operation which is more backwards compatible.

Observation5: In CLTM, it’s UE to maintain the validity of TA and UE knows the validity of the TA better than the gNB.

Proposal2:The above option2.3 is preferred.

Regarding the handling of the TA timer of the other candidate cells, the timer is either stopped or kept running. In RAN2#129, it is agreed that the fast recovery of R18 is reused. In R18 intra-CU LTM, there is no valid TA of other candidate cells available at the UE. However, in intra-CU CLTM, if the user has the valid TA values of the other CLTM cells, the UE can still perform the RACH-less CLTM to the other cells. To enable UE to perform RACH-less CLTM to other candidate cells with valid TA values after the first CLTM cell switch failure, the TA timers of these cells should be kept running. However, in RAN2#129 companies think there is no need to perform CG-based C-LTM fast recovery. Thus, the TAT  of the other candidate cells shall be stopped upon the CLTM switch.

Observation6: Companies think there’s no need to perform the CG-based C-LTM fast recovery.

Proposal3: The TA timers of other candidate cells can be stopped.

If the above proposal is agreed, the TA values of other candidate cells are released. Because the validity of the TA is unknown to the UE, if the TA timers of other candidate cells are stopped.

Proposal4: If the proposal above is approved, the TA values of other candidate cells can be released.

Which beam to perform the CLTM cell switch for L3 based C-LTM

In the last meeting, the following agreements for beam selection for RACH-less C-LTM  in L1 based C-LTM case are as follows:

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

However, the beam selection for RACH-less C-LTM in L3 based C-LTM needs to be considered too. If the cell switching is triggered by the L3 execution conditions to a CLTM candidate cell, and there are activated TCI states for this CLTM candidate cell, UE select one of the activated TCI states for accessing the cell. However, if there is no activated TCI state for this CLTM candidate cell, then the L3 beam level measurement results of this CTLM candidate cell can be used by UE for selecting the best beam as the target beam for accessing the target candidate cell.

Proposal5: If the CLTM cell switching is triggered by the L3 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the best beam as the target beam for accessing the target candidate cell.

- Conclusion

In this contribution, we discussed the issues related to Conditional LTM. We have the following Observations and Proposals:

Observation1: In R18 LTM, DU has already generated the LTM-CSI-ReportConfig for the LTM.

Observation2: In R19 LTM, for measurement resource configuration, R18 LTM CSI resource configuration is reused.

Observation3: In R19 LTM, for measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused.

Observation4: Re-use existing L1 events for CLTM.

Proposal1: DU can configure the L1 execution condition in the LTM-CSI-ReportConfig.

Observation5: In CLTM, it’s UE to maintain the validity of TA and UE knows the validity of the TA better than the gNB.

Proposal2:The above option2.3 is preferred.

Observation6: Companies think there’s no need to perform the CG-based C-LTM fast recovery.

Proposal3: The TA timers of other candidate cells can be stopped.

Proposal4: If the proposal above is approved, the TA values of other candidate cells can be released.

Proposal5: If the CLTM cell switching is triggered by the L3 execution conditions, the beam used by UE for accessing the cell can have the following options:

Option A: UE selects the beam associated with an activated TCI state as the target beam for accessing the target candidate cell.

Option B: If there is no activated TCI state available, UE selects the best beam as the target beam for accessing the target candidate cell.

References

- RAN2 #128 chairman notes

- RAN2 #129 chairman notes