3GPP TSG-RAN WG2 #129bis	R2-2502299

Wuhan, China, 7-11 April 2025

Agenda item:		8.6.2	Inter-CU LTM

Title:		Discussion on inter-CU LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During the last meeting, RAN2 further discussed several aspects to support inter-CU LTM and some agreements for inter-CU LTM have been reached [1].

Agreements on inter-CU LTM:

1.	Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s).

2.	Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

3.	Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

4.	For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

5.	For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

6.	How to support shared RACH resources for inter-CU LTM is left to RAN3.

In this contribution, we further discuss the remaining issues or details for inter-CU LTM and provide our consideration.

# 2. Discussion

2.1 Security and L2 behavior

2.1.1 security for inter-CU LTM

SA3 has sent the reply LS [2] on security handling for inter-CU LTM in non-DC cases.

1. Overall Description: SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover. 2. Actions: To RAN2 and RAN3 ACTION: SA3 kindly asks RAN2 and RAN3 to take the above information into account. To RAN3 ACTION: SA3 kindly asks RAN3 to decide and update the SA3 on the key distribution mechanism.

Based on the reply LS, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message. Therefore, from RAN2 perspective, we think RAN2 should agree the following:

- For inter-CU LTM, the NCC should be included in LTM cell switch command MAC CE.

2.1.2 PDCP reestablishment determination

During the previous meeting, RAN2 agreed that the UE shall perform the PDCP re-establishment for inter-CU LTM. And as baseline whether the PDCP re-establishment is performed based on the new Rel-19 ID (i.e., ltm-NoSecurityChangeID) in RRC, i.e., if the ltm-NoSecurityChangeID of target cell is different from the source cell, the UE performs PDCP re-establishment. Of course, it should be noted that RAN2 also agreed that we can revisit it dependent on SA3 response for security issue.

RAN2#127

RLC and PDCP re-establishment in inter-CU LTM:

6.	If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

In addition, RAN2 further agreed that some details on PDCP re-establishment determination for DRBs and SRBs in different scenarios.

RAN2#128:

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

RAN2#129

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

Actually, the Rel-19 ID (i.e., ltm-NoSecurityChangeID) is used by the UE to determine whether the cell switch is an inter-CU LTM, thereby determining whether the UE needs to perform PDCP re-establishment. If the NCC is provided to the UE by RRC, it is reasonable for the UE to perform PDCP re-establishment based on the ltm-NoSecurityChangeID.

However, based on the SA3 response for the security issue [2], the NCC can be transmitted to the UE via MAC CE message. Therefore, whether to perform security updates and the PDCP re-establishment should be determined based on whether the NCC is provided in LTM cell switch command MAC CE, since the network can include the NCC in LTM cell switch command in case the inter-CU LTM is triggered or in case the security key updated is needed.

Therefore, take the SA3 replay LS, the above agreements, and the above analyses into consideration, we propose:

- For inter-CU LTM, whether the PDCP re-establishment should be performed based on whether the NCC is provided in LTM cell switch command, instead of based on ltm-NoSecurityChangeID.

- RAN2 to revise the previous agreements for PDCP re-establishment based on ltm-NoSecurityChangeID as based on NCC:

- For DRB:

- For inter-CU MCG LTM: if the NCC is included in LTM cell switch command, the UE performs PDCP re-establishment to all radio bearers

- For inter-CU SCG LTM: if the NCC is included in LTM cell switch command,

  - For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

  - For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

  - If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRB:

- For inter-CU MCG LTM, if the NCC is included in LTM cell switch command, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, if the NCC is not included in LTM cell switch command, the UE performs PDCP SDU discard for SRB1/SRB2.

- For inter-CU SCG LTM, whether the NCC is included in LTM cell switch command is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

2.1.3 L2 handling not supported in Rel-18 due to too late

During discussion of Rel-18 mobility correction last meeting, triggering of PDCP recovery for AM DRB with RLC bearer released in case of LTM cell switch without L2 reset was discussed, and companies think that it is technical correct, but it is too late to support. The Rel-18 network implementation should ensure that the concerned case never happens.

R2-2410063 Remaining issues on L2 reset for LTM NEC discussion Rel-18 NR_Mob_enh2-Core Proposal: In case of LTM cell switch without L2 reset, UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released. The draft CR in Annex is adopted. Not pursued. [Apple]: Wonders if the scenario is really valid one. [Ericsson]: Considering it is NBC change, prefer not to have it. We still can survive w/o this change. [Xiaomi]: If companies are not favourable with this change, it’s good to capture that NW ensures this situation doesn’t happen in the meeting minutes. [Samsung, Qualcomm]: Although we agree in technical point of view, also share the concern with Ericsson. It may be too late to specify it now. [Vivo]: It’s ok not to agree with this correction, but considering it is valid scenario since it was already considered in SCPAC and specified it, it would be better to capture that NW ensures this situation doesn’t happen. [Ericsson]: Prefer to do nothing. Reasonable NW implementation takes care of it. [MediaTek]: Agree with Ericsson. [Google]: Agree with Ericsson. We may consider for Rel-19.

For legacy handover procedure, RLC bearer release can happen in case of intra-CU handover case, which means that there is no such restriction compared with LTM. To the remove this restriction for LTM, the related UE behavior can be supported in Rel-19.

- Support in Rel-19 that the UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released in case of LTM cell switch without L2 reset. And RAN2 to agree the corresponding TP in Annex.

2.2 LTM fast recovery for inter-CU LTM

On the issue of LTM fast recovery, RAN2 agreed the following agreements during previous meeting.

RAN2#127bis: The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

RAN2#128: RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

Based on the SA3 reply LS on security handling for inter-CU LTM, we observed that the NCC can be transmitted to the UE in plaintext via MAC CE message, i.e., LTM cell switch command MAC CE. For LTM fast recovery upon LTM failure, regardless of intra-CU LTM failure or inter-CU LTM failure,e the UE does not have NCC to update the security key because there is no LTM cell switch command MAC CE message to provide the NCC in the LTM failure case. Therefore, the simplest way is not to support the inter-CU LTM fast recovery upon LTM failure.

- Inter-CU LTM fast recovery upon LTM failure is not supported.

Regarding how to determine whether the selected cell is an inter-CU LTM candidate cell upon LTM failure, we think it can be based on the ltm-NoSecurityChangeID. If the selected cell is one of the LTM candidate cells and if the ltm-NoSecurityChangeID of the source cell is same as the selected cell, the UE performs the LTM cell switch procedure for the selected LTM candidate cell for fast recovery.

- Upon inter-CU LTM failure, the UE determine whether the selected cell is inter-CU LTM candidate cell based on the ltm-NoSecurityChangeID. If the selected cell is one of the LTM candidate cells and if the ltm-NoSecurityChangeID of the source cell is same as the selected cell, the UE perform the fast recovery procedure, otherwise, the UE perform the RRCReestablishment procedure.

2.3 Inter-CU SCG LTM

In CPAC, upon execution of CPAC procedure, the UE transmits a MN RRCReconfigurationComplete including SN RRCReconfigurationComplete message and the identity of the applied conditional reconfiguration. The motivation behind is to assist the MN to identify the target SN of the CPAC procedure. During previous meeting, it was agreed that upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. Similar as CPAC, to assisted the MN to identify the target SN, the UE should include the identity of the applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

- For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Additionally, in Rel-18 intra-CU LTM, it was agreed to include the identity of the applied LTM candidate configuration in the RRCReconfigurationComplete message, with the motivation behind is to indicate to the gNB-CU that the RRCReconfigurationComplete is for the LTM cell switch procedure, in case that LTM and CHO are configured for the same candidate cell, or multiple LTM configuration are associated with the same candidate cell. The corresponding chairman’s note can be seen in the below table. For SN/SCG, it is also possible that LTM and CPAC are configured for the same candidate PSCell, or multiple LTM configuration are associated with the same candidate PSCell. Therefore, for inter-CU SCG LTM, to assist the target SN to identify the applied LTM candidate configuration, the UE should also include the identity of the applied LTM candidate configuration in the SN RRCReconfigurationComplete message.

R2-2403177 [E068][E231][E074][E240][S792]Resolution of remaining RILs for LTM Ericsson discussion Rel-18 NR_Mob_enh2-Core Late E74: “Transaction ID” - Apple agrees with Ericsson proposal, and think this is helpful for R19 as well. - CATT think it is not a valid case that multiple configurations are applicable for one target cell. - MTK think now we have some cases where this may apply and think this is helpful for R19 - HW think the only scenario is LTM and CHO config for the same cell, think only 1 bit is needed. - vivo prefer to not do any optimization, can configure different C-RNTI for different config. - LGE think that with extended transaction ID space this is resolved. - Xiaomi think that complete message already contains the conditional configuration ID. - Ericsson think there can be multiple LTM configs per target cell, and think that CRNTI check does not work. - Intel support this proposal, it seems to be a clean solution. Lenovo as well. - Nokia wonders if this is really a real problem .. Upon an LTM cell switch, the UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

- For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

2.4 Additional complete indicator

During the last meeting, there was an FFS on whether only one indication is needed to indicate LTM configuration as complete configuration. The mainly purpose of company proposes to introducing an additional complete indicator for MCG in LTM-Candidate-r18 [3] is to achieve the case that the SN and MN can use the different configuration (e.g., SN uses the complete configuration and the MCG to use delta configuration).

For SCPAC, both the MCG part and SCG part can also be included in conRRCReonfig, and there in only one scpac-ConfigComplete indication to indicate whether the SCPAC configuration is delta or complete configuration, there is no enhancement to implement the different configuration (i.e., delta or complete) for MCG and SCG in SCPAC. Similarly, for inter-CU SCG LTM, we think there is no need to introduce the additional complete indicator to indicate LTM configuration as complete configuration.

- For inter-CU LTM, there is no need to enhance the complete indication, i.e., only one indication is needed to indicate LTM configuration as complete configuration.

# 3. Conclusion

In this contribution we discussed some issues related to inter-CU LTM and made the following proposals.

- For inter-CU LTM, the NCC should be included in LTM cell switch command MAC CE.

- For inter-CU LTM, whether the PDCP re-establishment should be performed based on whether the NCC is provided in LTM cell switch command, instead of based on ltm-NoSecurityChangeID.

- RAN2 to revise the previous agreements for PDCP re-establishment based on ltm-NoSecurityChangeID as based on NCC:

- For DRB:

- For inter-CU MCG LTM: if the NCC is included in LTM cell switch command, the UE performs PDCP re-establishment to all radio bearers

- For inter-CU SCG LTM: if the NCC is included in LTM cell switch command,

  - For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

  - For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

  - If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRB:

- For inter-CU MCG LTM, if the NCC is included in LTM cell switch command, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, if the NCC is not included in LTM cell switch command, the UE performs PDCP SDU discard for SRB1/SRB2.

- For inter-CU SCG LTM, whether the NCC is included in LTM cell switch command is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- Support in Rel-19 that the UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released in case of LTM cell switch without L2 reset. And RAN2 to agree the corresponding TP in Annex.

- Inter-CU LTM fast recovery upon LTM failure is not supported.

- Upon inter-CU LTM failure, the UE determine whether the selected cell is inter-CU LTM candidate cell based on the ltm-NoSecurityChangeID. If the selected cell is one of the LTM candidate cells and if the ltm-NoSecurityChangeID of the source cell is same as the selected cell, the UE perform the fast recovery procedure, otherwise, the UE perform the RRCReestablishment procedure.

- For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

- For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

- For inter-CU LTM, there is no need to enhance the complete indication, i.e., only one indication is needed to indicate LTM configuration as complete configuration.

# 4. References

[1] Draft_RAN2_129_Meeting_Report_v3

[2] S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, source SA3

[3] 2500442, Discussion on Inter-CU LTM, ZTE

# 5. Annex (TS 38.331)

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch conditions, the UE shall:

1>	else if the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> else:

2>	for each drb-Identity value included in each RadioBearerConfig for the cell group for which the LTM cell switch procedure is triggered:

3>	if an associated RLC bearer is released in the LTM candidate configuration to be applied:

4>	if the bearer is an AM DRB:

5>	after the end of this procedure, trigger the PDCP entity of the bearer to perform PDCP data recovery as specified in TS 38.323 [5];