3GPP TSG RAN WG2 Meeting #129bis	R2-2502302

Wuhan, China, Apr 7th – 11th, 2025

Agenda item:	8.7.5

Source:	Quectel

Title:	Further discussion on RLC retransmission for XR

Document for:	Discussion

- Introduction

In RAN2 #129 meeting, following agreement is achieved [1]:

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling

In the contribution, some details for RLC retransmission are discussed furtherly.

- Discussion

The first issue is whether the remaining time is determined based on discardTimer at PDCP or new timer at RLC. Since the object of autonomous retransmission is improving the probability of successful transmission of packet before discardTimer expiry, it is natural to trigger this retransmission based on discardTimer at PDCP.

Proposal 1: The remaining time is determined based on discardTimer at PDCP.

If the remaining time is determined based on discardTimer at PDCP, PDCP shall indicate to RLC when the remaining time is below the threshold. There are two alternatives:

- Alternative 1: PDCP indicates to RLC for the PDCP PDU which is not acknowledged by RLC status report.

- Alternative 2: PDCP indicates to RLC for every PDCP PDU, even if it is acknowledged by RLC status report.

In the current RLC specification, RLC shall indicate to upper layers of successful delivery of the RLC SDU when the positive acknowledgement is received via RLC status report [2]. So alternative 1 is a better than alternative 2 because of less indication from PDCP to RLC.

Proposal 2: when the remaining time is below the threshold, PDCP indicates to RLC for the PDCP PDU which is not acknowledged by RLC status report.

From RLC point of view, there are 3 cases when the indication is received from PDCP:

- Case 1: the PDCP PDU has been transmitted, and received non-positive acknowledgement via RLC status report.

For this case, RLC retransmission is needed, but is not the autonomous retransmission.

- Case 2: the PDCP PDU has been transmitted, and still not receive any acknowledgement.

For this case, RLC shall retransmit this packet without RLC status report.

- Case 3: the PDCP PDU has not been transmitted.

For this case, it is not needed to insert this packet into the RLC buffer again.

Based on the discussion above, RLC shall only prepare autonomous retransmission for case 2.

Proposal 3: Receiving the indication from PDCP, RLC shall prepare autonomous retransmission if the RLC SDU has been transmitted and still not receive any RLC feedback.

For the uplink XR data, once the autonomous retransmission is triggered, when the actual retransmission happens depends on the gNB scheduling. If the positive acknowledgement is received before the uplink grant which is for autonomous retransmission, this RLC retransmission shall be cancelled.

Proposal 4: If the positive acknowledgement is received before the uplink grant which is for autonomous retransmission, this RLC retransmission shall be cancelled.

- Conclusion

Based on the discussion above, the following proposals are suggested:

Proposal 1: The remaining time is determined based on discardTimer at PDCP.

Proposal 2: when the remaining time is below the threshold, PDCP indicates to RLC for the PDCP PDU which is not acknowledged by RLC status report.

Proposal 3: Receiving the indication from PDCP, RLC shall prepare autonomous retransmission if the RLC SDU has been transmitted and still not receive any RLC feedback.

Proposal 4: If the positive acknowledgement is received before the uplink grant which is for autonomous retransmission, this RLC retransmission shall be cancelled.

- References

- RAN2 #129 minutes

- 3GPP TS 38.322 V18.2.0