3GPP TSG-RAN WG2 #129bis	R2-2502303

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.2.4

Source:	Ericsson

Title:	A-IoT Data Transmission and Other Aspects

Document for:	Discussion/Decision

# 1	Introduction

In RAN#106, a new WI has been approved for Ambient IoT devices.

This paper discusses aspects of data transmission functionality and other remaining general aspects required for A-IoT D1T1 devices, such as MAC PDU format and control signaling, data (re)transmission for failure handling, segmentation/reassembly, ang AS ID handling.

# 2	Discussion

## 2.1	MAC PDU format and control signaling

In previous discussions, RAN2 included an FFS for multicast R2D messages other than Msg2. R2D messages after Msg2 are all in the nature of unicast messages, i.e., targeting only one device. However, reader can also send a R2D message in multicast manner to reduce overhead, e.g., a common acknowledgement in Msg4 to multiple devices after their Msg3, or a common command request to multiple devices. However, in this case, the size of such R2D message might be very large considering control information in Msg4, e.g., AS ID assignment, RN16 for addressing, security information for authentication. In addition, a device that has sent Msg3 may have to wait long until receiving a common Msg4. For simple design, we think there is no need of multiplexing of information for multiple devices in R2D message after Msg2.

Multiplexing of information for multiple devices in R2D message after Msg2 is not supported.

For MAC PDU design, it is worth listing possible A-IoT messages in both D2R and R2D transmission directions including field information and their use. Table 1 and table 2 below show possible messages to consider.

#### 2.1.1	D2R (including segmentation support)

Table 1: Possible D2R messages for MAC PDU design

In D2R direction, Msg1 contains RN16 and thus can be distinguished from other D2R message; given that all other D2R messages are larger in size. Meanwhile, the D2R message from Msg3 onwards can have the same format considering their content of having upper layer information, UL control information (e.g., segmentation support). Thus, we think there is no need of the message type field in the MAC header for D2R. If this construct is possible, the message size would be small.

Then Msg3 and onwards contains upper layer data (NAS PDU) with possible variable size. RAN2 already agreed 1-bit indication to support segmentation. However, to enable the reader to estimate the remaining size of next segment(s) to allocate radio resources, especially in case expected D2R message size is not provided by CN, the 1-bit indication is not sufficient.

In addition, Msg3 may need to carry security information in support of the challenge-response authentication mechanism if defined (as in SA3 TR). Figure 1 shows an example of MAC PDU content for Msg1 and other D2R messages, respectively. Note that this example does not comprehensively list options, e.g. for where a smaller optimized size could be made, due to e.g. a fixed size payload part or where a random ID is not needed.

Figure 1: Example of MAC PDU format for D2R messages.

No need of message type field in D2R message.

MAC PDU for Msg1 contains only RN16.

When it comes to the need of padding in the D2R MAC PDU with variable size, it has been agreed RAN1 that the reader provides the payload size (grant size) for the D2R message in the preceding R2D message.

Agreement from RAN1-120

The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.

Given that the reader may not know exact size of the D2R message irrespective of having assistant information from CN or not, padding is needed when the grant size (TBS-like) allocated by the reader is larger than actual data and control signal in the D2R message. This applies irrespective of bit or byte-alignment for MAC PDU format.

MAC PDU for D2R message other than Msg1 support inclusion of padding.

For the D2R message from Msg3, given that we may have command responses, the reader can know from which device (or RN16 sender) the D2R message comes from the resources where the transmission is on (i.e., scheduled in preceding R2D message). With this, AS ID (or RN16) does not need to be included in the D2R message.

On D2R MAC PDU design, as the availability of estimated sizes is still somewhat unclear, and that parsing a PDU with padding has not been discussed in detail, it is still reasonable to decide if the 1-bit indication is sufficient or not (see RAN2 FFS). Currently, for length field we foresee two options and the comparison including pros and cons in provided the table below.

Table 2

Length field options

| Option 1 | Option 2 |
| --- | --- |
| - Length field indicates total size of upper layer data (one can think of buffer reporting although system is buffer-less) - In case of segmented transmission, the reporting over length field must be provided in first segment; this enables reader to allocate resources efficiently for successive segments - The length field would be practically not useful for remaining segments if first segmented had indicated total upper layer data size - The reader needs to derive resources for various as per indicated total upper data size from device | - Length field indicates the size of included payload in current D2R transmission - In case of segmented transmission, the information carried in the length field for last segment is very crucial, as the PDU may contain padding in addition to payload, so the length field reporting enable network to identify payload in PDU - The length field usage in other non-last segments may not be practically important, as the PDU expected to be filled with payload without padding as there is remaining data (to be transmitted over next segment(s)) - The device needs to calculate value of length especially for last segment |

On aspects of retransmission, we believe the behavior of segmented and non-segmented transmission should be similar, and in both cases the offset is indicated. A dynamic inclusion of offset, e.g., only in segmented transmission may result in increased complexity compared to an unified behavour. For non-segmented transmission or retransmission, offset can be set to zero.

On handling erroneous cases where a segment fails, the reader can trigger retransmission for the failed segment or re-access. For re-access, the device always begins the transmission from the scratch, i.e., with the first segment if device passes contention.

In case of segmentation, 1-bit indication from device is sufficient to manage resources except for allocation resources of last segment.

The device ID selection behavior in Msg3 is not settled in SA3. This may pose difficulty related to offset based mechanism for segmented transmission.

No need of addressing information (i.e., AS ID/RN16) inside D2R message.

MAC PDU for D2R message containing upper layer data except Msg1 includes:

- A length field that indicates

  - Option 1: the total size of upper layer data (at least in the first segment transmission), or

    - If this option is agreed, then 1-bit indication is not needed

  - Option 2: the size of included payload in MAC PDU (at least in the last segment transmission)

- 1-bit segment indication (may not be needed depending on the use of length field

- Upper layer data (variable size)

- Padding if needed.

An unsegmented transmission can be retransmitted in similar manner as segmented transmission by indicating a zero offset.

For segmented transmission, if re-access is triggered, the device begins transmission from the scratch (or with a first segment if segmentation needed) in a new occasion after passing contention.

#### 2.1.2 R2D

In the R2D transmission direction, there are more message types over the air interface, as exemplified in Table 2. RAN2 already agreed a message type field in MAC header for device(s) to identify and parse the received message. Currently, there are approx. 3 (+1 FFS) R2D messages, however, we believe some messages are obtained as changing the parameters of given field, and in essence we don’t see the need for more than 4 R2D message types, see Table 3, and thus we believe, as a starting point at least 2 bits seems sufficient for a message type field. Note that for future extension, it may be useful to use a 3-bit type field

Table 3

Possible R2D message types for MAC PDU design

| Index | R2D message type | Content | Comment |
| --- | --- | --- | --- |
| 1 | Paging message | - Transaction ID - Paging identifier - Access resource information | MAC PDU design depends on if (which part of) scheduling info is MAC control info (RAN1 pending) and if there is security info (SA3 pending). |
| 2 | Msg2 or contention resolution message | - One or more RN16s - Resource allocation for Msg3 and possible subsequent R2D messages - Possible AS ID allocation | - Msg2 can be used to address one or more devices for - segment or non-segment initial transmission - segment or non-segment re-transmission - In random access agenda item, NACK is agreed for re-access, and we believe, Msg2 can be repurposed to deliver NACK |
| 3 | Command related message | - READ command request message - Resource allocation for D2R message - Upper layer information related to READ request - WRITE command message - Resource allocation for R2D message - Upper layer information corresponds to WRITE data |  |
| 4 | QueryRep like signalling | - Msg1 resource or access occasion indication in time domain (timing acquisition) - Possible Msg1 resource update | It is unclear now whether it will L1 message or L2 message, and for L2, whether it will be a new R2D message or a repurposed paging message |

Note: In the discussion here, the inclusion of reader ID like information in messages is not considered. As of now, R2D transmissions may carry AS or (possibly unique) transaction ID which can help devices to differentiate targeted and non-targeted transmission in overlapping scenarios.

Four R2D message types have been identified: Paging message, contention resolution message (Msg2), command message and QueryRep like message.

Aim is to restrict to fewer message types, and utilize a given message for multiple purposes. E.g., Msg2 can be utilized to perform/indicate initial or re-transmission of Msg3 with or without segments, or NACK to trigger re-access.

At least 2-bit message type field is used to identify R2D message at present.

It is still unclear exactly what the content of the A-IoT paging message is, including if impacted from subsequent paging. For example, the configuration and scheduling information regarding access occasion, frequency offset (to support FDMA + TDMA in D2R transmission), may be either carried by MAC, by PHY control information or both. RAN2 should wait for RAN1 progress on some of the aspects, but RAN2 can move forward with aspects necessary for contention resolution, for instance utilization of FDMA indices in MAC for mapping with successful RN16s. The same applies to the R2D message used to update/adjust configurations for all devices in an access/paging/inventory round. In addition, RAN2 can assume that that least the R2D message used for synchronization purposes (QueryRep-like) is part of PHY layer control signaling.

Msg2 used to echo the RN16 in response to Msg1 can have its own format. Msg2 may contain more than one RN16 value. In addition, RAN2 needs to wait for progress from RAN1 to see if any L2 control signaling is needed, e.g., to facilitate a device to process only some parts meant for it rather than processing the whole PDU. For example, if a relation to FDMA index to selected RN16 would be defined. That is, if valid RN16s are indicated in association with a FDMA index in the downlink (Msg2) message, a device needs only to parse the RN16 related to the FDMA occasion where it had transmitted. It remains open if any or what information for scheduling Msg3 is needed in the Msg2 MAC PDU.

Before Msg3 is successfully delivered, the Msg2 can also be used to support a feedback mechanism, i.e., to indicate explicitly or implicitly to a device to perform retransmission of the previous D2R message, or to perform re-access (NACK on Msg3), or to transmit the next D2R segment. These cases can be combined in one MAC PDU format. In these cases, there need to be a device ID in such an R2D message indicating the targeted device. Given that the reader has not successfully received the complete upper layer device ID (in Msg3), this device ID can only be the AS ID/RN16.

For each echoed RN16, there should have some mapping between the echoed RN16 and FDMA resource. This allows devices not to parse every RN16 and look for own RN16 only subject to mapping behaviour between the FDMA resource and indicated RN16. The mapping can be either:

- Static, or

- Dynamic,

  - Bitmap based,

  - Index based.

In case of static mapping, the Msg2 contains the same number of placeholders as number of available FDMA access occasions. For the FDMA occasions which is accessed successfully, the corresponding placeholder in Msg2 indicates RN16 which is addressed over those access FDMA occasion. Other placeholders may indicate dummy RN16 where no device is identified. The drawback is the Msg2 length becomes long even if the number of accessed devices is small.

In the other option, the Msg2 content can be dynamic and include only successful RN16s. Compared to static scenario, the message length is small as its size is limited by the number of devices responded to in a particular Msg2, not by total number of FDMA occasions configured. For a dynamic mapping, there can two options:

One option is bitmap based: For each FDMA occasion, there is a corresponding bit in the bit string which represents an association with FDMA occasion. If the bit is set, then corresponding payload contains successful RN16 transmitted over FDMA occasion which maps to the set bit in the string. One issue with this approach is that the devices need to know the order of bits mapped to the FDMA occasions a-priory, and if Msg1 resource is updated, then bitmap needs to be updated as well.

In the other option, the FDMA occasion index value is included in the header and the corresponding payload contains the accessed device’s RN16 transmitted over the FDMA occasion and is denoted by the same FDMA index value, see Fig. 2.

Figure 2: Dynamic mapping of RN16s with indicated FDMA indices.

We believe a dynamic option based on FDMA index as an additional header field associated with a corresponding RN16 in the message payload is most suitable. The option with bitmap can also work but there are minor issues as mapping order needs to be specified (e.g., which bit in a bit string maps to which FDMA resource) and the size of bitmap may vary if Msg1 resource configuration changes (especially if Msg1 resources added or removed relative to previous configuration).

It was agreed that MAC PDU for the R2D message with variable size contains message type field, a length field for the variable part. This format is for R2D from Msg4 that carries DL NAS data, i.e., command request received from CN and possibly other NAS information. Considering the upper bound of 1000 bits TBS, we think 7 bits are sufficient for the length. In addition, depending on whether the R2D message contains any security related information, either at NAS level or MAC level, other potential DL control information may be needed, pending SA3 progress.

MAC PDU for Msg2 includes:

- R2D message type

- One or multiple RN16s subject to message size limitation.

- FDMA index as subheader and corresponding payload indicates successful RN16. FFS if FDMA index related information is available at L1 or MAC.

  - Device assumes number of FDMA indices or RN16s are known for Msg2 decoding.

- FFS on AS feedback indication for re-access.

- FFS AS ID assignment

Support 7-bit length field in MAC PDU of R2D message with NAS information of variable size.

### 2.1.3 General message failure considerations

There may also be necessary to consider handling of erroneous situations pertaining to command related transmission failure. Notably there are two types of commands: read and write.

It is already established that after passing contention (Msg1 is successful), but when Msg3 has failed, the reader can trigger:

- Retransmission (retransmit Msg2 again)

- Re-access (transmit AS layer based NACK).

However, in a scenario (inventory plus command), if Msg3 is successful, but subsequent transmission has failed, then in such re-access is not. Reader always the failed device with reatrsnmission either in same CBRA occasion (where contention is successful) or via new CFRA procedure. Reader can establish failure at device accordingly

- For read command request scenario, if device does not transmit D2R message containing upper layer data or read data, reader can assume read command has failed, and retransmit read command again,

- For write command scenario, if device does not transmit D2R message containing upper layer acknowledgement in response to write command, reader can assume write command has failed and retransmit write command again. Note, the upper layer response is required in response to write command and SA2 has already agreed [S2-2501241]

Hence, we believe, the failure in command related transmissions always be dealt with retransmissions as device credentials are known and contention has passed.

In case of inventory plus command use case, the device is always expected to deliver D2R message containing upper layer data in response to command request. In case of read request, the D2R message carries read data, and in case of write command, the D2R message carries upper layer response.

In case of inventory plus command use case, if Msg3 (carrying device credentials) transmission is successful but subsequent command related transmission has failed, then reader triggers retransmission if it has failed to receive or decode D2R message containing upper layer data in response to command request. Re-access is not needed.

Upon negative acknowledgement of command reply, the device expects to retransmit not re-access.

## 2.2	AS ID

AS ID was discussed in RAN2-129 meeting and there remains some open aspects as below. In addition, the is the post-meeting email discussion to progress on this topic.

Agreements (AS ID) - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

In this section, we discuss our view in greater detail on some of the aspects of AS ID.

Regarding AS ID assignment in CBRA, there are several options to assign AS ID for a device, i.e., as part of Msg2, or in another R2D messages. In our view, irrespective of the options, for the device to know the AS ID is assigned for it, the addressing information is needed. In addition, this addressing cannot be based on the upper layer ID (e.g., CN allocated temporary ID or permanent ID), which is not visible to reader. Therefore, RN16 needs to be used for addressing purpose for AS ID assignment.

For CBRA, RN16 is used in the R2D message for AS ID assignment for addressing purpose.

When it comes to validity of AS ID, we think no need for device and network to keep AS ID after completion of the ongoing inventory + command procedure. Otherwise, the requirement on device side to keep the AS ID across access/paging round would imply significant device complexity unnecessarily. In addition, the need of addressing and scheduling is only valid among the devices accessing to the same access occasion. One related topic being in the email discussion is how device determines the completion of inventory + command procedure. Given that reader anyway needs to send R2D messages for Msg1 resource determination, e.g., QueryRep-like message. We think that this signaling can be used as an indication for the completion of the inventory + command procedure and thus the device can release AS ID, if available. Note that in case of re-access (upon reception of a NACK on D2R transmission), the procedure is considered completed even though not successfully completed. Therefore, the reception of a QueryRep-like message can be used as the indication for procedure completion.

For CBRA, from a device’s perspective, AS ID is valid until completion of the inventory + command procedure (Option 4b).

Device considers procedure completed upon reception of indication from reader for Msg1 resource allocation (e.g., QueryRep-like signaling).

For CBRA, we think that the size of AS ID can be much shorter than 16 bits, especially not all devices attempt to access in the same occasion. However, given additional overhead related to AS ID assignment (e.g., RN16 is anyway needed for addressing device in CBRA and there may need a new R2D message than Msg2 in case of shorter AS ID), we propose 16 bits for AS ID size, and this applies to both CFRA and CBRA.

16 bits for AS ID size for both CFRA and CBRA.

In CFRA, we assume the A-IoT paging can target a single device or multiple devices to access with CFRA. Considering AS ID is for addressing and D2R scheduling, in case of single device CFRA, there is no need of AS ID. We do not see the need of AS ID for CFRA. However, in case AS ID is agreed for multi-device CFRA, we think reader can assign AS ID in the Msg2 (with command request) to be unified with CBRA. In addition, we think the AS ID assignment can be done without the need of RN16 sent in Msg1.

For multi-device CFRA, AS ID can be allocated if the use case necessitates. The AS ID can be allocated based on Option 4, i.e., via Msg2 (command request) without RN16 selected by device in Msg1.

## 2.3	Other remaining aspects

Current SA2 specification for A-IoT (TS 23.xyz-020_AIoT_rm) provides general description of inventory and command procedures. Regarding the assistance information provided to RAN/reader, it is states:

The following assistance information may be provided:

-	AIoT service type (e.g. Inventory, Command);

-	Optionally, approximate number of AIoT devices based on AF request;

-	Optionally, approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

For the inventory + command procedure, it is not clear when/how the RAN/reader receives the command request (NAS container). For example, if the NAS container for command request is provided to RAN as part of the service request, RAN may deliver it to the targeted device(s) without a confirmation from CN (e.g., authentication steps). Otherwise, RAN needs to always wait for CN to verify the device ID or command reply before receiving the next NAS container to deliver to device(s).

In addition, from a RAN’s perspective, RAN/reader also needs to know if CN (AIOTF) intends to command the device(s) once or there are multiple commands for the ongoing service request. Without such information in advance, RAN/reader may need to keep the device(s) waiting long unnecessarily.

RAN2 should send an LS to ask SA2 for clarification on these aspects.

RAN2 send an LS to SA2 asking clarifications on following aspects:

- Whether the NAS container for command request is sent as part of the service request or after device identifier verification.

- Whether CN can command device(s) multiple times with one service request.

- Any assistance information in service request indicating single or multiple command rounds.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In case of segmentation, 1-bit indication from device is sufficient to manage resources except for allocation resources of last segment.

Observation 2	The device ID selection behavior in Msg3 is not settled in SA3. This may pose difficulty related to offset based mechanism for segmented transmission.

Observation 3	Four R2D message types have been identified: Paging message, contention resolution message (Msg2), command message and QueryRep like message.

Observation 4	Aim is to restrict to fewer message types, and utilize a given message for multiple purposes. E.g., Msg2 can be utilized to perform/indicate initial or re-transmission of Msg3 with or without segments, or NACK to trigger re-access.

Observation 5	In case of inventory plus command use case, the device is always expected to deliver D2R message containing upper layer data in response to command request. In case of read request, the D2R message carries read data, and in case of write command, the D2R message carries upper layer response.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Multiplexing of information for multiple devices in R2D message after Msg2 is not supported.

Proposal 2	No need of message type field in D2R message.

Proposal 3	MAC PDU for Msg1 contains only RN16.

Proposal 4	MAC PDU for D2R message other than Msg1 support inclusion of padding.

Proposal 5	No need of addressing information (i.e., AS ID/RN16) inside D2R message.

Proposal 6	MAC PDU for D2R message containing upper layer data except Msg1 includes:

	A length field that indicates

o	Option 1: the total size of upper layer data (at least in the first segment transmission), or

	If this option is agreed, then 1-bit indication is not needed

o	Option 2: the size of included payload in MAC PDU (at least in the last segment transmission)

	1-bit segment indication (may not be needed depending on the use of length field

	Upper layer data (variable size)

	Padding if needed.

Proposal 7	An unsegmented transmission can be retransmitted in similar manner as segmented transmission by indicating a zero offset.

Proposal 8	For segmented transmission, if re-access is triggered, the device begins transmission from the scratch (or with a first segment if segmentation needed) in a new occasion after passing contention.

Proposal 9	At least 2-bit message type field is used to identify R2D message at present.

Proposal 10	MAC PDU for Msg2 includes:

	R2D message type

	One or multiple RN16s subject to message size limitation.

	FDMA index as subheader and corresponding payload indicates successful RN16. FFS if FDMA index related information is available at L1 or MAC.

o	Device assumes number of FDMA indices or RN16s are known for Msg2 decoding.

	FFS on AS feedback indication for re-access.

	FFS AS ID assignment

Proposal 11	Support 7-bit length field in MAC PDU of R2D message with NAS information of variable size.

Proposal 12	Upon negative acknowledgement of command reply, the device expects to retransmit not re-access.

Proposal 13	For CBRA, RN16 is used in the R2D message for AS ID assignment for addressing purpose.

Proposal 14	For CBRA, from a device’s perspective, AS ID is valid until completion of the inventory + command procedure (Option 4b).

Proposal 15	Device considers procedure completed upon reception of indication from reader for Msg1 resource allocation (e.g., QueryRep-like signaling).

Proposal 16	16 bits for AS ID size for both CFRA and CBRA.

Proposal 17	For multi-device CFRA, AS ID can be allocated if the use case necessitates. The AS ID can be allocated based on Option 4, i.e., via Msg2 (command request) without RN16 selected by device in Msg1.

Proposal 18	RAN2 send an LS to SA2 asking clarifications on following aspects:

(a)	Whether the NAS container for command request is sent as part of the service request or after device identifier verification.

(b)	Whether CN can command device(s) multiple times with one service request.

(c)	Any assistance information in service request indicating single or multiple command rounds.