3GPP TSG-RAN WG2 Meeting #129bis	R2-2502317

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.12.2

Source:	Nokia

Title:	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

MIMO is one of the key technologies in NR systems, and Rel-19, that started with RAN1#116, will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID RP-242394.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this document, we present Nokia’s views regarding requirements that RAN2 need to discuss in relation to the WID, in particular about:

- Objective 5: Asymmetric DL sTRP/UL mTRP deployment scenarios.

# 1	Asymmetric DL sTRP/UL mTRP

Figure 1 shows examples of the asymmetric DL sTRP / UL mTRP scenario. In such scenario, the DL transmission is from only a single TRP, depicted in the figure as “DL/UL capable TRP”. When using this feature, one or more “UL-only TRPs” may be deployed, potentially close to the cell edge in order to improve coverage. A UE may be scheduled for UL transmissions toward more than one TRP. UL transmissions are scheduled by the DL/UL capable TRP, which is the only TRP sending data and control information to each UE. Moreover, there is no restriction for a certain UE on which TRPs the UE is scheduled for UL transmissions, as we may have with reference to the figure below:

- UE1/UE2 transmitting toward an UL-only TRP and the DL/UL capable TRP,

- UE3 transmitting only toward the DL/UL capable TRP (legacy UE),

- UE4 transmitting only to an UL-only TRP,

- UEs (not depicted in the figure) transmitting to even more than one UL-only TRP.

Figure 1. Asymmetric DL sTRP/UL mTRP scenario

An UL-only TRP exclusively receives signals and does not transmit any DL signals. In the absence of DL pathloss reference signal (PL-RS) transmission from an UL-only TRP, estimating the pathloss to the UL-only TRP within the current NR framework is not feasible for determining the UL transmit power needed for transmission to the UL-only TRP. However, a UE can estimate the pathloss to the UL-only TRP by using the pathloss of the Anchor TRP along with a pathloss offset specified by the NW. The NW indicates an UL RSRP difference as a pathloss (PL) offset to assist the UE in estimating the pathloss between itself and the UL-only TRP. This specified PL offset is standard and typically applies to power control for all UL signals/channels (e.g., PUCCH, PUSCH, SRS, PRACH). During the RAN1#116 meeting, it was agreed that the initial PL offset would be conveyed to the UE through RRC signaling. Prior to establishing a connection with the UL-only TRP, the UL-only TRP may lack reliable uplink measurements, thus the NW can only provide the UE with an initial PL offset, ensuring a significant reduction in transmit power towards the UL-only TRP to mitigate potential interference. The UE utilizes this initial PL offset to adjust its initial transmission power to the UL-only TRP. After the first UL signal is received from the UE, the NW can precisely estimate the actual PL offset between the UL-only TRP and the Anchor TRP and update the PL offset through a MAC Control Element (CE) message. The UE should then revise the initially incorrect RRC-configured PL offset according to the updated, more accurate offset received from the NW measurement.

In the RAN2#129 the following agreement is made:

One PL offset value is indicated for each TCI state included in the new MAC CE. The new MAC CE contains one serving cell ID and one BWP ID TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) The new MAC CE can include flexible number of PL offset values. - Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In the last meeting,  RAN2 confirmed  that PL offset has no impact on PHR triggering condition as the PL offset is applied in the  PHY during the calculation of PL value. On the FFS part on whether to capture this, we propose to add some clarification in MAC spec.

Proposal 1: For PHR trigger, add a clarification in MAC specification: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated with the joint/UL TCI, where the PL change takes into account the PL offset.

At RAN2#128 and RAN2#129, companies raised a discussion point regarding which PL offset the UE should apply when multiple, consecutive RRC or MAC CE indications contain a PL offset. Specifically, concerns were raised about potential conflicts or out-of-order indications and whether they might cause ambiguity in determining the correct PL offset to apply at Layer 1 . However, most companies assumed there was no issue and RAN2 went with the assumption that the UE would always apply the latest received PL offset configuration. This current agreement becomes problematic when the UE transitions from RRC_INACTIVE back to RRC_CONNECTED. During the resumption procedure, the UE first applies default L1 parameters and then restores the last RRC configuration. Resuming the connection using the last RRC-configured PL offset is problematic for two reasons: first, because the restored configuration could be out-of-date relative to a subsequent MAC CE-indicated PL offset, and second, even in cases where the network never sent an updated PL offset via MAC CE, the UE could have moved elsewhere in the cell while in RRC_INACTIVE and the old (RRC-configured) PL offset configuration could still be out-of-date.

Observation 1: Following resumption from RRC_INACTIVE, the applicable PL offset configuration could be ambiguous or out-of-date.

Proposal 2: Network should indicate the applicable PL offset configuration to the UE during the RRC resume procedure.

When a UE goes through a RACH process and receives the random-access response from the serving gNB it has to apply the power ramping procedure based on pathloss, RRC indicated powerRampingStep, and other parameters specified in TS 38.213 for the PUSCH and PUCCH power control. In the asymmetric DL sTRP and UL mTRP deployment with pathloss offsets, the impacts on the power control of the PUSCH and PUCCH transmissions after the successful transmission of the RACH preamble (message 3 or message A) needs to be studied.

Proposal 3: Discuss  the impact of PL offsets on UL transmissions of RACH procedures, such as PRACH, Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

In the Random Access (RA) procedure, the UE measures  the RSRP of the downlink PL-RS and compares it with an RSRP threshold for RA resource selection. In an asymmetric downlink/uplink scenario, the UE may measure RSRP from a downlink TRP and transmit PRACH to an uplink-only TRP. Due to a PL offset, the measured RSRP from a downlink TRP might differ from the actual RSRP of the uplink-only TRP used in the RSRP comparison. This offset could influence the RSRP comparison involved in RA resource selection.

Proposal 4: If RAN2 determines that PL offset impacts the RACH procedures, then RAN2 should examine whether the PL offset affects the RSRP comparison in RA resource selection.

# 5	Conclusion

This document has made the following observations:

Observation 1: Following resumption from RRC_INACTIVE, the applicable PL offset configuration could be ambiguous or out-of-date.

And proposed the following:

Proposal 1: For PHR trigger, add a clarification in MAC specification: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated with the joint/UL TCI, where the PL change takes into account the PL offset.

Proposal 2: Network should indicate the applicable PL offset configuration to the UE during the RRC resume procedure.

Proposal 3: Discuss  the impact of PL offsets on UL transmissions of RACH procedures, such as PRACH, Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

Proposal 4: If RAN2 determines that PL offset impacts the RACH procedures, then RAN2 should examine whether the PL offset affects the RSRP comparison in RA resource selection.