3GPP TSG-RAN WG2 Meeting #129bis	R2-2502333

# Wuhan, China, April 7th – 11th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1909 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing UE capability for A4 A5 ReportOnLeave |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_Aerial-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In LTE RRC spec, A4/A5 ReportOnLeave has been supported since Rel-15, and new RRC parameter (i.e. a4-a5-ReportOnLeave-r15) was introduced in ReportConfigEUTRA. However, no explicit UE capability has been introduced for this, this causes problem in implementation at both network side and UE side. For example, it is unclear whether all Rel-15 version E-UTRAN UEs should by default support A4/A5 ReportOnLeave or not. Although this configuration was introduced under LTE UAV WI, this feature can also be applicable to other device types. Considering this feature was introduced long time ago, to reduce the rquirements for UE implementation, also to accomodate different UE implenentations (if we have) in the field, it is proposed to define the feature as “Optional feature without UE capability signalling”. This CR is provided to capture the new feature in TS 38.306. |  |  |  |  |  |  |  |  |
|  |  | “ |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add “A4/A5 ReportOnLeave” to optional feature without UE radio access capability parameters. Impact analysis Impacted 5G architecture options: LTE SA, (NG)EN-DC, NE-DC Impacted functionality: A4/A5 based L3 measurement Inter-operability: - If the UE is implemented according to the CR and the network is not, the network is unclear in which case a4-a5-ReportOnLeave can be configured, but inter-operability issue won’t happen if the network configures it blindly. - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Missing UE capability for A4/A5 ReportOnLeave configuration and reporting. From UE perspective, it is unclear whether all Rel-15 should mandatory support this; from network perspective, it is unclear in which cases the network can configure a4-a5-ReportOnLeave-r15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

6	Optional features without UE radio access capability parameters

The following clauses list the optional UE features not having UE radio access capability.

NOTE:	This clause does not yet contain complete analysis of all features of this release of specification.

***ignore non-related part***

6.8	Other features

***ignore non-related part***

6.8.x	A4/A5 ReportOnLeave

It is optional for UE to support the measurement configuration of a4-a5-ReportOnLeave-r15, if supported, the UE shall initiate measurement reporting procedure when the leaving condition is met for A4 or A5 event as specified in TS 36.331 [5].