3GPP TSG-RAN WG2 Meeting #129bis	R2-2502354

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.8.4

Source:		Lenovo

Title:		Further considerations on ETWS support in NR NTN

Document for:		Discussion

- Introduction

The latest RAN2#129 meeting made the following agreements regarding emergency broadcast in NR NTN:

Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In this contribution we further discuss the signaling details of “warning area coordinates” to be introduced for ETWS in NR NTN.

- Discussion

In previous releases before Rel-19 the PWS mechanisms are designed for TN, and UE can simply judge whether it is in an emergency broadcast service area by its serving cell and the corresponding PWS broadcast configuration it receives. This mechanism is workable based on the assumption that a PWS broadcast service area is larger than a TN cell coverage so that the service area can be mapped to one or more TN cells.

However, the diameter of an NTN cell could be much larger than an emergency broadcast service area. An NTN cell coverage may also across country border on one side of which an emergency broadcast service is not allowed or authorized. It is observed that, for ETWS there is no IE to describe a geographical area in the contents of legacy NR SIB6 and SIB7, while for CMAS as an optional IE warningAreaCoordinatesSegment was introduced in SIB8 to describe the alert area coordinates of a CMAS message.

Regarding the area indication for ETWS in NR NTN, there were discussions in RAN2#129, a straight-forward way is to reuse the IE warningAreaCoordinatesSegment in SIB8 for CMAS, with its formats following ATIS-0700041:

ATIS-0700041 5.2 Warning Area Coordinates Information Element This information element conveys the description of Warning Area Geometries specified by the Alert Originator to the mobile device to perform geo-fencing on the alert. These elements are coded as a finite bit string. In the figures, the bit string is described by octets from top downward, and in the octet from left to right. Number encoding strings start with the most significant bit. The total number of all circles and polygons is limited to 10. The total number of coordinates of all circles and polygons is limited to 100. 5.2.1 WAC Tag-Length-Value (TLV) At the most basic level the WAC IE is made up of one or more TLVs indicating specific parameters and the values of the parameters. Each WAC TLV is illustrated in Table 7. Tag (4 bits): Indicates the type of parameter Length (10 bits): Indicates the length of the entire WAC TLV in octets Reserved (2 bits): Shall be set to 0 by the network and ignored by the device Value (variable number of octets, up to 1022): The value of the parameter 5.2.2 WAC TLV characteristics The following attributes apply to the WAC TLV in the WAC IE: • WAC may contain multiple TLVs • There may be more than one WAC TLV with the same tag unless otherwise specified • There is no ordering significance to WAC TLVs in a WAC IE • If a WAC tag is not recognized by a device the WAC TLV shall be ignored • WAC TLVs shall be padded with binary 0s to octet boundaries …

By reviewing the ATIS-0700041, the following observations are given:

Observation 1: The IE warningAreaCoordinatesSegment for CMAS can describe circles and polygons.

Observation 2: The IE warningAreaCoordinatesSegment has a total number limit 10 for all circles and polygons, and a total number limit 100 for coordinates of all circles and polygons.

Observation 3: When describing circles, in the IE warningAreaCoordinatesSegment the granularity of radius is 1km and the value range is 64km.

Meanwhile for comparation, the granularity of radius introduced in NR NTN is 10km and the value range is 2560km.

Proposal 1: For ETWS broadcast service area indication, RAN2 can discuss whether the format of warningAreaCoordinatesSegment for CMAS is sufficient for NTN deployment or NR NTN solution (e.g. MBS broadcast service area indication) is also considered.

Based on the ATIS-0700041, it is further observed that in addition to circles and polygons, warningAreaCoordinatesSegment can also be used to indicate the Geo-Fencing Maximum Wait Time (when the tag bits are set to 0001):

ATIS-0700041 … 5.2.3 WAC tag coding Table 8 identifies the WAC TLV tags defined for WEA. … 5.2.8 WAC Geo-Fencing Maximum Wait Time This WAC TLV describes the Geo-Fencing Maximum Wait Time a device shall allow to determine its position meeting operator policy. If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert. This WAC TLV is optional and when included shall only occur once in a WAC IE. If it is not included, the device shall assume the Geo-Fencing Maximum Wait Time value 255 described in Table 12. … Table 12 identifies the Geo-Fencing Maximum Wait Time values defined for WEA. The values have the following meaning: 0: Use existing device position meeting operator policy on position age. If existing device position does not meet that operator policy, then present the alert rather than initiating a new positioning determination. 1-254: Use the provided value for the maximum wait time to establish a device position meeting operator policy on position age. If a position meeting the operator policy is not established within the wait time, then present the alert to the user and discontinue further position determination. 255: Use the device default or implementation Geo-Fencing Maximum Wait Time to establish a device position meeting operator policy on position age. At the end of this time, if a position meeting the operator policy is not established, then present the alert to the user and discontinue further position determination.

Observation 4: The warningAreaCoordinatesSegment can be used to indicate Geo-Fencing Maximum Wait Time, which is the time a device shall allow to determine its position meeting operator policy.

Observation 5: If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert.

Based on the description in ATIS-0700041, our understanding is that reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time triggers a positioning procedure, and the positioning procedure is expected to success within the indicated time or it will be terminated. Meanwhile the alert seems to be for upper layers which is out of RAN scope or even 3GPP scope.

From RAN2 perspective, the following issues can be further discussed:

- Whether reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time always triggers a positioning or a current UE position (e.g. for UL synchronization in NTN) can be reused.

- Whether the TA at UE for UL synchronization in NTN needs to be updated based on the positioning result triggered by warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time.

Proposal 2: If warningAreaCoordinatesSegment is used to indicate the Geo-Fencing Maximum Wait Time, RAN2 to decide whether to discuss its impact on positioning triggering and TA update in NR NTN or leave it to UE implementation.

As per TS38.413, the legacy PWS mechanism between gNB and AMF includes the following PWS procedures:

- Write-Replace Warning: to start or overwrite the broadcasting of warning messages, including Request procedure from AMF and Response procedure from gNB.

- Kill: to cancel an already ongoing broadcast of a warning message, including Request procedure from AMF and Response procedure from gNB.

- PWS Restart Indication: to inform the AMF that PWS information for some or all cells of the eNB are available for reloading.

- PWS Failure Indication: to inform the AMF that ongoing PWS operation for one or more cells of the eNB has failed.

It is observed that, the Warning Area Coordinates IE can only be included in Write-Replace Warning Request procedure, while for other procedures, the IEs Emergency Area ID, Broadcast Completed Area and Broadcast Cancelled Area used are represented by cell ID E-CGI. It means that except for Write-Replace Warning Request, the legacy PWS procedures between gNB and AMF can only indicate cell-level PWS information.

Observation 6: Except for Write-Replace Warning Request, the legacy PWS procedures between gNB and AMF can only indicate cell-level PWS information.

For emergency broadcast service in a geographical area smaller than an NTN cell, emergency broadcast service area information may also be needed for PWS procedures other than Write-Replace Warning Request, e.g.:

- In Kill Request from AMF to gNB: the information is used to indicate in which area an already ongoing broadcast of a warning message need to be cancelled.

- In Kill Response from gNB to AMF: the information is used to indicate in which area the broadcast of a warning message is cancelled.

- In PWS Restart Indication from gNB to AMF: the information is used to inform the AMF that PWS information for one or more areas under gNB coverage are available for reloading.

- In PWS Failure Indication from gNB to AMF: the information is used to inform the AMF that ongoing PWS operation for one or more areas under gNB coverage has failed.

Proposal 3: For emergency broadcast service in a geographical area smaller than an NTN cell, RAN2 to discuss whether emergency broadcast service area information is needed for PWS procedures other than Write-Replace Warning Request. (Send LS to RAN3 if needed)

- Conclusion

In this contribution we discuss the signaling details of “warning area coordinates” to be introduced for ETWS in NR NTN. The following observations are given:

Observation 1: The IE warningAreaCoordinatesSegment for CMAS can describe circles and polygons.

Observation 2: The IE warningAreaCoordinatesSegment has a total number limit 10 for all circles and polygons, and a total number limit 100 for coordinates of all circles and polygons.

Observation 3: When describing circles, in the IE warningAreaCoordinatesSegment the granularity of radius is 1km and the value range is 64km.

Observation 4: The warningAreaCoordinatesSegment can be used to indicate Geo-Fencing Maximum Wait Time, which is the time a device shall allow to determine its position meeting operator policy.

Observation 5: If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert.

Observation 6: Except for Write-Replace Warning Request, the legacy PWS procedures between gNB and AMF can only indicate cell-level PWS information.

And it is proposed that:

Proposal 1: For ETWS broadcast service area indication, RAN2 can discuss whether the format of warningAreaCoordinatesSegment for CMAS is sufficient for NTN deployment or NR NTN solution (e.g. MBS broadcast service area indication) is also considered.

Proposal 2: If warningAreaCoordinatesSegment is used to indicate the Geo-Fencing Maximum Wait Time, RAN2 to decide whether to discuss its impact on positioning triggering and TA update in NR NTN or leave it to UE implementation.

Proposal 3: For emergency broadcast service in a geographical area smaller than an NTN cell, RAN2 to discuss whether emergency broadcast service area information is needed for PWS procedures other than Write-Replace Warning Request. (Send LS to RAN3 if needed)