3GPP TSG-RAN WG2 Meeting #129bis	R2-2502357

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.9.4

Source:		Lenovo

Title:		Further considerations on PWS broadcast support in IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements to support PWS broadcast for NB-IoT UEs in NTN:

RAN2#127bis agreements: 1. RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS 2. The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. 3. For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields): - SystemInformationBlockType10-NB for primary ETWS notification; - SystemInformationBlockType11-NB for secondary ETWS notification; - SystemInformationBlockType12-NB for CMAS notification. 4. Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB): - etws-Indication; - cmas-Indication. 5. Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means) 6. Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases) RAN2#128 agreements: 1. We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. 2. We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed 3. Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication. RAN2#129 agreements: 1. Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells 2. Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information. 3. Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB 4. We don’t introduce a new SIB for carrying geographical information for emergency areas

In this contribution we discuss further issues and potential solutions to support PWS broadcast for NB-IoT in NTN.

- Discussion

In previous releases before Rel-19 the PWS mechanisms are designed for TN, and UE can simply judge whether it is in an emergency broadcast service area by its serving cell and the corresponding PWS broadcast configuration it receives. This mechanism is workable based on the assumption that a PWS broadcast service area is larger than a TN cell coverage so that the service area can be mapped to one or more TN cells.

However, the diameter of an NTN cell could be much larger than an emergency broadcast service area. An NTN cell coverage may also across country border on one side of which an emergency broadcast service is not allowed or authorized. It is observed that, for ETWS there is no IE to describe a geographical area in the contents of legacy NR SIB10 and SIB11, while for CMAS as an optional IE warningAreaCoordinatesSegment was introduced in SIB12 to describe the alert area coordinates of a CMAS message.

## Area information indicating Geo-Fencing Maximum Wait Time

Regarding the area indication for ETWS in NR NTN, in RAN2#129 it was agreed to reuse the IE warningAreaCoordinatesSegment in SIB12 for CMAS, to be included in SIB10-NB/11-NB, with its formats following ATIS-0700041. Based on the ATIS-0700041, it is further observed that in addition to circles and polygons, warningAreaCoordinatesSegment can also be used to indicate the Geo-Fencing Maximum Wait Time (when the tag bits are set to 0001):

ATIS-0700041 … 5.2.3 WAC tag coding Table 8 identifies the WAC TLV tags defined for WEA. … 5.2.8 WAC Geo-Fencing Maximum Wait Time This WAC TLV describes the Geo-Fencing Maximum Wait Time a device shall allow to determine its position meeting operator policy. If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert. This WAC TLV is optional and when included shall only occur once in a WAC IE. If it is not included, the device shall assume the Geo-Fencing Maximum Wait Time value 255 described in Table 12. … Table 12 identifies the Geo-Fencing Maximum Wait Time values defined for WEA. The values have the following meaning: 0: Use existing device position meeting operator policy on position age. If existing device position does not meet that operator policy, then present the alert rather than initiating a new positioning determination. 1-254: Use the provided value for the maximum wait time to establish a device position meeting operator policy on position age. If a position meeting the operator policy is not established within the wait time, then present the alert to the user and discontinue further position determination. 255: Use the device default or implementation Geo-Fencing Maximum Wait Time to establish a device position meeting operator policy on position age. At the end of this time, if a position meeting the operator policy is not established, then present the alert to the user and discontinue further position determination.

Observation 1: The warningAreaCoordinatesSegment can be used to indicate Geo-Fencing Maximum Wait Time, which is the time a device shall allow to determine its position meeting operator policy.

Observation 2: If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert.

Based on the description in ATIS-0700041, our understanding is that reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time triggers a positioning procedure, and the positioning procedure is expected to success within the indicated time or it will be terminated. Meanwhile the alert seems to be for upper layers which is out of RAN scope or even 3GPP scope.

From RAN2 perspective, considering that the schemes introduced in Rel-18 IoT NTN for GNSS positioning (GNSs positioning triggering and reporting), there could be some impacts or conflicts between GNSS positioning for UL synchronization and the positioning triggered by warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time.

For the positioning procedure triggering, whether the reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time always triggers a new positioning procedure, or an on-going GNSS positioning for UL synchronization in NTN can fulfill the requirement (obtaining UE positioning within the Geo-Fencing Maximum Wait Time) as well. On the opposite, if a GNSS positioning is triggered by reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time, whether its result can be used for UL synchronization in NTN.

Proposal 1: RAN2 to discuss whether the outcome of GNSS positioning for UL synchronization in NTN at UE can be used to fulfill the requirement of the positioning from reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time.

Proposal 2: RAN2 to discuss whether the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN.

For Proposal 2 if agreeable, a followed question is whether UE shall trigger report of gnss-ValidityDuration.

Proposal 2-bis: If the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN, RAN2 to discuss whether UE shall trigger report of gnss-ValidityDuration.

In the GNSS positioning for UL synchronization at UE, there is a parameter gnss-PositionFixDuration reported by UE to indicate the time duration required for GNSS positioning. If the Geo-Fencing Maximum Wait Time indicated in warningAreaCoordinatesSegment is shorter than the gnss-PositionFixDuration, whether UE shall stop triggering GNSS positioning, or attempt for other positioning method, or present the alert to the user as described in ATIS-0700041 immediately.

Proposal 3: RAN2 to discuss if the Geo-Fencing Maximum Wait Time indicated in warningAreaCoordinatesSegment is shorter than the gnss-PositionFixDuration, whether UE shall stop triggering GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time. The UE may still attempt for other positioning methods.

## Area information between eNB and MME

As per TS36.413, the legacy PWS mechanism between eNB and MME includes the following PWS procedures:

- Write-Replace Warning: to start or overwrite the broadcasting of warning messages, including Request procedure from MME and Response procedure from eNB.

- Kill: to cancel an already ongoing broadcast of a warning message, including Request procedure from MME and Response procedure from eNB.

- PWS Restart Indication: to inform the MME that PWS information for some or all cells of the eNB are available for reloading.

- PWS Failure Indication: to inform the MME that ongoing PWS operation for one or more cells of the eNB has failed.

It is observed that, the Warning Area Coordinates IE can only be included in Write-Replace Warning Request procedure, while for other procedures, the IEs Emergency Area ID, Broadcast Completed Area and Broadcast Cancelled Area used are represented by cell ID E-CGI. It means that except for Write-Replace Warning Request, the legacy PWS procedures between eNB and MME can only indicate cell-level PWS information.

Observation 3: Except for Write-Replace Warning Request, the legacy PWS procedures between eNB and MME can only indicate cell-level PWS information.

For emergency broadcast service in a geographical area smaller than an NTN cell, emergency broadcast service area information may also be needed for PWS procedures other than Write-Replace Warning Request, e.g.:

- In Kill Request from MME to eNB: the information is used to indicate in which area an already ongoing broadcast of a warning message need to be cancelled.

- In Kill Response from eNB to MME: the information is used to indicate in which area the broadcast of a warning message is cancelled.

- In PWS Restart Indication from eNB to MME: the information is used to inform the MME that PWS information for one or more areas under eNB coverage are available for reloading.

- In PWS Failure Indication from eNB to MME: the information is used to inform the MME that ongoing PWS operation for one or more areas under eNB coverage has failed.

Proposal 4: For emergency broadcast service in a geographical area smaller than an NTN cell, RAN2 to discuss whether emergency broadcast service area information is needed for PWS procedures other than Write-Replace Warning Request. (Send LS to RAN3 if needed)

## Association of the PWS broadcast service area

Another issue for the PWS broadcast service area indication is the expected PWS operation related to the indicated area. In legacy PWS mechanisms there is no area indication in the paging message, meaning that UE in the cell will be notified to receive PWS SIBs anyway. If PWS broadcast service area indication is supported for NB-IoT UEs, the following options for area-based PWS broadcast service operation can be considered:

- Option 1: UE is paged with area indication, and determines whether to acquire the PWS SIBs based on whether it is in the indicated area.

- Option 2: UE is paged without area indication (as in legacy), and acquires the PWS SIBs including area indication to decide whether to process (e.g., decode and deliver to upper layers) the PWS contents in the SIBs.

Either option can facilitate area-based PWS broadcast service delivery to the UE and the difference is that whether UE needs to receive the PWS SIBs first.

Observation 4: The PWS broadcast service area indication can be associated to the paging indication or PWS contents in PWS SIBs to facilitate area-based delivery to NB-IoT UEs.

Proposal 5: RAN2 to discuss the association between PWS broadcast service area and paging indication or PWS contents in PWS SIBs.

- Conclusion

In this contribution we discuss further issues and potential solutions to support PWS broadcast for NB-IoT in NTN. The following observations are given:

Observation 1: The warningAreaCoordinatesSegment can be used to indicate Geo-Fencing Maximum Wait Time, which is the time a device shall allow to determine its position meeting operator policy.

Observation 2: If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert.

Observation 3: Except for Write-Replace Warning Request, the legacy PWS procedures between eNB and MME can only indicate cell-level PWS information.

Observation 4: The PWS broadcast service area indication can be associated to the paging indication or PWS contents in PWS SIBs to facilitate area-based delivery to NB-IoT UEs.

And we propose:

Proposal 1: RAN2 to discuss whether the outcome of GNSS positioning for UL synchronization in NTN at UE can be used to fulfill the requirement of the positioning from reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time.

Proposal 2: RAN2 to discuss whether the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN.

Proposal 2-bis: If the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN, RAN2 to discuss whether UE shall trigger report of gnss-ValidityDuration.

Proposal 3: RAN2 to discuss if the Geo-Fencing Maximum Wait Time indicated in warningAreaCoordinatesSegment is shorter than the gnss-PositionFixDuration, whether UE shall stop triggering GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time. The UE may still attempt for other positioning methods.

Proposal 4: For emergency broadcast service in a geographical area smaller than an NTN cell, RAN2 to discuss whether emergency broadcast service area information is needed for PWS procedures other than Write-Replace Warning Request. (Send LS to RAN3 if needed)

Proposal 5: RAN2 to discuss the association between PWS broadcast service area and paging indication or PWS contents in PWS SIBs.