3GPP TSG-RAN WG2 #129bis	R2-2502366

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.1.2.2

Source:		Lenovo

Title:	Left issues related to applicability report for AIML based BM

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss left issues related to LCM for AIML based beam management.

# Discussion

## Option B as superset of Option A for applicability report

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

As replied in RAN1 LS, RAN1 considers the following two approaches for gNB to provide the applicable inference configuration, which can be summarized as following in our understanding

Option A): in step 3, via RRCReconfiguration, gNB provides one or more of CSI-ReportConfig for inference configuration; in step 4, via RRCReconfigurationComplete, UE indicates the applicable CSI-ReportConfig; after step 4,

  - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW.

  - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Option B): in step 3, via RRCReconfigurationàOtherConfig, gNB provides candidate sets of inference related parameters; in step 4, via UAI, UE indicates the applicable ones among the candidate sets of parameters; in step 5, accordingly, via RRCReconfiguration, gNB will generate and provide the CSI-ReportConfig for inference configuration.

One thing that worth clarification/discussion is that in Option B, after step 5 the full inference configuration is provided to UE, will UE also indicate the applicability/inapplicability to gNB as in Option A, e.g.,

  - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW.

  - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

As reflected in the Note from RAN1 agreements below, the UE is expected to be only configured with applicable full inference configurations based on applicable set of inference parameters. In that case, the full inference configurations in RRCReconfiguration (step 5) can be assumed to be already applicable. Thus, the UE may not be required to report any further inapplicability in RRCReconfigurationComplete (step 6). While this can be a simple approach and similar to legacy procedure, we think this is an idealistic scenario for AI/ML functionality. As per RAN1 discussion and agreements, in Option B, applicability is determined solely based on the set of inference parameters such that the applicable status is not subject to change (until RRCReconfiguration) and the UE can be configured with full inference configuration for the reported applicable functionalities in Step 4.

In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig.

On the other hand, RAN2 has formed the following understanding that the applicability is determined by UE based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. In that sense, in Option B even if gNB provides the full inference configuration according to the applicable parameters, there is still a chance that the full inference configuration in Option B becomes inapplicable and thus the indication/update of applicability/inapplicability after receiving full inference configuration is needed in Option B similar as in Option A (as illustrated in the figure below).

RAN2#127: UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

RAN2 is suggested to clarify that in Option B, after UE receives the full inference configuration, will UE indicates/updates the applicability/inapplicability of configured inference configuration as in Option A (i.e., in RRCReconfigurationComplete and UAI).

If the answer to Proposal 1 is yes, comparing Option A) and Option B), the fundamental difference seems to be in Option B) gNB will receive the applicable parameters (in Option B step 3-4) before providing the inference configuration to UE. The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Option A) and B). Even though in Option B) UE is not expected to be configured with a non-applicable CSI-ReportConfig for inference, it may not be reflected in the specification as it is eventually upon gNB implementation how CSI-ReportConfig for inference is generated.

Comparing Option A) and Option B), the fundamental difference seems to be in Option B) gNB will receive the applicable parameters (in Option B step 3-4) before providing the inference configuration to UE.

The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Option A) and B).

Considering above analysis, a unified procedure (combining approach A and B) can be supported from specification point of view:

Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

Step 4: (Optional) UE reports in UAI/RRCReconfigurationComplete the applicability of the sets of parameters for inference configuration

Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

Step 8: For the subsequent change of applicability related to either inference configuration, or parameters for inference configuration, UE will indicate to gNB in UAI.

- Note that since the applicability/inapplicability of inference configuration and applicability/inapplicability of parameters for inference configuration serve for different purposes, it is possible that in the same UAI message UE will indicate both

  - Applicability/inapplicability of parameters for inference configuration, which was previously provided in OtherConfig, and

  - Applicability/inapplicability of full inference configuration, which was provided in RRCReconfiugration message.

It’s worth noting that step 3 and step 4 can be beneficial for gNB to reduce the possibility of providing a non-applicable inference configuration.

For applicability report, Option A and B are not mutual exclusive. Option B can be regarded as a superset of Option A.

If Proposal 1 is agreed, for applicability report, RAN2 is suggested to support a unified solution with Option B as superset of Option A:

  - Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

  - Step 4: (Optional) UE reports in UAI/RRCReconfigurationComplete the applicability of the sets of parameters for inference configuration

  - Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

  - Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

  - Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

  - Step 8: For the subsequent change of applicability related to either inference configuration or parameters for inference configuration, UE will indicate to gNB in UAI.

## Prohibit timer of UAI for applicability report

Another aspect related to the UAI configuration for applicability related info report is the concept of prohibit timer. In the legacy, prohibit timer is usually configured to avoid frequent report of UAI. Then the question is whether the concept of prohibit timer is needed for the applicability related info report as well, e.g., after the UAI is triggered in the following two cases, should UE starts a prohibit timer in case:

- Case#1: UAI triggered due to a non-applicable UE sided AI functionality becoming applicable

- Case#2: UAI triggered due to an applicable UE sided AI functionality becoming now non-applicable

In case#1, after sending the UAI, the UE sided AI functionality could be activated by NW. Then it is quite critical to not put any restriction (e.g. prohibit timer) on the next UAI report for UE to timely notify gNB about the non-applicability at any time to deactivate the UE sided AI functionality before too late. In this sense, it would be better not to restrict the report of non-applicability using a prohibit timer.

In case#2, after sending the UAI, the UE sided AI functionality could be deactivated and UE may use another UE sided AI functionality or fallback to non-AIML method. A prohibit timer can be used in this case to avoid UE immediately sending a new UAI indicating the deactivated AI functionality now becomes applicable again, which may lead to frequent or unnecessary UE sided AI functionality activation/deactivation. And it does not harm if the UE continues using the non-AIML method for a while.

UE can send UAI indicating a functionality/CSI-ReportConfig becomes non-applicable at any time without restriction (e.g., prohibit timer).

UE can be configured with a prohibit timer that after UE reports via UAI that a functionality/CSI-ReportConfig becomes non-applicable, UE will not send another UAI message for the same functionality/CSI-ReportConfig again within a time period.

## Applicability report in CA

1> if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations related to radio measurement predictions according to 5.7.4.2: 2> include applicabilityAssistanceList in this UEAssistanceInformation message; 2> for each serving cell configured with at least one CSI-ReportConfig configured to be used for measurement predictions (i.e. including the resourcesToBeMeasuredForChannelPrediction) for which the applicability information has changed: 3> include an entry in applicabilityAssistanceList in the UEAssistanceInformation message, and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell ; 4> set the applicableReportConfigIdList to the list of CSI-ReportConfigId including the resourcesToBeMeasuredForChannelPrediction determined by the UE to have changed from non-applicable to applicable; 4> set the nonApplicableReportConfigIdList to the list of CSI-ReportConfigId including the resourcesToBeMeasuredForChannelPrediction determined by the UE to have changed from applicable to non-applicable ; Editor's Note: FFS if applicability reporting is supported for multiple serving cells and applicabilityCellId is needed to unambiguously identify CSI report configurations for prediction. Editor's Note: FFS option B (sets of inference related parameters).

Another open issue is if the AIML based BM use case can be configured for CA scenario. In particular, if UE is configured with multiple SCells, within each SCell, can UE be configured to perform beam prediction based on Set A and Set B beams within the same SCell?

In our understanding, intra-frequency spatial/time domain beam prediction is in general supported, thus, as long as the Set A and Set B beams are within the same SCell, the so far discussed LCM procedure can be applied as well. In contrast, the beam prediction with Set A and Set B beams of different cells is not supported.

Beam prediction with Set A and Set B beams of the same frequency is supported.

RAN2 confirms the support of beam prediction in CA scenario, wherein UE can be configured to perform beam prediction with Set A and Set B beams of the same cell (PCell/SCell).

In the applicability report, cell information is needed to unambiguously identify CSI report configurations for prediction.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Comparing Option A) and Option B), the fundamental difference seems to be in Option B) gNB will receive the applicable parameters (in Option B step 3-4) before providing the inference configuration to UE.

Observation 2	The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Option A) and B).

Observation 3	For applicability report, Option A and B are not mutual exclusive. Option B can be regarded as a superset of Option A.

Observation 4	Beam prediction with Set A and Set B beams of the same frequency is supported.

Based on the discussion above, we propose:

Proposal 1	RAN2 is suggested to clarify that in Option B, after UE receives the full inference configuration, will UE indicates/updates the applicability/inapplicability of configured inference configuration as in Option A (i.e., in RRCReconfigurationComplete and UAI).

Proposal 2	If Proposal 1 is agreed, for applicability report, RAN2 is suggested to support a unified solution with Option B as superset of Option A:

a.	Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

b.	Step 4: (Optional) UE reports in UAI/RRCReconfigurationComplete the applicability of the sets of parameters for inference configuration

c.	Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

d.	Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

e.	Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

f.	Step 8: For the subsequent change of applicability related to either inference configuration or parameters for inference configuration, UE will indicate to gNB in UAI.

Proposal 3	UE can send UAI indicating a functionality/CSI-ReportConfig becomes non-applicable at any time without restriction (e.g., prohibit timer).

Proposal 4	UE can be configured with a prohibit timer that after UE reports via UAI that a functionality/CSI-ReportConfig becomes non-applicable, UE will not send another UAI message for the same functionality/CSI-ReportConfig again within a time period.

Proposal 5	RAN2 confirms the support of beam prediction in CA scenario, wherein UE can be configured to perform beam prediction with Set A and Set B beams of the same cell (PCell/SCell).

Proposal 6	In the applicability report, cell information is needed to unambiguously identify CSI report configurations for prediction.

# Reference