3GPP TSG-RAN WG2 #129bis	R2-2502367

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.1.2.3

Source:		Lenovo

Title:	LCM for AIML based positioning with UE-sided model

Document for:		Discussion and Decision

# Introduction

In this paper we discuss the 1st priority use case for LCM for AIML based positioning with UE-sided model, i.e., case 1.

- (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

# Discussion

## 2.1	Avoiding frequent applicability report

RAN2 Agreement UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

In the previous RAN2 meeting, it was discussed that UE can trigger a report of LPP provide capabilities message if there is a change of applicable functionality. However, there is a concern of too frequent applicability report if LMF does not provide additional control.

First of all, we believe it is critical to inform NW about the “inapplicability” of the AIML based positioning method, which can be indicated via either:

- The UL LPP ProvideLocationInformation message to LMF during a positioning session as an error cause, or

- The UL LPP ProvideCapabilities message.

Then, the concern of too frequent applicability report lies in the case that right after UE indicates LMF about the “inapplicability”, UE determines the AIML based positioning method becomes applicable again and trigger another applicability report immediately.

In our understanding, for the positioning use case, a UE sided model is usually trained for positioning within a area, of which the applicability/inapplicability is not expected to change frequently as concerned by some companies. On the other hand, if mechanism is really needed to avoid this issue, it could be straight forward to introduce the mechanism of prohibit timer. In particular, a prohibit timer can be configured to UE that, after UE indicating the AIML based positioning method is/becomes inapplicable, UE shall not trigger another report indicating the same method becomes applicable again within a time period.

It is critical to inform LMF about the inapplicability of AIML based positioning method timely.

Right after UE indicates LMF about the “inapplicability” of AIML based positioning, UE is not expected to (or should not) report that the same method becomes applicable again and trigger another applicability report immediately.

There is no additional LMF control for UE to report that a AIML based positioning method becomes inapplicable.

If any mechanism is needed to avoid UE reporting “applicability” right after “inapplicability” has been reported for the same AIML based positioning, a prohibit timer based approach can be considered.

## 2.2	Positioning accuracy monitoring

RAN1 Agreement Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

RAN1 also discussed the monitoring of AI-based positioning for case 1, and at least (Option A) target UE can perform the monitoring metric calculation and may transmit the outcome to the LMF. In our understanding, there are two possible scenarios:

- Option A initiated by UE: based on UE’s own demand, UE may perform monitoring by itself. UE may need LMF to provide related configuration or assistance information. UE will not send the monitoring outcome to LMF, instead UE may indicate to LMF if the AI-based positioning become non-applicable (e.g. via an Error cause)

  - This can be supported by LPP Request Assistance Data and Provide Assistance Data messages.

- Option A initiated by LMF: LMF configures UE to perform monitoring and send the monitoring outcome back to LMF.

  - This can be supported by LPP Request Location Information and Provide Location Information messages.

(Monitoring Option A initiated by UE) LPP RequestAssistanceData and ProvideAssistanceData messages are used for UE to request LMF to provide relevant configuration/data for monitoring metric calculation at UE without sending the outcome to LMF.

(Monitoring Option A initiated by LMF) LPP RequestLocationInformation and ProvideLocationInformation messages are used for LMF to configure the UE to calculate monitoring metric and send the outcome to LMF.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	It is critical to inform LMF about the inapplicability of AIML based positioning method timely.

Observation 2	Right after UE indicates LMF about the “inapplicability” of AIML based positioning, UE is not expected to (or should not) report that the same method becomes applicable again and trigger another applicability report immediately.

Based on the discussion above, we propose:

Proposal 1	There is no additional LMF control for UE to report that a AIML based positioning method becomes inapplicable.

Proposal 2	If any mechanism is needed to avoid UE reporting “applicability” right after “inapplicability” has been reported for the same AIML based positioning, a prohibit timer based approach can be considered.

Proposal 3	(Monitoring Option A initiated by UE) LPP RequestAssistanceData and ProvideAssistanceData messages are used for UE to request LMF to provide relevant configuration/data for monitoring metric calculation at UE without sending the outcome to LMF.

Proposal 4	(Monitoring Option A initiated by LMF) LPP RequestLocationInformation and ProvideLocationInformation messages are used for LMF to configure the UE to calculate monitoring metric and send the outcome to LMF.

# Reference