3GPP TSG-RAN-WG2 Meeting #129bis	R2-2502378

Wuhan, China, 7th– 11th Apr, 2025

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	discussion on Relay discovery and (re)selection for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#129 meeting, RAN2 reached following agreements [1]:

Agreements: - The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell. FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive. - From RAN2 perspective, when an intermediate/last relay UE serves two different downstream UEs, there is no assumption on whether it presents the same or different L2IDs for connection establishment. - Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay. - Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay. - For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient). - For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied. FFS if the PC5 link is not already established. - For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE. - For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition: - - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold. - - When U2N Remote UE receives a PC5-S link release message from current first relay UE. - - When U2N Remote UE detects PC5 RLF with the current first relay UE. - - Indicated by upper layer. - L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - - release of RRC connection between an intermediate relay UE and the network. - - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - - FFS: link quality degradation of the upstream links resulting in a notification. - - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. - - FFS applicability to L3. - The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. - Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification. - FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger) - FFS if the notification is required in all cases - FFS the content in the notification - FFS the child node behaviour after receiving this notification - FFS other condition/timing that the intermediate relay UE notifies its child node

In this paper, we discuss how to introduce multi-hop relay.

Discussion

  - Discovery related issues

For discovery transmission conditions, RAN2 reached following agreements:

- Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay.

- Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay.

- For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient).

- For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied.  FFS if the PC5 link is not already established.

RAN2 made above agreements based on the SA2’s conclusion (for model A, UE establishes PC5-RRC connection to a parent UE before transfer the discovery message. For model B, UE doesn’t establish PC5-RRC connection to a parent UE before transfer the response message). And companies assumed that the PC5 link quality is enough if there is a PC5-RRC connection. However, there is no condition to establish a PC5-RRC connection. So, remote UE may select a path with poor quality, which will likely cause multiple relay reselections, resulting in high communication delays and wasting a lot of radio resources. Relay reselection includes a RRC re-establishment request, so it has a large impact on the gNB. Therefore, we still think the link qualities between relay UEs should be evaluated.

Observation 1. If there is no evaluation of multi-hop relay link quality, remote UE cannot select an appropriate path, then it causes wasteful signalling between relay UEs and between UE and gNB due to multiple relay reselections.

For model A, we assume that there is a PC5 connection between relay UEs before discovery message forwarding. So, we can consider two ways to evaluate multi-hop relay link if it is needed.

- Opt1. An intermediate relay UE evaluates PC5 link quality before establishment of a PC5 connection.

- Opt2. A remote UE is notified the information that the link qualities of PC5 connections.

For model B, SA2 said that UE doesn’t establish PC5-RRC connection to a parent UE before transfer the response message and RAN2 has not decided whether there is AS layer criteria on discovery message forwarding are applied if the PC5 link is not already established. Here, we have an additional way for multi-hop relay link evaluation.

- Opt3. An intermediate relay UE doesn’t forward the discovery message if the SD-RSRP is below than a threshold.

To avoid complicating the specifications, it is desirable to have a unified solution for model A and model B. From that perspective, option 1 and option 3 are not appropriate. Because option 1is not applicable for model B if there is no establishment of a PC5 connection before response forwarding. And option 3 is not applicable for model A since RAN2 already agreed that there are no additional AS criteria on discovery message forwarding. therefore, we prefer option2 from spec perspective.

Proposal 1. To avoid performing wasteful relay reselections and complicating the specification, a remote UE is notified the information that the link qualities of PC5 connections while relay (re)selection procedure.

And it is FFS if there would be additional restrictions for the intermediate relay UE in the case of no upper bound.

Proposal 2. There are no additional restrictions to act as an intermediate relay UE in the case of no upper bound.

RAN2 already agreed that the multi-path topology should not be supported in multi-hop relay. In that case, remote UEs and intermediate relay UEs should use the SIBs broadcasted on the last relay UE’s serving cell. In such a case, to prevent mis-configuration due to multipath topology, the last relay UE's child UEs need to know the last relay UE's serving cell. In Rel-17 relay discovery, the RRC container is included in the discovery message, and the cell-related information is included in the RRC container. In multi-hop relay, based on the discovery message received from the last relay UE, the corresponding cell-related information needs to be included in the discovery message to be transmitted.

Proposal 3. When the UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

  - Relay (re)selection related issues

RAN2 have discussed on trigger of relay reselection, and there are some FFS as below:

- FFS whether the link quality degradation should be indicated in the notification.

In Rel-17/18 U2N/U2U relay, this mechanism was rejected since a remote UE should initiate relay reselection based on Uu/PC5 RLF indication. So, same principle can be reused for multi-hop relay. Furthermore, for path switching scenario, intermediate relay UE can be configured to report (sidelink) measurement result. Therefore, in multi-hop case, link quality degradation can be resolved by path switching procedure if UEs are in RRC_CONNECTED.

Proposal 4. The link quality degradation should not be indicated in the notification.

- FFS whether the conditions for L2 multi-hop relay can be applied L3 multi-hop relay.

We don’t see any issues to apply the condition of relay reselection to L3 multi-hop relay. However, it should be made clear that these conditions do not directly trigger relay reselection but are instead a notification to higher layers.

Proposal 5. The agreed trigger conditions for relay reselection can be applied to L3 multi-hop relay but should only be an indication to higher layers.

RAN2 agreed that an intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. But RAN2 hasn’t decided whether there is an upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. In the January meeting, i.e., SA2 Ad Hoc, SA2 agreed that intermediate UE can initiate relay reselection while keeping PC5-RRC connection with child UEs upon detecting PC5-unicast link failure [2]. In the CR, the intermediate relay UE is also acting as a remote UE. Therefore, there is no need to support a relay reselection procedure initiated by an intermediate relay UE separated from a relay reselection procedure initiated by a remote UE, but a relay reselection procedure initiated by a remote UE can be supported by an intermediate relay UE which is also acting as a remote UE from RAN2 perspective. Furthermore, in approach 1 C-plane procedure, intermediate relay UEs need to receive RRC reconfiguration messages for a remote UE configuration. In this case, intermediate relay UE has E2E RRC connection to the gNB and it should be in RRC_CONNECTED. The topology for these connections is exactly the same as for a (multi-hop) remote UE. Therefore, an intermediate relay UE should always act as a remote UE in approach 1.

Observation 2. SA2 agreed that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 6. RAN2 confirm that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 7. An intermediate relay UE always acts as a remote UE in approach 1.

In case that intermediate relay UE initiated relay reselection, child UEs should be able to re-establish an RRC connection via the same intermediate relay UE. RAN2 already agreed that intermediate relay UE sends an indication if it initiates relay reselection. If the cause value of the indication is legacy PC5-RLF used for U2U relay, a child UE couldn’t recognize that the parent UE performs relay reselection. If the intermediate relay UE performs relay reselection, child UEs should not perform relay reselection independently. Therefore, new cause value or a flag should be introduced.

Proposal 8. An intermediate relay UE should send an indication to a child UE with new cause value or new flag upon initiating relay reselection.

Furthermore, since transmissions during a link failure may result in packet loss, the child UEs should suspend UL transmission when the child UEs are indicated that a link failure has occurred and that the relay reselection is occurring. Similar mechanism was introduced for the case of MP relay handover.

Proposal 9. Child UEs suspend UL transmission to avoid packet loss when it is indicated the intermediate relay UE reselection.

Furthermore, a child node should send RRCReestablishmentRequest to NW when the relay reselection performed by the intermediate relay UE is completed. So, to initiate sending RRCReestablishmentRequest, the intermediate relay UE should indicate the relay reselection completion if the relay reselection performed by the intermediate relay UE is completed.

Proposal 10. An intermediate relay UE should indicate the completion of the relay reselection to a child UE.

Furthermore, we think these mechanisms can be adopted to intra-gNB path switching of intermediate relay UE with not much enhancement. It is reasonable that an intermediate relay UE performs path switching with child UEs as well as the case of relay reselection.

Proposal 11. A mechanism for an intermediate relay UE to initiate relay reselection while retaining a child UE can be adopted for path switching performed by the intermediate relay UE.

Conclusion

Observation 1. If there is no evaluation of multi-hop relay link quality, remote UE cannot select an appropriate path, then it causes wasteful signalling between relay UEs and between UE and gNB due to multiple relay reselections.

Proposal 1. To avoid performing wasteful relay reselections and complicating the specification, a remote UE is notified the information that the link qualities of PC5 connections while relay (re)selection procedure.

Proposal 2. There are no additional restrictions to act as an intermediate relay UE in the case of no upper bound.

Proposal 3. When the UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

Proposal 4. The link quality degradation should not be indicated in the notification.

Proposal 5. The agreed trigger conditions for relay reselection can be applied to L3 multi-hop relay but should only be an indication to higher layers.

Observation 2. SA2 agreed that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 6. RAN2 confirm that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 7. An intermediate relay UE always acts as a remote UE in approach 1.

Proposal 8. An intermediate relay UE should send an indication to a child UE with new cause value or new flag upon initiating relay reselection.

Proposal 9. Child UEs suspend UL transmission to avoid packet loss when it is indicated the intermediate relay UE reselection.

Proposal 10. An intermediate relay UE should indicate the completion of the relay reselection to a child UE.

Proposal 11. A mechanism for an intermediate relay UE to initiate relay reselection while retaining a child UE can be adopted for path switching performed by the intermediate relay UE.

Reference

- R2-2501334, Report from session on positioning and sidelink relay

- S2-2501292, Clarification on Multi-hop Relay Reselection (Huawei)