3GPP TSG-RAN-WG2 Meeting #129bis	R2-2502381

Wuhan, China, 7th– 11th April, 2025

Agenda item	:		8.6.2

Source	: 	Sharp

Title	: 	Discussion on issues for supporting inter-CU LTM

Document for	:	Discussion and Decision

Introduction

WID [1] for mobility enhancement was approved and RAN2 started discussion on the WI. In RAN2#129, RAN2 made agreements as follows [2];

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this paper, we discuss inter-CU LTM related issues.

Discussion

Inter-CU SCG LTM

RAN2 agreed that source SN initiates inter-CU SCG LTM preparation, source SN generates SCG part configuration and MN includes it into its MN RRC configuration message in RAN2#127.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

And, in RAN2#128, RAN2 assumes that source SN may send measurement results of candidate PSCells to the MN, the MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. These agreements and assumptions are captured in draft 37.340 CR discussion. Here, the above agreement is captured as “the candidate SN provides the SCG part configuration of each candidate PSCell” in 37.340 CR. We are ok with the thinking since it is wasteful if the agreement means that source SN receives LTM candidate configurations collected by MN, generates aggregated configuration based on the LTM candidate configurations and sends it to MN. It is reasonable that MN collects LTM candidate configurations, generates the aggregated configuration and indicates it to candidate SNs including source SN to support subsequent LTM.

Observation 1. The agreement made in RAN2#127 “The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message” means that “The candidate SN provides the SCG part configuration of each candidate PSCell.”

In the RRC CR discussion, Rapp suggested to use a new IE (ltm-ConfigNRDC) to indicate the SCG part configuration. We think there is no agreement to use a new IE for SCG LTM configuration. If RAN2 don’t introduce the new IE, LTM candidate configuration is merged in MCG ltm-Config. It means that the maximum number of LTM candidate settings for MCG and SCG will be 8 in total. 8 may be too small by considering both MCG and SCG in total. We think the maximum number should be 16 in total. So, we support to introduce a new IE for SCG LTM configuration.

Proposal 1. ltm-ConfigNRDC used in draft RRC CR discussion should be introduced.

Proposal 2. ltm-Config (which doesn’t contain inter-CU LTM candidate configuration) for MCG and ltm-ConfigNRDC are simultaneously configured.

Here, we’ll consider whether ltm-Config for SCG and ltm-ConfigNRDC are simultaneously configured. RAN2 agreed that intra-SCG LTM candidate configuration can be mixed in MN RRC format, i.e., ltm-ConfigNRDC. It means that 8 intra-SCG LTM candidate configurations can be provided by MN RRC format while 8 intra-SCG LTM candidate configurations are configured by SN RRC format. It seems a violation of the agreement “The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.” And it is difficult how to differentiate the configuration which is indicated in the LTM CSC/TCI (de)activation MAC CE. Therefore, we suggest prohibiting that ltm-Config for SCG and ltm-ConfigNRDC are simultaneously configured.

Proposal 3. ltm-Config for SCG and ltm-ConfigNRDC are not simultaneously configured.

“ltm-ConfigNRDC” is signalled via MCG RRC, but there can be UE variables which are used for MCG LTM. And RAN2 already agreed to support intra-MN LTM and inter-SN LTM. Even if the inter-SN LTM candidate configuration is provided by MN RRC format, UE variables, i.e., VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID and VarLTM-ServingCellNoSecurityChange (ID), for SCG should be used for ltm-ConfigNRDC same as ltm-Config for SCG. Otherwise, UE overwrites UE variables for MCG upon SCG LTM cell switch execution while inter-SN LTM candidate configuration is configured. And, we could consider introducing new UE variables for this purpose, but we believe that reusing existing UE variables would have less spec impact. So, for ltm-ConfigNRDC, UE should use UE variables for SCG.

Proposal 4. UE uses SCG UE variables for inter-SN LTM candidate configurations, e.g., ltm-ConfigNRDC.

Then, RAN2 needs to discuss how to treat the ltm-Config for SCG when the ltm-ConfigNRDC is configured. According to the current RRC CR and some agreements, inter-SN LTM candidate configurations are provided by MCG and a MAC CE indicating inter-SN LTM cell switch is provided by SCG. If SN provides intra-SN LTM candidate configuration via RRC signalling for the UE, UE holds ltm-Config for SCG and UE variables associated with ltm-Config for SCG. Thus, UE needs to release or replace the configuration and UE variables. If UE needs to release these, two implementation methods can be considered. First one is that NW (SCG) sends RRC signalling to the UE to release the configurations and variables, second one is that UE autonomously releases the configurations and variables. It seems wasteful if UE releases it by explicit signalling from the SCG from the perspective of radio resource efficiency. That is, the UE needs to autonomously release the contents of ltm-Config for SCG.

Proposal 5. UE autonomously releases the contents of ltm-Config for SCG when the ltm-ConfigNRDC is configured.

There is an editor’s note in the RRC CR as below:

Editor’s Note: FFS whether for the SRBs we use the field reestablishPDCP within the LTM candidate configuration (same as for the RLC re-establishment in Rel-18).

Basically, in case of intra-CU cell switch, there is no need to re-establish PDCP entity. And PDCP re-establishment always includes security key update since same COUNT value cannot be used for same security key. It means all RBs (i.e., not only SRBs) are re-established if the SRBs are re-established. Therefore, if PDCP re-establishment is needed while subsequent LTM is performed except in a case of inter-CU LTM execution, PDCP entity should be re-established by explicit signalling. Furthermore, if it is needed, it should be supported in Rel-18 LTM. However, RAN2 decided not to use the field for LTM configuration. So, the field reestablishPDCP for the SRB within the LTM candidate configuration should not be used.

Proposal 6. The field reestablishPDCP for the SRB within the LTM candidate configuration is not used.

Early DL synchronization

RAN2 agreed that R18 TCI state activation/deactivation MAC CE is reused. However, it is not discussed which node sends the MAC CE. For inter-CU MCG LTM case, the MN-DU should send the MAC CE since MCG receives L1 measurement report for inter-CU MCG LTM and this MAC CE is used for early DL sync with the candidate PCell. For inter-CU SCG LTM case, RAN2 already agreed that source SN sends the LTM CSC MAC CE to the UE. Therefore, it is natural that source SN sends TCI state activation/deactivation MAC CE even if the inter-CU SCG LTM configuration is signalled via MN RRC format.

Proposal 7. Source MN sends TCI state activation/deactivation MAC CE for inter-CU MCG LTM, and source SN sends TCI state activation/deactivation MAC CE for inter-CU SCG LTM.

Security

In RAN2#128 meeting, it was agreed that the discussion on fast recovery upon inter-CU LTM failure will be postponed until the security solution is finalized.

RAN2#128 agreement RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

SA3 sent a reply LS [4] about security update in inter-CU LTM cell switch procedure. According to the LS, SA3 decided that NCC value is sent to the UE in plaintext via MAC CE for inter-CU LTM cell switch in non-DC cases. The details on key distribution between the source gNB and the target gNB are left for RAN3’s decision. Thus, RAN2 is suggested to continue the discussion on the fast recovery upon inter-CU LTM failure.

Proposal 8. RAN2 confirms that SA3 solution (NCC value is indicated in MAC CE) is specified.

From the security aspect, whether the NCC is included in LTM cell switch command MAC CE upon inter-CU LTM execution or in a new MAC CE should be discussed in RAN2. We understood that, in order to minimize RRC reconfiguration during subsequent LTM, SA3 has discussed on a compromise solution that provides security for inter-CU LTM. This solution may not have forward compatibility i.e., it is not applicable to inter-CU Conditional LTM if the NCC value is included in LTM CSC MAC CE. However, due to lack of remaining time, RAN2 needs to confirm that the NCC in MAC CE solution is agreed. The LTM CSC MAC CE has some reserved bit which can include NCC value (3bit). Therefore, a new field to include NCC value in the LTM CSC MAC CE for inter-CU MCG LTM is specified.

Proposal 9. The NCC value is included in LTM cell switch command MAC CE.

Proposal 10. A new field to include NCC value in the LTM CSC MAC CE for inter-CU LTM is specified.

Next, it should be discussed how to handle inter-CU failure case. It is advantageous to support the fast recovery upon inter-CU LTM failure to reduce the interruption time. In case of CHO, UE can generate new security key from NCC value included in CondRRCReconfig, if the selected cell is a candidate cell. And if the selected cell is an intra-CU LTM candidate cell, UE can apply the LTM configuration associated with the candidate cell. Therefore, if the selected cell is an LTM target cell (UE has already received the MAC CE including NCC value), UE can apply the LTM configuration and indicated NCC value for the selected cell. In our memory, RAN2 discussed and made a consensus that NCC value can be used for failure recovery before the NCC value is used for transmission of RRC reconfiguration complete message. Thus, we think that UE can perform LTM fast recovery by using indicated NCC value in an LTM CSC MAC CE when the UE fails inter-CU LTM cell switch and if the selected cell is the LTM target cell indicated in the LTM CSC MAC CE. Furthermore, RAN2 should discuss whether the indicated NCC value can be used for other cases, e.g., the selected cell is not the target cell.

Proposal 11. RAN2 supports the fast recovery upon inter-CU LTM failure.

Proposal 12. UE can perform inter-CU LTM fast recovery by using indicated NCC value in an LTM CSC MAC CE when the UE fails inter-CU LTM cell switch and if the selected cell is the LTM target cell indicated in the LTM CSC MAC CE. FFS the case when the selected cell is not the target cell.

Furthermore, RAN2 needs to discuss how to derive SN key for inter-MN LTM execution with SCG. As mentioned above, SA3 replies that the NCC value is sent via MAC CE if it is a non-DC case. To support inter-MN LTM with SCG, RAN2 needs to apply the MAC CE solution to DC-casesince there is limited time to discuss other solutions for DC-cases in RAN2 and SA3.

Proposal 13. MAC CE solution for security key derivation (NCC value is indicated in LTM CSC MAC CE) is also applied to DC-case.

After MN key is derived, SN key should be derived. SN key is derived from K_gNB and sk-Counter, so there are some options how to select sk-Counter as below:

Option 1. Same sk-Counter is used.

Option 2. Default sk-Counter value (i.e., 0) is used.

Option 3. New sk-Counter is provided in the LTM CSC MAC CE.

Option 4. New sk-Counter is pre-configured in ltm-CandidateConfig.

For option 2, it seems to be a big security concern, because if the NCC on which K_gNB is based is provided by an unprotected MAC CE and a known value is used for the sk-Counter, it becomes easier for a third party to derive the SN key. For option 3, it provides the same level of security as option 2. For option 1 and 4, since the values ​​are provided in a secure environment, we don't think there are any more security concerns than with other options. Furthermore, option 4 has no spec impact. Therefore, we prefer option 4. However, since the SN key is derived based on the MN key, a security level greater than that of the MN key may not be necessary. At least, RAN2 needs to discuss how to derive SN key when inter-MN LTM with SCG based on these options.

Proposal 14. RAN2 discuss how to select sk-Counter when inter-MN LTM with SCG based on the following options:

Option 1. Same sk-Counter

Option 2. Default sk-Counter value (i.e., 0)

Option 3. New sk-Counter which is provided in LTM CSC MAC CE.

Option 4. New sk-Counter which is pre-configured in ltm-CandidateConfig.

In the RRC CR discussion there were some opinions on how to select the sk-Counter upon inter-CU SCG LTM execution. In case of SCPAC, UE selects the first unused sk-Counter in the key stream associated with the securityCellSetId of target configuration as below.

1> if the securityCellSetId is included in the entry in VarConditionalReconfig containing the RRCReconfiguration message: 2> if servingSecurityCellSetId is not included within VarServingSecurityCellSetID; or 2> if the value of the securityCellSetId is not equal to the value of servingSecurityCellSetId within VarServingSecurityCellSetID: 3> consider the first sk-Counter value in the sk-CounterList associated with the securityCellSetId within the VarConditionalReconfig as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7; 3> remove the selected sk-Counter value from the sk-CounterList associated with the securityCellSetId within the VarConditionalReconfig;

This mechanism allows SN to perform early decoding since SN can predict the next SN key. We think this advantage should be applied to inter-CU SCG LTM. Therefore, we provide Text Proposal to specify the mechanism.

<TP1>

1> if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 1> if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID: 2> if the LTM cell switch is triggered on the MCG: 3> UE actions are FFS; 2> else if the LTM cell switch is triggered on the SCG: 3> consider the first sk-Counter value in the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7; 3> remove the selected sk-Counter value from the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange; Editor’s Note: How to understand which sk-counter the UE shall select is FFS.

Proposal 15. To select appropriate sk-Counter, UE needs to select the first sk-Counter value in the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange.

Proposal 16. RAN2 agrees with above TP 1.

Completion

In SCPAC, UE sends a selected configuration ID and a selected sk-Counter to MN via RRCReconfigurationComplete.

In intra-CU LTM, UE sends a selected configuration ID to MN if it is MCG LTM and UE sends a selected configuration ID to SN if it is SCG LTM. In case of intra-CU MCG LTM, inter-CU MCG LTM or inter-CU SCG LTM, UE should send the selected configuration ID to MN via complete message since the reconfiguration message is sent from the MN. And MN should indicate the selected PSCell to SN, the same as SCPAC. And, in SCPAC, the selected sk-Counter is informed from UE to MN via complete message, then MN informs the selected sk-Counter to the SN as well as the selected configuration ID. However, the selected configuration ID and selected sk-Counter is not included in SN RRCReconfigurationComplete message. The principle should be applied to inter-CU LTM. Therefore, we suggest RAN2 to approve the following TP.

<TP2>

2> if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6: 3> include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration; 3> if this RRCReconfiguration message was included in LTM-Candidate within ltm-ConfigNRDC: 4> if a new sk-Counter value has been selected due to the LTM cell switch execution procedure: 5> include selectedSK-Counter and set its value to the selected sk-Counter value; Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

Proposal 17. UE sends the selected configuration ID and the selected sk-Counter via MN RRCReconfigurationComplete message upon inter-CU SCG LTM execution.

Proposal 18. RAN2 agrees with above TP 2.

LTM and L3 HO

In RAN2#127, it was agreed that L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE while inter-CU LTM is being configured (w/o DC). It was also agreed that when performing L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations. That is, the UE simultaneously keeps the RRC configuration of L3 mobility (HO and CHO) and inter-CU LTM. Depending on the triggering timing, the UE performs either L3 mobility or inter-CU LTM. However, it is the MAC entity of the UE that receives the LTM CSC MAC CE from the source DU to trigger the inter-CU LTM, while the RRC layer of the UE receives the L3 HO command from the source CU to trigger L3 mobility. Based on Rel-18 intra-CU LTM, when the intra-CU LTM is triggered, the MAC entity of the UE subsequently notifies the RRC layer of the UE. Therefore, we propose that when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE further notifies the RRC layer of the UE, by following the rationale in Rel-18. Afterwards, RAN2 further discuss the cases when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and the execution order of L3 HO and inter-CU LTM.

Proposal 19. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 20. RAN2 further discuss:

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. The agreement made in RAN2#127 “The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message” means that “The candidate SN provides the SCG part configuration of each candidate PSCell.”

Proposal 1. ltm-ConfigNRDC used in draft RRC CR discussion should be introduced.

Proposal 2. ltm-Config (which doesn’t contain inter-CU LTM candidate configuration) for MCG and ltm-ConfigNRDC are simultaneously configured.

Proposal 3. ltm-Config for SCG and ltm-ConfigNRDC are not simultaneously configured.

Proposal 4. UE uses SCG UE variables for inter-SN LTM candidate configurations, e.g., ltm-ConfigNRDC.

Proposal 5. UE autonomously releases the contents of ltm-Config for SCG when the ltm-ConfigNRDC is configured.

Proposal 6. The field reestablishPDCP for the SRB within the LTM candidate configuration is not used.

Proposal 7. Source MN sends TCI state activation/deactivation MAC CE for inter-CU MCG LTM, and source SN sends TCI state activation/deactivation MAC CE for inter-CU SCG LTM.

Proposal 8. RAN2 confirms that SA3 solution (NCC value is indicated in MAC CE) is specified.

Proposal 9. The NCC value is included in LTM cell switch command MAC CE.

Proposal 10. A new field to include NCC value in the LTM CSC MAC CE for inter-CU LTM is specified.

Proposal 11. RAN2 supports the fast recovery upon inter-CU LTM failure.

Proposal 12. UE can perform inter-CU LTM fast recovery by using indicated NCC value in an LTM CSC MAC CE when the UE fails inter-CU LTM cell switch and if the selected cell is the LTM target cell indicated in the LTM CSC MAC CE. FFS the case when the selected cell is not the target cell.

Proposal 13. MAC CE solution for security key derivation (NCC value is indicated in LTM CSC MAC CE) is also applied to DC-case.

Proposal 14. RAN2 discuss how to select sk-Counter when inter-MN LTM with SCG based on the following options:

Option 1. Same sk-Counter

Option 2. Default sk-Counter value (i.e., 0)

Option 3. New sk-Counter which is provided in LTM CSC MAC CE.

Option 4. New sk-Counter which is pre-configured in ltm-CandidateConfig.

Proposal 15. To select appropriate sk-Counter, UE needs to select the first sk-Counter value in the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange.

Proposal 16. RAN2 agrees with above TP 1.

Proposal 17. UE sends the selected configuration ID and the selected sk-Counter via MN RRCReconfigurationComplete message upon inter-CU SCG LTM execution.

Proposal 18. RAN2 agrees with above TP 2.

Proposal 19. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 20. RAN2 further discuss:

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution order of L3 HO and inter-CU LTM.

Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- R2-2501331, Report from session on V2X/SL, R19 NES and MOB

- R2-2410928, [POST128][106][MOB] (China Telecom) 37.340 Running CR

- R2-2501756, Reply LS on security handling for inter-CU LTM in non-DC cases

Annex draft 37.340 CR (excerpt of R2-2410928)

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

Editor’s Note: RAN2 assumes that the figure should be anchored together with RAN3. The final figure is up to the progresses made in RAN3.

1.	The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required (Editor’s Note: FFS up to RAN3.) to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells to the MN.

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides a list of KSN and associated sk-Counter values for each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may select one of the candidate SN(s) and request providing the reference SCG configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message.

7.	The MN transfers, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to each candidate SN.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared.

12.	Editor’s Note: FFS up to RAN3.

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN.

Editor’s Note: Details for TA information transfer procedure is up to RAN3 discussion.

15.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9.

16.	The source SN decides to execute cell switch to a target PSCell.

17.	The source SN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID including PSCell change.

18/19.	The source SN sends a notification message to the target SN via the MN, to indicate the triggering of LTM to the UE.

Editor’s Note: Details for Cell Switch Notification procedure is up to RAN3 discussion.

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.