3GPP TSG-RAN WG2 Meeting #129bis                              	  R2-2502394

Wuhan, China, April. 7th – 11th, 2025

Agenda Item:	8.11.2

Source: 	OPPO

Title:  	Remaining issues of SBFD RACH procedure

Document for:	Discussion and Decision

# Introduction

According to the previous RAN2 discussion, RAN2 made the following agreements regarding the RACH procedure for SBFD operation:

RAN2#127: Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

RAN2#127bis: RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2#128: CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

RAN2#129: When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In this contribution, we provide some analysis on the remaining issues of the SBFD RACH procedure

# Discussion

## RO type indication for CBRA

- RAN2#129 agreement: For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

Regarding the signalling details of the RO type indication provided the UE for the initial RA transmission, we think that we can unify the UE behaviours for both the SIB configuration and the UE-dedicated RRC configuration, by providing the roType (i.e. legacy RO or additional RO) in RACH-ConfigCommon. If the network wants to have dedicated control for a specific-UE to prioritize the SBFD-RO, the gNB can use PDCCH-order to indicate the RO type.

Proposal 1: The configuration of the prioritized RO type (legacy RO or additional RO) for CBRA is per BWP, i.e. within RACH-ConfigCommon.

## RSRP threshold for RO type selection

- RAN2#129 agreement: - If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

To ensure a more reliable transmission of the preamble transmission via SBFD symbol, the RSRP associated to the additional RO should be above a configured threshold, as the preamble transmission over the additional RO in the SBFD symbol is expected to experience more interference, compared with the legacy RO in the non-SBFD symbol. On the other hand, as the legacy RO on the non-SBFD symbol does not have the extra CLI interference issue, the legacy RO on the non-SBFD symbol should be allowed to be used by both the cell-centre UE and the cell-edge UE. Thus, when the RSRP used for the RO type selection is above a threshold, we can leave the selection of the RO type to the UE implementation

Proposal 2: If no RO type indication is provided by the network, the UE selects the legacy RO type when the SSB RSRP is below a threshold.

Proposal 3: If no RO type indication is provided by the network, both the legacy RO and the additional RO can be used when the SSB RSRP is above a threshold, and it is up to the UE implementation whether to select the legacy RO or the additional RO.

## RACH procedure step of RO type selection

RAN2#129 agreement: FFS whether RO type selection is performed before or after the RA type selection.

Regarding the RACH procedure step for the RO type selection, we could have the following two solutions:

- Solution 1: The RO type selection is performed before the RA type selection (i.e. the selection between 2-step RACH and 4-step RACH).

- Solution 2: The RO type selection is performed after the RA type selection (i.e. the selection between 2-step RACH and 4-step RACH).

According to the discussion in RAN1, the 2-step RACH is not supported for the additional RO. If the additional RO is firstly selected before the RA type selection as given in Solution 1, and the UE could later-on select the 2-step RACH (not supporting additional RO) according to the RA type selection criterion. From our understanding, both Solution 1 and Solution 2 should have some extra specification texts to avoid the combination of the 2-step RACH and the additional RO. The difference between Solution 1 and Solution 2 is that Solution 2 lets the UE to have more opportunities to select the 2-step RACH, compared with Solution 1. From our understanding, as the 2-step RACH of the legacy RO has less RACH latency compared with the 4-step RACH of the additional RO, we should adopt Solution 2 to allow the UE to have more chances to select 2-step RACH so that the UE can have more opportunities of reducing the RACH latency.

Example of calculating the probability of selecting 2-step RACH: Assuming the combination of the 2-step RACH with the additional RO is not allowed. Assuming the same RSRP threshold is used for RO type selection and RA type selection, and the same probability of selecting the 2-step RACH and the additional RO based on the RSRP threshold: P2-step_RSRP = 50%: the probability of selecting the 2-step RACH for the RA type selection based on RSRP Padditional_RO_RSRP = 50%: the probability of selecting the additional RO for the RO type selection based on RSRP Then the probability of selecting 2-step RACH by using Solution 1 and Solution 2 would be as follows: P2-step(Solution_1) = (1 - Padditional_RO_RSRP) * P2-step_RSRP = 25% P2-step(Solution_2) = P2-step_RSRP = 50% This is due to the fact: (1 - Padditional_RO_RSRP) * P2-step_RSRP < P2-step_RSRP

Observation 1: Compared with the solution that the RO type selection is performed before the RA type selection, the solution that the RO type selection is performed after the RA type selection provides the UE with more chances of selecting 2-step RACH, as the combination of 2-step RACH with additional RO is not supported.

Observation 2: The 2-step RACH of the legacy RO has less RACH latency, compared with the 4-step RACH of the additional RO.

Proposal 4: RO type selection is performed after the RA type selection.

## Switching from SBFD to non-SBFD

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

From our understanding, the RACH attempt failure could be caused by either the coverage issue or the preamble collision issue. The main reason of supporting the switching from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbols is the coverage issue of the SBFD symbols, due to more CLI interference to the SFBD symbols. We understand that the failure of the non-SBFD symbol could be due to the preamble collision issue. As both the SBFD UE and the legacy UE are able to use the non-SBFD symbol, switching from the non-SBFD symbol to the SBFD symbol can bring less preamble collision. To allow more flexible network deployment/configuration, we think that the switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols can be supported.

Proposal 5: The switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

## RACH event for SFBD RO

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

According to the previous RAN2 discussion, RAN2 assumes that the random access procedure in SBFD symbols is supported for all the existing RACH trigger events. However, a few companies consider that the SI request does not need to support the additional RO in SBFD symbol, as the on-demand SI is not delay-sensitive. From our understanding, supporting the additional RO in SBFD symbol for SI request could bring the benefit of reducing the number of RO/preamble resources allocated for the on-demand SI, given that the PRACH resources (i.e. RO/preamble) configured for one on-demand SI can be more than one [1]. Then the performance (e.g. less contention) of the non-SI-request RACH using the legacy RO can be improved, due to the use of the additional RO in SBFD symbol for SI-request.

Proposal 6: RAN2 confirms that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

## Msg3 indication for SBFD

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

Firstly, we consider that if the gNB is able to know the UE capability of supporting SBFD earlier, it would be beneficial for the gNB to use the SBFD symbols for the UE immediately after RACH. Some companies consider that as the UE could select the legacy RO, the gNB is not able to know the UE capability of supporting SBFD during RACH. Then indicating the UE capability (e.g. 1 bit) of supporting SBFD in Msg3 can be useful. However, from our understanding, the UE would have several different capability bits on how to support different SBFD features (e.g. the support of different types of SBFD RACH configurations, the support of different SBFD Tx/Rx/measurement procedures and the support of different CLI handlings). Then indicating only 1 bit on the SBFD capability via Msg3 is not able to help the gNB to provide the proper SBFD configurations to the UE. Thus, apart from the RACH procedure, the gNB would anyway need the full UE capability bits of SBFD (e.g. via Msg5 of the UECapabilityInformation message) to configure the SBFD function to the UE. Thus, we think that the 1-bit Msg3 indication for SBFD is not needed.

Observation 3: Using 1-bit Msg3 indication of SBFD cannot help the gNB to provide the correct SBFD configuration, as a few UE capability bits will be introduced for indicating which sub-feature of SBFD function is supported.

Proposal 7: Msg3 indication for SBFD is not supported.

# Conclusion

According to the analysis provided above, we have the following proposals:

Observation 1: Compared with the solution that the RO type selection is performed before the RA type selection, the solution that the RO type selection is performed after the RA type selection provides the UE with more chances of selecting 2-step RACH, as the combination of 2-step RACH with additional RO is not supported.

Observation 2: The 2-step RACH of the legacy RO has less RACH latency, compared with the 4-step RACH of the additional RO.

Observation 3: Using 1-bit Msg3 indication of SBFD cannot help the gNB to provide the correct SBFD configuration, as a few UE capability bits will be introduced for indicating which sub-feature of SBFD function is supported.

Proposal 1: The configuration of the prioritized RO type (legacy RO or additional RO) for CBRA is per BWP, i.e. within RACH-ConfigCommon.

Proposal 2: If no RO type indication is provided by the network, the UE selects the legacy RO type when the SSB RSRP is below a threshold.

Proposal 3: If no RO type indication is provided by the network, both the legacy RO and the additional RO can be used when the SSB RSRP is above a threshold, and it is up to the UE implementation whether to select the legacy RO or the additional RO.

Proposal 4: RO type selection is performed after the RA type selection.

Proposal 5: The switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

Proposal 6: RAN2 confirms that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Proposal 7: Msg3 indication for SBFD is not supported.

# Reference

[1] 3GPP TS38.331, “NR; Radio Resource Control (RRC) protocol specification”