3GPP TSG-RAN WG2 Meeting #129bis              			          R2-2502396

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.6.4

Source: 	Lenovo

Title: 	CLTM Scenarios and remaining points

Document for:	Discussion and decision

# Introduction

In the past RAN2 meeting, we have made a good progress for Intra-CU conditional MCG LTM. This document aims to address the remaining points for the Intra-CU MCG C-LTM work.

# Discussion

Early TA handling

It was agreed that the early TA is signalled to the UE from the source cell via a new MAC CE, and each early TA for a candidate cell is maintained by a new timer. That means UE will maintain multiple timers if UE receives multiple early TA values for the multiple candidate cells. Once the condition is met for a target cell, UE switches to this target cell which is one of LTM candidate cells.

How does UE handle early TA timers for the remaining candidate cells after UE is triggered to switch to the target cell? RAN2 can discuss the following options.

- Option 1: UE releases remaining early TA values once cell switching is triggered;

- Option 2: UE releases remaining early TA values only after successful cell switching;

- Option 3: UE maintains them after cell switching until the timer expires.

Proposal 1: RAN2 to discuss how the UE handle early TA timers for other candidate cells after UE is triggered to switch to one of candidate cells.

- Option 1: UE releases remaining early TA values once cell switching is triggered;

- Option 2: UE releases remaining early TA values after successful cell switching;

- Option 3: UE maintains them after Cell switching until timer expires.

Since releasing early TA seems like a “lost effort”, we prefer the Option 3 above since it is possible that in certain cases this will avoid another PDCCH-order based RACH to obtain Early TA afresh for a subsequent mobility.

The next question is how to correctly implement early TA Timer in the specifications.

There must be a timeAlignmentTimer running in the UE for the source cell when the (new) MAC CE including a TA value for a candidate cell is received. A UE will start a ltm-candidate1-timeAlignmentTimer when it receives a MAC CE (from the source cell) including a TA value for a candidate cell1. In the Rel. 18 LTM the behaviour was as follows:

## 5.2 Maintenance of Uplink Time Alignment <removed text> 1> when an LTM Cell Switch Command MAC CE is received and the Timing Advance Command is not set as FFF: 2> apply the Timing Advance Command for the PTAG as specified in clause 6.1.3.75; 2> start or restart the timeAlignmentTimer associated with the PTAG as specified in clause 6.1.3.75.

The association to PTAG is indeed the association for the PTAG-to-be (and in that sense R18 specification is not really accurate since the UE is still in the source side when the UE receives the Absolute Timing Advance Command for a target cell).

In rel. 19, the UE will receive an Absolute Timing Advance Command for likely many candidate cells (and as many ltm-candidateX-timeAlignmentTimer(s) will need to be started) until the condition for one of these is fulfilled and the UE can start mobility execution. So, an “association with the PTAG” is not a clean way since there must be only ONE PTAG.

If one of the candidate cell (Cell1) becomes target (i.e., conditions for said candidate cell is met), UE shall use the received TA to make the first UL transmission (handover complete). After this point, the UE could release the ltm-candidate1-timeAlignmentTimer which will effectively also throw away UL Synchronization until a fresh Timing Advance Command is received from the new serving cell.

An easier way to get around this problem will be to “promote” the ltm-candidate1-timeAlignmentTimer to the timeAlignmentTimer of the new serving cell as soon as the UE has successfully announced its arrival in the target cell (i.e., upon receiving a DCI addressed to the same HARQ Process ID as used to transmit the handover complete message). This will mean that the ltm-candidate1-timeAlignmentTimer becomes the timeAlignmentTimer and is running with the "remaining" timer value. An example implementation is as below:

## 5.2 Maintenance of Uplink Time Alignment <removed text> 1> when an LTM Timing Advance Command MAC CE is received and the Timing Advance Command is not set as FFF: 2> apply the Timing Advance Command for the included candidate cell as specified in clause 6.1.3.xx; 2> start or restart the ltm-candidate-timeAlignmentTimer associated with the candidate cell as specified in clause 6.1.3.xx. 2> upon successful arrival in the target cell as specified in clause xx the ltm-candidate-timeAlignmentTimer is set as the timeAlignmentTimer with its remaining value.

Accordingly, we make the following proposal:

Proposal 2: UE promotes the candidate cell’s ltm-candidate1-timeAlignmentTimer to the timeAlignmentTimer of the new serving cell upon successful arrival in the target cell (i.e., upon receiving a DCI addressed to the same HARQ Process ID as used to transmit the handover complete message). The timeAlignmentTimer is started with the "remaining" ltm-candidate1-timeAlignmentTimer value.

Restarting LTM evaluation

According to the section 5.3.5.18.6 of LTM running CR, once LTM is triggered due to reception of command MAC CE, cell selection or fulfilment of conditions, UE will stop the LTM conditions evaluation for all the LTM candidate configurations. And UE restarts LTM conditions evaluation immediately if the LTM-Candidate IE for the selected LTM candidate configuration includes the field cltm-ExecutionConditions. It may result in that the second LTM cell switch will be triggered due to fulfilment of LTM condition while the first LTM cell switch is ongoing. To avoid this, UE should restart evaluating the LTM execution conditions after completion of LTM cell switching as legacy subsequent CPAC.

In legacy subsequent CPAC, The UE starts evaluating the execution conditions for the initial execution of subsequent CPAC once reception of S-CPAC configuration. Once the S-CPAC is triggered, the UE stops evaluating S-CPAC execution condition. And the UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it). The UE restarts evaluating the execution conditions of candidate PSCells (if provided for the following execution of subsequent CPAC) after completion of cell switch.

If UE does not stop evaluating S-CPAC execution condition while one S-CPAC is ongoing. The second S-CPAC will be triggered while UE is performing the first S-CPAC.

10.20	Subsequent Conditional PSCell Addition or Change

A Subsequent Conditional PSCell Addition or Change (subsequent CPAC) is defined as a conditional PSCell addition or change procedure that is executed after a (conditional) PSCell addition, a (conditional) PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA. The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells (if provided for the following execution of subsequent CPAC) after completion of a PSCell addition, a PSCell change, a PCell change, or an SCG release (if subsequent CPAC configuration is received via an MN RRC message). Subsequent CPAC configuration can be initiated either by the MN or by the SN.

The following principles apply to subsequent CPAC:

……

- While executing subsequent CPAC, the UE is not required to continue evaluating the execution condition of other candidate PSCell(s) or PCell(s).

- The UE is not required to continue evaluating the execution conditions of other subsequent CPAC candidate PSCell(s) when PSCell change/addition or PCell change is triggered

Observation 1: In legacy subsequent CPAC, UE is not allowed to evaluate the execution condition while PSCell change is ongoing.

Proposal 3: UE restarts evaluating the LTM execution conditions after completion of LTM cell switching due to reception of command MAC CE, cell selection or the fulfilment of the execution condition.

LTM based Fast Recovery

Regarding Rel-18 LTM based failure recovery, if the cell selection is triggered by MCG RLF or failure of LTM cell switch after reception of LTM command MAC CE and the selected cell is one of the LTM candidate cells, the UE performs the LTM cell switch procedure instead of re-establishment procedure. Namely, Rel-18 Intra-CU LTM based recovery can be applied to two failure cases i.e., to RLF on MCG and failure of LTM cell switch triggered by LTM cell switch command MAC CE.

Last meeting, RAN2 agreed to support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA). Based on the latest running CR, Rel-19 Intra-CU conditional LTM based recovery can be applied to two failure cases i.e., RLF on MCG and failure of LTM cell switch triggered by LTM cell switch command MAC CE. Therefore, one case is missing. Namely, whether Intra-CU MCG LTM based fast recovery or rather Intra-CU MCG CLTM based fast recovery should be applied to the failure of Intra-CU MCG CLTM? It seems unnecessary to restrict it. Therefore, we propose:

Proposal 4: Intra-CU MCG LTM based fast recovery or Intra-CU MCG CLTM based fast recovery can be applied to the failure of Intra-CU MCG CLTM.

According to reply LS from SA3 (S3-251124), MAC CE will be used to carry NCC. Therefore, we cannot support Inter-CU LTM based fast recovery since the UE would not know which NCC to apply towards the new candidate cell. In addition, after the failure of Inter-CU MCG LTM, both Intra-CU MCG LTM based fast recovery and Intra-CU MCG CLTM based fast recovery are not applied as legacy.

Proposal 5: Inter-CU LTM based fast recovery is not supported in Rel-19.

Proposal 6: both Intra-CU MCG LTM based fast recovery and Intra-CU MCG CLTM based fast recovery are not applied to Inter-CU MCG LTM failure as legacy.

Co-existence between CLTM and Intra-CU LTM

It seems unnecessary to restrict the simultaneous configuration of Intra-CU conditional MCG LTM and intra-CU MCG LTM for the different candidate cell i.e. different PCI based on the previous RAN2 discussion. The question could be whether the simultaneous configuration of Intra-CU MCG CLTM and intra-CU MCG LTM for the same candidate cell can be allowed. If allowed, there are two cases as follows.

- Case 1: The serving cell provides two different LTM candidate cell IDs for CLTM and MAC command based LTM, respectively. Namely, there are separate LTM candidate configurations. Up to RAN3 decide whether the separate handover request for the same candidate cell (same PCI) is needed.

- Case 2: The serving cell provides one LTM candidate cell ID both for CLTM and MAC command based LTM. In this case, LTM cell switch command MAC CE can be used to trigger LTM execution for a candidate cell configured with LTM execution condition.

Proposal 7: Simultaneous configuration of Intra-CU MCG CLTM and Intra-CU MCG LTM for the different candidate cell (i.e. different PCI) is supported.

Proposal 8: Simultaneous configuration of Intra-CU MCG CLTM and Intra-CU MCG LTM for the same candidate cell is supported.

Scenario Discussion based on timeline of Early TA reception and C-LTM Condition fulfilment

The LTM Condition fulfilment and Early TA availability are two independent events that may occur at different point of time. In the following a few scenarios are depicted and also some basic solutions are mentioned, RAN2 should discuss these.

Scenario 1

In case of conditional handover procedure, a handover may only be executed by the UE when included condition(s) for at least one candidate cell are fulfilled. The fulfilment of radio condition(s) is not dependent on acquisition of an Early TA – UE receives it from the source.

One question therefore arises: What if at the point of fulfilment of included condition(s) for a first candidate cell, the early TA acquisition of that candidate cell has not been received by the UE? In other words, if an Early TA value is not yet available with the UE but radio condition(s) for a potential candidate is fulfilled, should it initiate afresh RACH procedure towards the candidate (target) cell?

Figure 1: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., before receiving a PDCCH Order for C1

In our opinion, the UE shall start handover procedure execution immediately after Point A. To this end, the UE shall use the PRACH resource indicated in the Conditional Configuration message – which may include a Contention Free PRACH resources. The RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 9: If the LTM condition is fulfilled before a PDCCH order for the same target cell is received or the Msg1 transmission has not started, RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Scenario 2

Figure 2: Point A is after transmitting Msg1 (a PDCCH Order for C1 was received)

Scenario 2 depicted in Figure 2 similar to Scenario1 but here the UE can wait to receive a valid TA from the source cell if Msg1 has already been transmitted. Or, to have unified behaviour same solution as in Scenario 1 applied. If however, CFRA resources distinguishing if the TA is sent back to the UE (and not to the source cell) is not available, UE may initiate CBRA, if Msg1 has not yet been transmitted.

Proposal 10: RAN2 to discuss correct UE behaviour when the LTM condition is fulfilled, a PDCCH order for the same target cell is received and the Msg1 transmission has started, UE may wait to receive an “Early” TA from the source or may initiate CBRA transmission.

Scenario 3

UE starts condition evaluation immediately, as soon as possible based on the measurement gaps received in the configuration, of one or more candidate cells. In one example, as shown in Figure 3, the condition(s) for a candidate cell C1 is fulfilled at point A, i.e., after receiving a TA for C1 from the source cell but at which point it is unclear if the TA for C1 is still valid.

Figure 3: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., after receiving an Early TA for C1

UE starts a TA-timer, which may have been configured earlier in the Conditional Configuration message.

If Point A occurs before the expiry of this timer (Case1 in Figure 3), UE transmits handover completion. If Point A occurs after the expiry of this timer (Case2 in Figure 3), UE shall start handover procedure execution immediately after Point A. To this end, the UE shall (re)use the PRACH resource indicated in the Conditional Configuration message irrespective of if a Msg1 transmission to cell C1 has been made already. Previously received configuration may include a Contention Free PRACH resources. The RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 11: A TA-timer is used to check the validity of received TA value. If LTM execution is fulfilled before the expiry of the TA-timer, UE sends the HO Complete to the target cell using RACH-less LTM procedure; else, UE may start a CFRA indicating that Msg2 be sent to the UE directly (not to the source cell).

# Conclusion

This document attempted to develop a basic understanding of “conditional LTM” and proposed some first scenarios for RAN2 consideration. In summary, we have following observations and proposals:

Early TA delivery

Proposal 1: RAN2 to discuss how the UE handle early TA timers for other candidate cells after UE is triggered to switch to one of candidate cells.

-	Option 1: UE releases remaining early TA values once cell switching is triggered;

-	Option 2: UE releases remaining early TA values after successful cell switching;

-	Option 3: UE maintains them after Cell switching until timer expires.

Proposal 2: UE promotes the candidate cell’s ltm-candidate1-timeAlignmentTimer to the timeAlignmentTimer of the new serving cell upon successful arrival in the target cell (i.e., upon receiving a DCI addressed to the same HARQ Process ID as used to transmit the handover complete message). The timeAlignmentTimer is started with the "remaining" ltm-candidate1-timeAlignmentTimer value.

Restarting LTM evaluation

Observation 1: In legacy subsequent CPAC, UE is not allowed to evaluate the execution condition while PSCell change is ongoing.

Proposal 3: UE restarts evaluating the LTM execution conditions after completion of LTM cell switching due to reception of command MAC CE, cell selection or the fulfilment of the execution condition.

Proposal 4: Intra-CU MCG LTM based fast recovery or Intra-CU MCG CLTM based fast recovery can be applied to the failure of Intra-CU MCG CLTM.

Proposal 5: Inter-CU LTM based fast recovery is not supported in Rel-19.

Proposal 6: both Intra-CU MCG LTM based fast recovery and Intra-CU MCG CLTM based fast recovery are not applied to Inter-CU MCG LTM failure as legacy.

Co-existence between CLTM and Intra-CU LTM

Proposal 7: Simultaneous configuration of Intra-CU MCG CLTM and Intra-CU MCG LTM for the different candidate cell (i.e. different PCI) is supported.

Proposal 8: Simultaneous configuration of Intra-CU MCG CLTM and Intra-CU MCG LTM for the same candidate cell is supported.

Scenario Discussion based on timeline of Early TA reception and C-LTM Condition fulfilment

Proposal 9: If the LTM condition is fulfilled before a PDCCH order for the same target cell is received or the Msg1 transmission has not started, RACH Msg1 transmission shall include an implicit/ explicit indication that the UE is awaiting a timing advance in Msg2 and the same need not be sent to the source cell.

Proposal 10: RAN2 to discuss correct UE behaviour when the LTM condition is fulfilled, a PDCCH order for the same target cell is received and the Msg1 transmission has started, UE may wait to receive an “Early” TA from the source or may initiate CBRA transmission.

Proposal 11: A TA-timer is used to check the validity of received TA value. If LTM condition is fulfilled before the expiry of the TA-timer, UE sends the HO Complete to the target cell using RACH-less LTM procedure; else, UE may start a CFRA indicating that Msg2 be sent to the UE directly (not to the source cell).

# References

- Chair notes for RAN2#129