3GPP TSG-RAN2 Meeting #129bis	R2-2502399

Wuhan, China, Apr 7-11, 2025

Agenda Item:	8.6.4

Source: 	ITRI

Title: 	Discussion on coexistence of conditional LTM and LTM

Document for:	Discussion/Decision

Background

A revised Work Item on NR mobility enhancements Phase 4 was approved in RAN plenary #107 [1]. One objective of this Work Item is to specify support of conditional Intra-CU Layer1/Layer 2 Triggered Mobility (LTM) as listed below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

According to the Work Item Description (WID), this objective is aimed at supporting subsequent mobility. This means that after a conditional LTM (CLTM) cell switch, the UE can continue to evaluate the CLTM execution conditions and execute subsequent CLTM cell switches without RRC reconfiguration.

During RAN2#129 meeting, an offline discussion on stage 3 aspects of mobility enhancements took place and led to the following agreements [2].

RAN2#129 Agreements [2]:

- A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

- The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

Based on the agreements, the UE can be simultaneously configured with LTM candidate configurations and CLTM candidate configurations. The (network-triggered) LTM cell switch is triggered via a cell switch command sending from the source cell, while the (UE-triggered) CLTM cell switch is triggered by the UE based on execution condition evaluation.

Observation 1: The UE can be simultaneously configured with LTM candidate configurations and CLTM candidate configurations.

In this contribution, we discuss the coexistence issue of CLTM and LTM.

Discussion

The LTM cell switch may be triggered based on L1 measurements. In this case, it is not as robust as the L3-based handover and is more prone to cell switch failures.

Observation 2: Different from the L3-based handover, the LTM cell switch may be triggered based on L1 measurements, in which case it is not as robust as the L3-based handover.

Regarding CLTM, upon receiving the CLTM configurations, the UE starts evaluating the CLTM execution conditions. Once the UE finds that a CLTM candidate fulfills its corresponding execution condition, it immediately triggers a CLTM cell switch.

Observation 3: The CLTM cell switch is triggered upon CLTM execution condition fulfilment.

Therefore, if the CLTM execution condition evaluation continues during the LTM cell switch, it may trigger another cell switch, interrupting the ongoing cell switch and increasing the mobility latency.

On the other hand, if a cell that fulfills its execution condition can be discovered during the LTM cell switch, it could be used for failure recovery if the LTM cell switch fails, since LTM is not as robust as L3-based handover. Otherwise, cell selection procedure is performed after the failure occurs and the connection failure is handled subsequently.

Based on the observations and analysis above, determining how to handle the CLTM execution condition evaluation during the LTM cell switch is an essential issue that needs to be discussed and decided.

Proposal 1: RAN2 should decide how to handle the CLTM execution condition evaluation during the LTM cell switch.

Conclusion

In this contribution, the following observations and proposal are made:

Observation 1: The UE can be simultaneously configured with LTM candidate configurations and CLTM candidate configurations.

Observation 2: Different from the L3-based handover, the LTM cell switch may be triggered based on L1 measurements, in which case it is not as robust as the L3-based handover.

Observation 3: The CLTM cell switch is triggered upon CLTM execution condition fulfilment.

Proposal 1: RAN2 should decide how to handle the CLTM execution condition evaluation during the LTM cell switch.

References

- RP-250339 Revised Work Item: NR mobility enhancements Phase 4, RAN#107, Incheon, South Korea, March 12-14, 2025.

- R2-250xxxx Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17 - 21 February, 2025.