3GPP TSG-RAN WG2 Meeting #129bis		R2-2502420

Wuhan, China, 7th – 11th Apr. 2025

Source:	vivo

Title:	Discussion on service continuity for scenario C and D

Agenda Item:		8.13.4

Document for:	Discussion and Decision

- Introduction

In RAN2 #129 meeting[1], first priority scenarios (A: intra-gNB multi-hop indirect to direct path switch, B: intra-gNB multi-hop indirect to single-hop indirect path switch) of service continuity have been discussed and some agreements have been reached. However, for second priority scenarios, there was not enough time to discuss about them.

This contribution will further discuss service continuity for scenario C and D, and give our proposals on general procedure, definition of event for evaluation, and other issues.

- (C) intra-gNB direct to multi-hop indirect path switch;

- (D) intra-gNB single-hop indirect to multi-hop indirect path switch.

- Discussion

  - Scenario C (intra-gNB direct to multi-hop indirect path switch)

In single-hop U2N relay scenario, the following events have been agreed[2]:

- Event X1 (Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2) - Event X2 (Serving L2 U2N Relay UE becomes worse than threshold) - Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) - Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) - Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2)

For scenario C, the serving link is the direct Uu link, and the target link is the multi-hop indirect link, so event Y1 and Y2 can be reused with modification that the ‘candidate L2 U2N relay UE’ should be ‘candidate first relay UE’. Therefore, we propose:

Proposal 1: Event Y1 and Event Y2 are reused in intra-gNB direct to multi-hop indirect path switch, with modifying ‘candidate L2 U2N Relay UE’ to ‘candidate first relay UE’.

For measurement configuration, in last meeting, some agreements have been reached for scenario A and B[1], as follows:

Agreements:

For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

Similar agreements can also be considered for scenario C. For measurement configuration, the scenario A and scenario C are much alike, as the measurements are both on one direct link and one multi-hop indirect link. Therefore, we propose to reuse the agreements for scenario A to scenario C.

Proposal 2: For intra-gNB direct to multi-hop indirect path switch, remote UE evaluates both relay link (for the link between remote UE and candidate first relay UE) and Uu link, where the sidelink relay measurement report shall include at least candidate first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

For scenario C, as there is no serving multi-hop link, there may not need separate agreement on measurement configuration at ‘candidate first relay’, because before the candidate first relay UE connects to a remote UE, it can be seen as a normal remote UE.

Proposal 3: Before the candidate first relay UE connects to a remote UE, it can be seen as a normal remote UE and follows the legacy measurement configuration.

  - Scenario D (intra-gNB single-hop indirect to multi-hop indirect path switch)

For scenario D, the serving link is single-hop indirect link and the target link is the multi-hop indirect link, so event X2, Y2 and Z1 can be reused with modification that the ‘candidate L2 U2N relay UE’ should be ‘candidate first relay UE’.

Proposal 4: Event X2, Event Y2 and Event Z1 are reused in intra-gNB single-hop indirect to multi-hop indirect path switch, with modifying ‘candidate L2 U2N Relay UE’ to ‘candidate first relay UE’.

And similar as we analysed in section 2.1, the scenario B and scenario D are much alike on measurement configuration, as the measurements are both on one single-hop indirect link and one multi-hop indirect link. Therefore,

Proposal 5: For intra-gNB single-hop indirect to multi-hop indirect path switch, the remote UE reports serving single-hop relay UE and candidate first relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

Besides, in RAN2 #128 meeting[3], we have some agreements on scenario B:

Agreements:

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

In scenario D, there is a similar question that when the UE is switched to multi-hop indirect path, whether it is possible that the original single-hop U2N relay may become the first/intermediate relay UE or last relay UE in the target path.

In our understanding, to mimic the agreement we have made for scenario B, it is possible that the case where target first/intermediate/last relay UE are all new relay UEs which are not the source single-hop U2N relay UE, can be supported. However, for the case where the target first/intermediate/last relay UE is the source single-hop U2N relay UE, it should not be specified in RAN2 to support, as they are quite complicated. Therefore, we have the following proposals:

Proposal 6: For scenario D, RAN2 supports the case that the target first/intermediate/last relay UE are all new relay UEs which are not the source single-hop U2N relay UE.

Proposal 7: For scenario D, RAN2 does not specify anything to support the case that that either the target first/intermediate/last relay UE is the source single-hop U2N relay UE.

- Conclusion

In this contribution, we discussed the measurement configuration and event definition for scenario C and D. We also discussed the case that whether the target first/intermediate/last relay UE can be the source single-hop U2N relay UE in scenario D. We have the following Observations and Proposals:

For scenario C:

Proposal 1: Event Y1 and Event Y2 are reused in intra-gNB direct to multi-hop indirect path switch, with modifying ‘candidate L2 U2N Relay UE’ to ‘candidate first relay UE’.

Proposal 2: For intra-gNB direct to multi-hop indirect path switch, remote UE evaluates both relay link (for the link between remote UE and candidate first relay UE) and Uu link, where the sidelink relay measurement report shall include at least candidate first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 3: Before the candidate first relay UE connects to a remote UE, it can be seen as a normal remote UE and follows the legacy measurement configuration.

For scenario D:

Proposal 4: Event X2, Event Y2 and Event Z1 are reused in intra-gNB single-hop indirect to multi-hop indirect path switch, with modifying ‘candidate L2 U2N Relay UE’ to ‘candidate first relay UE’.

Proposal 5: For intra-gNB single-hop indirect to multi-hop indirect path switch, the remote UE reports serving single-hop relay UE and candidate first relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

Proposal 6: For scenario D, RAN2 supports the case that the target first/intermediate/last relay UE are all new relay UEs which are not the source single-hop U2N relay UE.

Proposal 7: For scenario D, RAN2 does not specify anything to support the case that that either the target first/intermediate/last relay UE is the source single-hop U2N relay UE.

- References

- RAN2 #129 meeting chair notes;

- 3GPP TS 38.331 V18.5.1 (2025-03);

- RAN2 #128 meeting chair notes;