3GPP TSG-RAN WG2 Meeting #129	R2-2502431

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.2.4

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT data transmission

Document for:	Discussion and Decision

1 Introduction

In RAN2#129 meeting, RAN2 discussed the A-IoT data transmission and achieved the following agreements [1]:

How to assign AS ID 1. For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. 2. From device perspective, it is only required to use one AS ID. 3. CFRA is not supported for group ID 4. RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure 5. For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: • Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. • Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 • Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2 Agreements on segmentation 1. To support segmentation, 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient. 2. Segment retransmission is supported. 3. For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3 4. R2D segmentation is not supported for R19 A-IoT. Procedures to consider From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”. Agreements on MAC PDU format design 1. Aim to design simple MAC PDU format design 2. Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages 3. At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). 4. FFS whether for D2R we need message type field, any length and need for padding 5. Specify message types and contents. As starting point consider the following MAC message types. • R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) • D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) • R2D MAC PDU (MSG2) • D2R MAC PDU (MSG3 and data) • R2D MAC PDU (R2D data) • Other message types are FFS. The message types may evolve based on functionality agreements.

In this contribution, we will discuss the left issues on the A-IoT data transmission.

2 Discussion

## 2.1 MAC PDU format of A-IoT

In legacy 5G NR, a MAC PDU is a bit string that is byte aligned in length. First, it can improve the efficiency of hardware processing. The byte is the basic unit of memory. Byte-aligned design can directly match the data access granularity of hardware and avoid additional bit operations (e.g., shift, mask).  Second, it can meet the protocol stack compatibility. PHY transport block of NR usually have the smallest unit in bytes. MAC PDUs are aligned byte-wise to seamlessly adapt to PHY layer coding, modulation, and resource mapping. Higher layer protocols (such as RLC, PDCP) also commonly use byte-aligned formats (such as IP packets). Alignment design reduces the overhead of conversion between protocols.

The principle of A-IoT system design is to minimize the complexity of the device. Considering device limitation, it might create excessive complexity on device side to have bit-aligned buffers and stored data. Moreover, current NW design assumes byte alignment. Changing this may create excessive implementation effort on network side. So A-IoT MAC PDU should also be byte aligned.

Proposal 1: A-IoT MAC PDU should be byte aligned.

In last meeting, RAN2 discussed the MAC PDU format for A-IoT and concluded that R2D MAC header at least includes message type, length for SDU and variable part(s). For D2R, whether we need message type field, any length and need for padding are FFS.

In R2D, we defined message type in MAC PDU for differentiating the MAC PDUs for different functionalities. For example, R2D MAC PDU message type may include paging/R2D trigger, Msg2 and data. Similarly, in D2R，message type should also include in MAC PDU for differentiating the MAC PDUs for different functionalities. For example, D2R MAC PDU for Msg1, Msg3 and data. Moreover, the message type values for D2R and R2D MAC PDU are separate.

Proposal 2: Apply message type to D2R MAC header as in R2D direction, the message type values for D2R and R2D MAC PDU are separate.

At present, reader does not always know the exact D2R message size of device. If the TB size provided by reader and MAC PDU are not fully matched, padding may be needed in MAC PDU.

Proposal 3: Support padding in A-IoT MAC PDU.

## 2.2 AS ID

In last meeting, RAN2 concluded that it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID for CBRA. But how this is signalled, which message is used and size of AS ID is FFS.

In post129 email discussion of AS ID [2], there are three options for which signaling to assign the AS ID as shown in the figure 1. During email discussion, companies already explained the Pros/Cons of each option.

Figure 1 AS ID assign for CBRA

We prefer the option 1, the AS ID assigned in the “Msg2 “can be used for “the first Command message i.e. Msg 4 and subsequent Command messages” to associate the resources and identify the device. Option 1 is simpler and does not introduce new procedures.

Proposal 4: For CBRA, the AS ID assigned in the Msg2.

For CFRA, there are also three options for which signaling to assign the AS ID as shown in the figure 2.

Figure 2 AS ID assign for CFRA

We prefer the option 4, the AS ID assigned in the “Msg 2 Command message “can be used for “Msg 4” and subsequent R2D Command message” to associate the resources and identify the device. Option 4 no impact on Msg 1 (Inventory Response) and not need to introduce new procedures.

For further optimization the signaling overhead, Msg1 only includes Device ID and any other upper layer data. After reader receiving Msg1, reader assigns an AS ID for the device in Msg2 which includes the “Command”.

Proposal 5: For CFRA, AS ID is assigned in the Msg2 (command message).

For CBRA, if RN16 is reused as the AS ID, the size of AS ID should be same as RN16, i.e. 16 bits. Moreover, A-IoT system assumes to support max around 60000 devices, 16 bits ID is needed at least for CBRA. And it could be good to have common ID size for CFRA and CBRA.

Proposal 6: The AS ID size is 16 bits for both CFRA and CBRA.

For validity of AS ID, RAN2 further down-selection among option 1, option 3 and option 6 according email discussion, while other options are all exclude.

Option 1: The device releases the AS ID upon receiving paging with same/new transaction id, i.e. same/different session/service.

Option 3: The device releases the AS ID upon new random ID is generated or receiving new assigned AS ID from the reader.

Option 6: The device releases the AS ID upon receiving explicit release indication from the Reader

Option 6 needs additional R2D message for this purpose, which increases signalling overhead. And the explicit indication may be missed by device. Option 3 needs new assignment will lead the AS ID to be occupied unnecessarily. We prefer option 1.

Proposal 7: The device releases the AS ID upon receiving paging with same/new transaction id, i.e. same/different session/service.

## 2.3 Command type information

In SI phase, we discussed that command type (e.g. read/write/disable) should be visible to reader for effective resource scheduling, but no agreement was reached, and the conclusion was that we need to wait for SA2 LS response on expected D2R message size.

Recently, SA2 responded that AIOTF can provides the reader with the approximate D2R message size if provided by AF, whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2 [3]. So we still need to wait for the final conclusion of SA2. If CN does not provide D2R message size to reader, the command type is useful information to reader for resource allocation.

Proposal 8: Waiting for the final conclusion of SA2 on expected D2R message size.

## 2.4 Message size information

In order for reader to schedule appropriate resource for device D2R transmission, the assistance information (e.g. D2R message size) to reader is studied. For message size information, two options are studied:

- Option 1: The estimate of expected following D2R message size from the CN to the reader;

- Option 2: The simple message size indication from the device to the reader.

For option 1, if this information is available from CN, it saves the overhead/latency caused by option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed.

In option 2, it is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

We think both the option 1 and option 2 should be supported, if reader obtains estimate of expected following D2R message size from the CN, then device no longer needs to report. Otherwise, reader can indicate device to report following D2R message size.

Proposal 9: For D2R message size information indication to reader, support both the option 1 and option 2.

Proposal 10: Reader can indicate device to report following D2R message size, if CN does not provide the estimate of expected following D2R message size to the reader.

3 Conclusion

In this contribution we discuss the functionalities required for A-IoT, with the following proposals:

Proposal 1: A-IoT MAC PDU should be byte aligned.

Proposal 2: Apply message type to D2R MAC header as in R2D direction, the message type values for D2R and R2D MAC PDU are separate.

Proposal 3: Support padding in A-IoT MAC PDU.

Proposal 4: For CBRA, the AS ID assigned in the Msg2.

Proposal 5: For CFRA, AS ID is assigned in the Msg2 (command message).

Proposal 6: The AS ID size is 16 bits for both CFRA and CBRA.

Proposal 7: The device releases the AS ID upon receiving paging with same/new transaction id, i.e. same/different session/service.

Proposal 8: Waiting for the final conclusion of SA2 on expected D2R message size.

Proposal 9: For D2R message size information indication to reader, support both the option 1 and option 2.

Proposal 10: Reader can indicate device to report following D2R message size, if CN does not provide the estimate of expected following D2R message size to the reader.

- References

- Draft report of 3GPP TSG RAN WG2 #129, RAN2 chair (interDigtal), Athens, Greece, Feb. 17th – 21st, 2025.

- Report of [POST129][036][AIoT] AS ID, Xiaomi.