3GPP TSG-RAN WG2 Meeting #129bis	R2-2502439

Wuhan, China, 7th – 11th April 2025

Agenda item:	8.10.4

Source:	Samsung

Title:	SON/MDT for NTN

Document for:	Discussion and Decision

# Introduction

One of the objectives for Rel-19 SON/MDT WI [1] is to support SON/MDT enhancements for Intra-NTN mobility. RAN3 had send a LS [2] to RAN2 including the agreements for SON/MDT enhancements for NTN. In this contribution, we discuss the objective based on the LS.

# Discussion

## MDT for NTN

RAN3 LS considers MDT enhancements for NTN and the details are as given below.

On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

For logged MDT, the basic principle is that UE is not required to perform any new measurements, but logs the measurements performed for cell reselection etc.

Observation 1: The basic principle of Logged MDT is that UE just logs the measurements it has and is not required to perform any new measurements.

RAN3 has asked RAN2 if geographical area can be used for Area Scope of logged MDT over Uu. We understand that the most important issue with the geographical area based Area Scope over Uu is the impact on the UE performance, especially UE’s battery performance. A UE is not required to have updated location in the RRC_IDLE and RRC_INACTIVE states. Any new requirement contrary to this will have serious implications on the UE battery performance. It will also affect the computational load of the UEs.

From TS 38.300, we can see that the GNSS is required to have been performed before entering connected mode, and the GNSS location needs to be maintained in connected mode. This means that there is no requirement for the UE to acquire GNSS location in RRC_IDLE/RRC_INACTIVE.

16.14.2.2       Timing Advance and Frequency Pre-compensation

For the serving cell, the network broadcast valid ephemeris information and Common TA parameters. The UE shall have valid GNSS position as well as ephemeris and Common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to an NTN cell, the UE shall compute the RTT between UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213 [38]), and autonomously pre-compensate the TTA for the RTT between the UE and the RP as illustrated in Figure 16.14.2.1-1 (see clause 4.3 of TS 38.211 [52]).

Thus, we can observe that geographical area based Area scope over Uu brings in the requirements for new measurements seriously affecting UE battery performance.

Observation 2: Geographical area based Area Scope over Uu requires UE to perform additional measurements and has a huge impact on UE battery performance.

For cell reselection measurements, it is possible to configure a distanceThresh and referenceLocation to restrict the measurements, but even here, there is no requirement for UE to perform location measurements. The measurement restrictions is applicable only when the UE has obtained its location information, and it is up to UE’s instantaneous decision to obtain location information or not.

5.2.4.2            Measurement rules for cell re-selection

Following rules are used by the UE to limit needed measurements:

-     If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-     If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN (quasi-)Earth-fixed cell and has obtained its location information:

-     If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-     Else, the UE shall perform intra-frequency measurements;

-     else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving cell and has obtained its location information:

-     if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-     else, the UE shall perform intra-frequency measurements;

For logged MDT, such a dynamic behaviour may not be suitable and the network may need a more deterministic behaviour. For e.g. if geographic area is configured for fifty UEs and all those UEs do not perform measurement logging because the location measurements is not performed solely based on UE decision, the purpose will not be served. So using geographical area based Area scope over Uu is not a feasible option.

Proposal 1: RAN2 to send LS to RAN3 informing that geographical area based Area Scope over Uu is not feasible for logged MDT.

## MRO for NTN

There could be some issues specific to NTN which may lead to radio link failures. A handover failure may occur if the Timing Advance value that the UE estimated to the target cell is incorrect. Thus including the Timing Advance field of the Timing Advance Report MAC CE, if it was triggered during the random access procedure will be helpful for the networks to identify the exact cause of failure.

Similarly, in order to get the satellite position for the UE to self-precompensate the TA, the network signals the satellite position in SIB19, also called the ephemeris. This ephemeris value has a validity duration, which forces the UE to re-acquire the SIB19 every now and then. Some of the typical cause of failures in NTN are related to the uplink validity. It will be useful to report information related to the uplink validity and ephemeris.

Proposal 2: UE reports following additional information in RLF report for NTN related failures

- Timing Advance reported in the Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

- A flag indicating whether T430 had expired when RLF or HOF occurred.

- Information related to satellite position.

## Supporting multiple TACs

UE logs the TAC of the relevant cells in various SON/MDT reports. CGI-Info-Logging including tracking area code of the cell. Since a NTN cell can have multiple TAC(s), UE may log the tackingAreaList when it is logging CGI-Info-Logging in various SON/MDT reports.

Proposal 3: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

# Conclusion

Based on the discussion, we have the following observations and proposals.

Observation 1: The basic principle of Logged MDT is that UE just logs the measurements it has and is not required to perform any new measurements.

Observation 2: Geographical area based Area Scope for logged MDT requires UE to perform additional measurements and has a huge impact on UE battery performance.

Proposal 1: RAN2 to send LS to RAN3 informing that geographical area based Area Scope over Uu is not feasible for logged MDT.

Proposal 2: UE reports following additional information in RLF report for NTN related failures

- Timing Advance reported in the Timing Advance Report MAC CE, if it was triggered during the random access procedure during a handover.

- A flag indicating whether T430 had expired when RLF or HOF occurred.

- Information related to satellite position.

Proposal 3: UE may log tracking area list in CGI-Info-Logging in various SON/MDT reports.

# References

[1] RP-234038: New WID on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] R3-247908: LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT