3GPP TSG-RAN WG2 Meeting #129bis	R2-2502449

China, Wuhan, April. 7th – 11th, 2025

Source:                Qualcomm Incorporated

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# Discussion

## 2.0 RAN4 supported cases

RAN4 agreed to support the following cases for measurement offloading to LP-WUS.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

In case #1, serving cell is fully offloaded to LR

In case #3, serving cell can be measured by MR and LR, but combined measurement results from both LR and MR are not supported. The scenario can be shown in the following Figure 1.

Figure 1

2.1 Exit conditions

In RAN2 #128 meeting, there is FFS on exist condition as following.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

If RRM relaxation exit shares the same condition with entry, then there could be a ping-pong switching between entry and exit. To avoid this ping-pong issue, it is preferred that a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation and offloading exit condition.

- To avoid ping-pong switching, a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation and offloading exit condition.

For RRM relaxation mode, one open issue is whether both MR and LR threshold to be configured to the UE. Network can configure both of MR and LR threshold as RRM relaxation mode exit condition, and when either MR or LR measurement is below the corresponding threshold, UE should exit from RRM relaxation mode.

- Network can configure both of MR and LR threshold as RRM relaxation mode exit condition.

- The condition for leaving RRM relaxation mode: when any of LR or MR measurement is below a threshold.

## 2.3 Measurement quantities for offloading evaluation

During running CR discussion, there are some FFS issue to be discussed.

Editor’s NOTE: FFS for the metrics of serving cell RRM relaxation (i.e. RSRP and/or RSRQ).

Editor’s NOTE: FFS serving cell quality by MR is existing Srxlev/ Squal or Qrxlevmeas/ Qqualmeas (i.e. measured value).

Editor’s NOTE: FFS serving cell quality by LR is measured value.

Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements.

Editor’s NOTE: FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

Similar with LP-WUS monitoring condition, RSRP should be supported, and RSRQ can be optionally configured.

- The MR measurement quantities for RRM relaxation and offloading evaluation are RSRP, and RSRQ (optional).

For the serving cell quality by MR, in order to have aligned UE behavior for LP-WUS and legacy serving cell evaluation, it is proposed to use existing Srxlev/Squal as MR measurement quantities.

- Use existing Srxlev/Squal for MR measurement based entry/exit condition evaluation.

For LR serving cell quality, if MR using Srxlev/Squal for relaxation evaluation, then LR should also use similar way instead of using measured value.

- Introduce Srxlev_lp/Squal_lp for LR measurement based entry/exit condition evaluation. The formular to derive Srxlev/Squal from measured MR value can be  used for deriving Srxlev_lp/Squal_lp from measured LR value.

## 2.3 Existing Neighboring measurement relaxation criterion

Currently, for intra-frequency measurement, in order to save UE power, a serving cell quality threshold (SIntraSearchP/SIntraSearchQ) are introduced for trigger intra-frequency measurement. When in relaxed measurement mode to LP-WUS, UE will measure using both of MR and LR, but the MR measurement is in a relaxed way. That means there are two measurement results. One issue is which measurement result will be used to trigger intra-frequency measurement. One way is using existing serving cell MR measurement, but serving cell measurement is in a relaxed way, the measurement accuracy could not be enough to evaluate the condition; another way is to introduce LP-WUS specific parameters.

Similarly, for inter-frequency measurement relaxation, the criteria to determine ‘not at cell edge’, whether MR measurement or LR measurement is used needs to be determined.

From RAN2 perspective, it should be assumed that existing MR serving cell measurement in a relaxed way criterion can be reused, then not to introduce LP-WUS specific criterion for neighboring measurement relaxation. It is up to gNB implementation to guarantee neighboring measurement relaxation should be triggered outside of LPWUS fully loading area.

It is proposed to ask RAN4 whether the relaxed MR serving cell measurements can be used to evaluate triggering neighboring cell measurements and measurement relaxation.

- RAN2 assumes to reuse existing MR serving cell measurement criterion, i.e. SIntraSearchP/SIntraSearchQ to trigger intra-frequency and inter-frequency neighboring measurement relaxation.

- RAN2 assumes to reuse existing MR serving cell measurement criterion for Rel-16 “not at cell edge” evaluation for inter-frequency measurement relaxation.

- It is up to gNB configuration to guarantee neighboring measurement relaxation should be triggered outside of LPWUS fully loading area.

## 2.4 Cell reselection criteria

Currently, cell reselection criteria is based on MR measurement evaluation. If the serving cell measurement is totally offloaded to LR, then there is no MR serving cell measurement for cell reselection evaluation.  From RAN2 perspective, it is assumed that existing MR serving cell measurement in a relaxed way criterion can be reused for cell reselection evaluation, then no need to introduce LP-WUS specific criteria for neighboring measurement relaxation. It is up to gNB implementation to guarantee cell reselection to be evaluated outside of LPWUS fully loading area.

- RAN2 assumes to reuse existing MR serving cell measurement for cell reselection evaluation. It is up to gNB configuration to guarantee cell reselection to be evaluated outside of LPWUS fully loading area.

- Ask RAN4 whether the relaxed MR serving cell measurements can be used to evaluate triggering neighboring cell measurement relaxation and cell reselection.

# Conclusion

- To avoid ping-pong mode switching, a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation/offloading exit condition. Network can configure both of MR and LR threshold as RRM relaxation mode exit condition.

- Network can configure both of MR and LR threshold as RRM relaxation mode exit condition.

- The condition for leaving RRM relaxation mode: when any of LR or MR measurement is below a threshold.

- The MR measurement quantities for RRM relaxation and offloading evaluation are RSRP, and RSRQ (optional).

- Use existing Srxlev/Squal for MR measurement based entry/exit condition evaluation.

- Introduce Srxlev_lp/Squal_lp for LR measurement based entry/exit condition evaluation. The formular to derive Srxlev/Squal from measured MR value can be  used for deriving Srxlev_lp/Squal_lp from measured LR value.

- RAN2 assumes to reuse existing MR serving cell measurement criterion, i.e. SIntraSearchP/SIntraSearchQ to trigger intra-frequency and inter-frequency neighboring measurement relaxation.

- RAN2 assumes to reuse existing MR serving cell measurement criterion for Rel-16 “not at cell edge” evaluation for inter-frequency measurement relaxation.

- It is up to gNB configuration to guarantee neighboring measurement relaxation should be triggered outside of LPWUS fully loading area.

- RAN2 assumes to reuse existing MR serving cell measurement for cell reselection evaluation. It is up to gNB configuration to guarantee cell reselection to be evaluated outside of LPWUS fully loading area.

- Ask RAN4 whether the relaxed MR serving cell measurements can be used to evaluate triggering neighboring cell measurement relaxation and cell reselection.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)