3GPP TSG-RAN WG2 Meeting #129bis	R2-2502451

China, Wuhan, April 7th – 11th 2025

Source:                Qualcomm Incorporated

Title:	SON and MDT for CHO with candidate SCGs

Agenda Item:	8.10.2.2

Document for:	Discussion and Decision

# Introduction & Background

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

### 2.1 CHO Recovery after CHO with candidate SCG execution failure

On MRO for CHO with candidate SCG, there could be some scenarios to be considered as following.

The CHO conditions are fullfilled but the RLF happens after CHO with candidate SCG execution; or SCG failure happens. In this case, it is worthy for UE to report time elapsed between the CHO with candidate SCG execution and RLF or SCG failure.

In existing specification, the timer timeConnFailure is used to indicate the time elapsed since the last HO execution until connection failure; and timer timeSCGFailure is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure

- Reuse the timer timeConnFailure in RLF Report to indicate time elapsed between the CHO with candidate SCG execution and RLF.

- Reuse the timer timeSCGFailure in SCGFailureInformation to indicate time elapsed between the CHO with candidate SCG execution and SCG failure.

### 2.2 CHO Recovery after CHO with candidate SCG execution failure

When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

Suppose a UE receives a CHO with candidate SCG config, both the execution conditions are met but the CHO execution fails (e.g., RACH in target cell fails). UE performs cell selection, and the selected cell is also a CHO candidate cell.

As per the below text from 38.331, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

1>  if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>  if attemptCondReconfig is configured; and

1>  if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>  if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>  if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>  apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:  It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

We therefore have the following proposal:

- UE should log the CHO recovery cell ID in RLF Report when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Consider Scenario 2 again where the UE encounters an RLF before the CHO with candidate SCG execution because candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

Suppose the UE attempts CHO recovery and observed that both PCell A2 and PCell A3 are suitable cells, the UE should be able to prioritize selecting PCell A2 for CHO recovery because its associated PSCell’s (B3) execution condition was met before rather than selecting PCell A3 for CHO recovery because its associated PCell’s (B4) execution condition was not met before.

We therefore have the following proposal:

- In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not.

# Conclusion

- Reuse the timer timeConnFailure in RLF Report to indicate time elapsed between the CHO with candidate SCG execution and RLF.

- Reuse the timer timeSCGFailure in SCGFailureInformation to indicate time elapsed between the CHO with candidate SCG execution and SCG failure.

- UE should log the CHO recovery cell ID in RLF Report when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

- In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not.

# Reference