3GPP TSG-RAN2#129bis	R2-2502461

Wuhan, China, April 7 – April 11, 2025

Agenda item:		8.7.6

Source:		Lenovo

Title:		 XR Rate Control

Document for:		Discussion and Decision

- Introduction

For XR traffic, it would be beneficial to enable the application to adapt the codec rate based on the network conditions to avoid dropping of packets over the air interface. Such discussion has been ongoing in the RAN Plenary with the following being agreed as the way forward in the last meeting, RAN #105 [1].

- Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Additionally, the following agreements were made at the RAN2 #128and RAN2 #129 meetings, as follows:

Agreements from RAN2 #128 1. RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. 2. gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN. 3. Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. 4. RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. 5. RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE. 6. FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful. Agreements from RAN2 #129 - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

This contribution looks at how the XR rate control information may be signaled in the downlink to support source rate adaption to uplink congestion.

- Discussion

For XR applications that have very strict requirements in terms of data rate and latencies, it is beneficial for the application to be able to adapt the source rates based on the congestion and network conditions on the uplink. During the plenary meetings, many companies have been discussing whether the Recommended Bitrate MAC CE can be reused for XR applications. In our view, the existing Recommended Bitrate MAC CE may pose certain inefficiencies with respect to XR:

- Granularity:

Currently the Recommended Bitrate MAC CE is signalled on a per LCH basis, i.e., on a per DRB basis. In the case of XR, multiple QoS flows may be mapped to the same DRB. Upon receiving a RBR MAC CE, the UE cannot differentiate the bitrate for each QoS flow which can impact the upper layer throughput.

- Bitrate Range:

Although the RBR MAC CE can cover up to 1.6Gbps, the granularity of the values may not be sufficient for XR applications. With only 56 indices and a single multiplier, the recommended bitrate value may not accurately match the bitrate required by the application which can degrade the user-experience.

- The UE cannot differentiate the bitrate for each QoS flow of a DRB from the RBR MAC CE.

During the RAN2#128 meeting, RAN2 had concluded that a UL rate indication from gNB to UE on a per QoS flow basis is supported. RAN2 had further proposed two options – whether to indicate the UL rate recommendation in an enhanced MAC CE at QoS flow level, or to reuse the existing RBR MAC CE format but with a RRC configuration to indicate the adaptable QoS flows.

During the RAN3#127 meeting, the two options were discussed and RAN3 informed RAN2 that both options have impact on the F1 interface [2]. In case of gNB CU-DU split, the gNBDU performs scheduling on a per DRB level and can determine the congestion status since radio resource is scheduled in the gNB-DU. However, the gNB-CU provides the QoS profiles for all QoS flows to the gNB-DU and can measure the bitrate on a per QoS level. In our view, it is easier and more reasonable for gNB-CU to monitor the UL bitrate per QoS flow and provide this information to the gNB-DU so that the gNB-DU can determine the recommended bitrate on a per QoS flow level based on the congestion status. Hence, we think that a new MAC CE that indicates rate control on a per QoS flow is the more feasible option.

RAN2 to support recommended UL rate indication on a per QoS flow in a new MAC CE.

Additionally, SA4 had provided a response regarding the range and granularity of UL bitrates that need to be supported. The range varies from 100kbps to 40Mbps whereas a granularity of 1% is preferred from the application perspective although a coarser granularity of 10% is also sufficient. According to the MAC specs, the current table for bitrate recommendation values provides sufficient range for XR applications but does not provide sufficient granularity. It was also agreed in the previous RAN2 meeting that a new table will be specified for XR rate control although it was still FFS regarding the distribution and codepoints. In our view, the most straightforward and simple solution is to have a fixed step size of 10% with 64 codepoints, i.e., the recommended bitrate indication can be achieved by 6 bits. Additionally, it is also cleaner to design a new MAC CE format that indicates the bitrate recommendation on a per QoS flow using the index from the new table that is specific for XR applications. The new table needs to have a range of 100kbps to 40Mbps as informed by SA4. Hence, we have formulated the new bitrate table as follows:

| Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) |
| --- | --- | --- | --- | --- | --- |
| 0 | 100 | 23 | 895.4302433≈895 | 46 | 8017.953205≈8018 |
| 1 | 110 | 24 | 984.9732676≈985 | 47 | 8819.748526≈8820 |
| 2 | 121 | 25 | 1083.470594≈1083 | 48 | 9701.723378≈9702 |
| 3 | 133.1≈133 | 26 | 1191.817654≈1192 | 49 | 10671.89572≈10672 |
| 4 | 146.41≈146 | 27 | 1310.999419≈1311 | 50 | 11739.08529≈11739 |
| 5 | 161.051≈161 | 28 | 1442.099361≈1442 | 51 | 12912.99382≈12913 |
| 6 | 177.1561≈177 | 29 | 1586.309297≈1586 | 52 | 14204.2932≈14204 |
| 7 | 194.87171≈195 | 30 | 1744.940227≈1745 | 53 | 15624.72252≈15625 |
| 8 | 214.358881≈214 | 31 | 1919.43425≈1919 | 54 | 17187.19477≈17187 |
| 9 | 235.7947691≈236 | 32 | 2111.377675≈2111 | 55 | 18905.91425≈18906 |
| 10 | 259.374246≈259 | 33 | 2322.515442≈2323 | 56 | 20796.50567≈20797 |
| 11 | 285.3116706≈285 | 34 | 2554.766986≈2555 | 57 | 22876.15624≈22876 |
| 12 | 313.8428377≈314 | 35 | 2810.243685≈2810 | 58 | 25163.77186≈25164 |
| 13 | 345.2271214≈345 | 36 | 3091.268053≈3091 | 59 | 27680.14905≈27680 |
| 14 | 379.7498336≈380 | 37 | 3400.394859≈3400 | 60 | 30448.16395≈30448 |
| 15 | 417.7248169≈418 | 38 | 3740.434344≈3740 | 61 | 33492.98035≈33493 |
| 16 | 459.4972986≈459 | 39 | 4114.477779≈4114 | 62 | 36842.27838≈36842 |
| 17 | 505.4470285≈505 | 40 | 4525.925557≈4526 | 63 | 40526.50622≈40527 |
| 18 | 555.9917313≈556 | 41 | 4978.518112≈4979 |  |  |
| 19 | 611.5909045≈612 | 42 | 5476.369924≈5476 |  |  |
| 20 | 672.7499949≈673 | 43 | 6024.006916≈6024 |  |  |
| 21 | 740.0249944≈740 | 44 | 6626.407608≈6626 |  |  |
| 22 | 814.0274939≈814 | 45 | 7289.048369≈7289 |  |  |

Based on the above approach, it can also be seen that the required granularity is achieved by the new table without a need for new bitRateMultiplier values. Hence, if the above table is agreed, RAN2 does not specify new bitRateMultiplier values.

The new bitrate table has 64 codepoints with a fixed step size of 10% with values ranging from 100kbps to 40Mbps, as follows:

| Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) |
| --- | --- | --- | --- | --- | --- |
| 0 | 100 | 23 | 895.4302433≈895 | 46 | 8017.953205≈8018 |
| 1 | 110 | 24 | 984.9732676≈985 | 47 | 8819.748526≈8820 |
| 2 | 121 | 25 | 1083.470594≈1083 | 48 | 9701.723378≈9702 |
| 3 | 133.1≈133 | 26 | 1191.817654≈1192 | 49 | 10671.89572≈10672 |
| 4 | 146.41≈146 | 27 | 1310.999419≈1311 | 50 | 11739.08529≈11739 |
| 5 | 161.051≈161 | 28 | 1442.099361≈1442 | 51 | 12912.99382≈12913 |
| 6 | 177.1561≈177 | 29 | 1586.309297≈1586 | 52 | 14204.2932≈14204 |
| 7 | 194.87171≈195 | 30 | 1744.940227≈1745 | 53 | 15624.72252≈15625 |
| 8 | 214.358881≈214 | 31 | 1919.43425≈1919 | 54 | 17187.19477≈17187 |
| 9 | 235.7947691≈236 | 32 | 2111.377675≈2111 | 55 | 18905.91425≈18906 |
| 10 | 259.374246≈259 | 33 | 2322.515442≈2323 | 56 | 20796.50567≈20797 |
| 11 | 285.3116706≈285 | 34 | 2554.766986≈2555 | 57 | 22876.15624≈22876 |
| 12 | 313.8428377≈314 | 35 | 2810.243685≈2810 | 58 | 25163.77186≈25164 |
| 13 | 345.2271214≈345 | 36 | 3091.268053≈3091 | 59 | 27680.14905≈27680 |
| 14 | 379.7498336≈380 | 37 | 3400.394859≈3400 | 60 | 30448.16395≈30448 |
| 15 | 417.7248169≈418 | 38 | 3740.434344≈3740 | 61 | 33492.98035≈33493 |
| 16 | 459.4972986≈459 | 39 | 4114.477779≈4114 | 62 | 36842.27838≈36842 |
| 17 | 505.4470285≈505 | 40 | 4525.925557≈4526 | 63 | 40526.50622≈40527 |
| 18 | 555.9917313≈556 | 41 | 4978.518112≈4979 |  |  |
| 19 | 611.5909045≈612 | 42 | 5476.369924≈5476 |  |  |
| 20 | 672.7499949≈673 | 43 | 6024.006916≈6024 |  |  |
| 21 | 740.0249944≈740 | 44 | 6626.407608≈6626 |  |  |
| 22 | 814.0274939≈814 | 45 | 7289.048369≈7289 |  |  |

If Proposal 2 is agreed, RAN2 does not specify new bitRateMultiplier values.

- Conclusion

- Observation 1: The UE cannot differentiate the bitrate for each QoS flow of a DRB from the RBR MAC CE.

- Proposal 1: RAN2 to support recommended UL rate indication on a per QoS flow in a new MAC CE.

- Proposal 2: The new bitrate table has 64 codepoints with a fixed step size of 10% with values ranging from 100kbps to 40Mbps, as follows:

| Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) | Index | NR Recommended Bitrate value (kbps) |
| --- | --- | --- | --- | --- | --- |
| 0 | 100 | 23 | 895.4302433≈895 | 46 | 8017.953205≈8018 |
| 1 | 110 | 24 | 984.9732676≈985 | 47 | 8819.748526≈8820 |
| 2 | 121 | 25 | 1083.470594≈1083 | 48 | 9701.723378≈9702 |
| 3 | 133.1≈133 | 26 | 1191.817654≈1192 | 49 | 10671.89572≈10672 |
| 4 | 146.41≈146 | 27 | 1310.999419≈1311 | 50 | 11739.08529≈11739 |
| 5 | 161.051≈161 | 28 | 1442.099361≈1442 | 51 | 12912.99382≈12913 |
| 6 | 177.1561≈177 | 29 | 1586.309297≈1586 | 52 | 14204.2932≈14204 |
| 7 | 194.87171≈195 | 30 | 1744.940227≈1745 | 53 | 15624.72252≈15625 |
| 8 | 214.358881≈214 | 31 | 1919.43425≈1919 | 54 | 17187.19477≈17187 |
| 9 | 235.7947691≈236 | 32 | 2111.377675≈2111 | 55 | 18905.91425≈18906 |
| 10 | 259.374246≈259 | 33 | 2322.515442≈2323 | 56 | 20796.50567≈20797 |
| 11 | 285.3116706≈285 | 34 | 2554.766986≈2555 | 57 | 22876.15624≈22876 |
| 12 | 313.8428377≈314 | 35 | 2810.243685≈2810 | 58 | 25163.77186≈25164 |
| 13 | 345.2271214≈345 | 36 | 3091.268053≈3091 | 59 | 27680.14905≈27680 |
| 14 | 379.7498336≈380 | 37 | 3400.394859≈3400 | 60 | 30448.16395≈30448 |
| 15 | 417.7248169≈418 | 38 | 3740.434344≈3740 | 61 | 33492.98035≈33493 |
| 16 | 459.4972986≈459 | 39 | 4114.477779≈4114 | 62 | 36842.27838≈36842 |
| 17 | 505.4470285≈505 | 40 | 4525.925557≈4526 | 63 | 40526.50622≈40527 |
| 18 | 555.9917313≈556 | 41 | 4978.518112≈4979 |  |  |
| 19 | 611.5909045≈612 | 42 | 5476.369924≈5476 |  |  |
| 20 | 672.7499949≈673 | 43 | 6024.006916≈6024 |  |  |
| 21 | 740.0249944≈740 | 44 | 6626.407608≈6626 |  |  |
| 22 | 814.0274939≈814 | 45 | 7289.048369≈7289 |  |  |

Proposal 3: If Proposal 2 is agreed, RAN2 does not specify new bitRateMultiplier values.

- References

[1] RP-242390: Way Forward on WID Scope Revision, Nokia

[2] R3-250805, Reply LS on XR UL Rate Control, RAN3