3GPP TSG-RAN WG2 Meeting #129bis					           R2-2502463

Wuhan, China, 7 – 11 April, 2025

Agenda Item:		8.3.2.2

Source:			Huawei, HiSilicon

Title:		RRM measurement prediction model generalization evaluation for different cell configurations

Document for:		Discussion and Decision

# 1   Introduction

In the previous meetings RAN2 analysed RRM measurement prediction accuracy when using AIML models in different scenarios. The following scenarios were prioritized for evaluation:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

During RAN2#129 meeting, companies provided the results for model generalization performance across different UE speeds (see e.g. our results in [1]), and the following was concluded:

For temporal-domain prediction: - For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across all UE speeds - Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size). - For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline. For frequency-domain prediction: - Generalization using GC#2 always outperform that of GC#1 - Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output. - Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss. - For GC#2 case, the prediction accuracy is acceptable and close to the baseline

Furthermore, RAN2 agreed to run another set of simulations to verify how the model performs across different cell configurations and channel models. In this Tdoc we provide the results for this exercise.

# 2   Simulation results

## 2.1 General

Our previous RSRP prediction accuracy results were provided in [2]. We evaluated AIML performance using LSTM model and we continue to use this model for the generalization performance evaluation, as presented in this Tdoc. All the simulation assumptions are aligned with RAN2 agreements and outlined in detail in the excel sheet attached to this Tdoc.

In RAN2#129 meeting, RAN2 made agreements on generalization based on cell configuration for both FR1 and FR2. Two generalization cases were agreed, i.e.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with one configuration but tested using the dataset with another configuration

- Generalization Case #2 (GC#2): The AI/ML model is trained using datasets obtained using both cell configurations and tested using the dataset for one of these configurations.

The sets of parameters (denoted as Configuration #1 and Configuration #2) agreed to be used for generalization analysis for FR1 and FR2 are presented in Table 1 and Table 2 respectively.

Table 1. Generalization parameters for FR1

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 2. Generalization parameters for FR2

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

## 2.2 Generalization evaluation for FR1

The performance of model generalization for FR1 was evaluated for Scenario 2, i.e. intra-frequency Case B with the following settings:

- MRRT=50%

- OW=400ms

- PW=200ms

- RRM sub use case 2, sliding L1 filtering

- UE speed: 30 km/h

The remaining simulation assumptions were aligned with those agreed by RAN2 earlier and details can be found in the excel sheet attached to this Tdoc. The results, i.e. an average difference for L3 RSRP [dB], are presented in the Table 3 below.

Table 3. Cell configuration-based generalization evaluation results for FR1

|  | Training @Configuration#1 (UMi) | Training @Configuration#2 (UMa) | Inference @Configuration#1 (UMi) | Inference @Configuration#2 (UMa) |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | 0.47 dB |  |
| GC#1 |  | Yes | 0.48 dB |  |
| GC#2 | Yes | Yes | 0.47 dB |  |
| Baseline |  | Yes |  | 0.48 dB |
| GC#1 | Yes |  |  | 0.49 dB |
| GC#2 | Yes | Yes |  | 0.48 dB |

As we can see the RSRP differences for the studied generalization cases are either the same as in the baseline case or very close (RSRP difference of 0.01 dB). GC#2 performs better and achieves the same RSRP as the baseline case, but the performance of GC#1 is also very good, i.e. the RSRP difference is equal to mere 0.01 dB. The same results are achieved regardless of which configuration is used for the training and which is used for the inference.

Based on the presented results, we can make the following observations:

Observation 1: For FR1, AIML model generalizes very well for different cell configurations and across different channel models. The prediction accuracy difference is non-existent or negligible (less than 0.01 dB).

Observation 2: For FR1, AIML model generalizes equally well for both “directions”, i.e. model trained with Configuration#1 and used for inference for Configurtion#2 and the other way around.

## 2.3 Generalization evaluation for FR2

The performance of model generalization for FR2 was evaluated for Scenario 4, i.e. intra-frequency Case A with the following settings:

- OW=480ms

- PW=240ms

- RRM sub use case 2, sliding L1 filtering

- UE speed: 60km/h

The remaining simulation assumptions were aligned with those agreed by RAN2 earlier and details can be found in the excel sheet attached to this Tdoc. The results are presented in tables below.

Table 4. Cell configuration-based generalization evaluation results for FR2

|  | Training @Configuration#1 (UMi) | Training @Configuration#2 (UMa) | Inference @Configuration#1 (UMi) | Inference @Configuration#2 (UMa) |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | 1.69 dB |  |
| GC#1 |  | Yes | 1.74 dB |  |
| GC#2 | Yes | Yes | 1.70 dB |  |
| Baseline |  | Yes |  | 1.22 dB |
| GC#1 | Yes |  |  | 1.27 dB |
| GC#2 | Yes | Yes |  | 1.23 dB |

The observations for FR2 evaluation are similar to those made for FR1. For FR2 however, the difference between the baseline and GC#1 is slightly more visible (0.05 dB), but GC#2 performs virtually as well as the baseline case (the difference of accuracy is 0.01 dB). Even though for this scenario, we can observe that inference at Configuration#2 results in higher prediction accuracy, this is an effect of UMi being a more complicated channel model. However, the generalization performance is equal for both “directions”, i.e. the difference of GC#1/GC#2 from the baseline is the same.

Based on the presented results, we can make the following observations:

Observation 3: For FR2, AIML model generalizes very well for different cell configurations and across different channel models. The prediction accuracy difference is non-existent or negligible, i.e. 0.05 dB for GC#1 and 0.01 dB for GC#2.

Observation 4: For FR2, AIML model generalizes equally well for both “directions”, i.e. model trained with Configuration#1 and used for inference for Configurtion#2 and the other way around.

# 3   Summary

Based on the simulations presented in this paper, the following observations are made:

Generalization for different cell configurations/channel models in FR1

Observation 1: For FR1, AIML model generalizes very well for different cell configurations and across different channel models. The prediction accuracy difference is non-existent or negligible (less than 0.01 dB).

Observation 2: For FR1, AIML model generalizes equally well for both “directions”, i.e. model trained with Configuration#1 and used for inference for Configurtion#2 and the other way around.

Generalization for different cell configurations/channel models in FR2

Observation 3: For FR2, AIML model generalizes very well for different cell configurations and across different channel models. The prediction accuracy difference is non-existent or negligible, i.e. 0.05 dB for GC#1 and 0.01 dB for GC#2.

Observation 4: For FR2, AIML model generalizes equally well for both “directions”, i.e. model trained with Configuration#1 and used for inference for Configurtion#2 and the other way around.

# References

- R2-2500891 Model generalization evaluation for RRM measurement prediction Huawei, HiSilicon

- R2-2410539 Simulation results for RRM measurement prediction Huawei, HiSilicon

- R2-2500287 Text proposal of 38.744	OPPO	 pCR	Rel-19	38.744	0.0.6

# Annex A – simulation assumptions

Below are the company selectable assumptions chosen by us.

| Configurable/selectable simulation assumption |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spatial consistency | Whether LOSsoft is modeled or not | UE trajectory model | UE trajectory boundary processing | L1/L3 filtering (sliding/non-sliding) | Number of Tx beams | Number of Rx beams |
| Procedure B in TR38.901 | Not modelled | Option 1 in 38.843 section 6.3.1 | Wrap-around model | Both non-filtered and non-sliding L1 filtering cases were simulated | FR1: 4 FR2: 64 | FR1: 1 FR2: 8 |

All the simulation assumptions are listed in the table below with the selectable ones highlighted in yellow.

Table 1: Summarized simulation assumptions (based on Table 5.1-1: Simulation assumptions of FR1 and FR2 in [3])

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model |